3GPP TSG-RAN WG4 Meeting #117	R4- 2520721

# Dallas, USA, 17th – 21st November, 2025

Source:	LG Electronics

Title:	Discussion of RAN4-driven AI/ML use cases for 6G AI

Agenda item:	8.8.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on AI/ML use cases for RRM based on WF [1].

Discussion

- Topic #6: AI/ML use case for RRM

6.1.1 Use Case#5: AI-based measurement prediction

#### Issue 6-1a: Use case definition and supporting level from companies

Agreement: - The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

During last RAN4#116bis meeting the AI/ML use cases for RRM were categorized based on each company’s proposal. We provide our views on each use cases.

Use case index 1. L1 beam measurement

L1 beam-level measurements are typically used in use cases that require rapid decision-making and are characterized by high variability compared to L3 measurements. On the other hand, AI/ML techniques generally require a certain amount of measurement data over a time interval to achieve sufficient performance, and perform best when the input metrics are highly correlated with the target inference. Therefore, L1 beam-level measurements are considered less suitable for use cases involving the application of AI/ML techniques.

Proposal 1: RAN4 to deprioritize the L1 beam-level measurements prediction use cases.

Use case index 2. L3 beam measurement

Regarding L3 beam measurement case, to avoid overlapping research with other WGs, it would be good to discuss frequency-domain prediction in non-collocated cell scenarios at 6G AI. From the RAN4 perspective, we are not sure what tasks should be carried out. It seems that NW-based AI would be appropriate, and prediction should be performed through cluster-based measurements. Considering these aspects, we should first perform environment modeling and then carry out an evaluation of feasibility.

Proposal 2: To avoid overlapping research with other WGs, it would be good to discuss frequency-domain prediction in non-collocated cell scenarios for L3 beam measurement.

Use case index 3. L3 cell measurement

Regarding L3 cell measurement case, to avoid overlapping research with other WGs, it would be good to discuss frequency-domain prediction in non-collocated cell scenarios at 6G AI. From the RAN4 perspective, we are not sure what tasks should be carried out. It seems that NW-based AI would be appropriate, and prediction should be performed through cluster-based measurements. Considering these aspects, we should first perform environment modeling and then carry out an evaluation of feasibility. Compared to L3 beam measurement, L3 cell measurement seems more feasible and more realistic from an evaluation perspective. Therefore, from our perspective, L3 cell measurement is considered more favorable than L3 beam measurement in evaluation.

Proposal 3: To avoid overlapping research with other WGs, it would be good to discuss frequency-domain prediction in non-collocated cell scenarios for L3 cell measurement.

Use case index 4. Multi-Domain L3 level Prediction for Measurement-Gap Reduction

We think the Multi-Domain L3-level prediction use case is highly valuable for research. When correlations are sufficiently strong, measurements can be replaced by prediction results, leading to measurement gap reduction. Since throughput loss due to measurement gaps can exceed 10% in some cases, achieving measurement gap reduction would provide significant benefits from a throughput perspective.

Proposal 4: Multi-Domain L3-level prediction use case is highly valuable for research in RAN4.

Use case index 5/6. Dynamic Adaptation of Measurement Procedure

We think that dynamic adaptation of measurement procedure is the right direction for 6G AI and the most suitable use case. For example, considering the RRM measurement prediction currently discussed in 5G-A, results measured during the Observation Window (OW) are used as input to an AI/ML model to predict values for the Prediction Window (PW). It is expected that the lengths of OW and PW will be configured based on channel conditions. Once the OW and PW lengths are set according to the channel condition at the configuration point, they are unlikely to change frequently. The accuracy of prediction values within the PW is expected to be higher closer to the OW. In actual 6G, depending on the reliability of prediction values, if the reliability remains above a certain threshold, the prediction window could be dynamically extended. Or if the reliability is low, the prediction window could be dynamically shortened.

Proposal 5: Dynamic Adaptation of Measurement Procedure use case is highly valuable for research in RAN4.

Use case index 7/8. Spatial domain RX beam sweeping reduction (L1/L3)

Spatial domain RX beam sweeping reduction use case is worth studying, as it helps reduce the UE’s measurement effort. And this use case is not duplicated with other WGs.

Proposal 6: Spatial domain RX beam sweeping reduction use case is worth studying in RAN4.

Use case index 9. AI/ML based RLM/BFD/CBD, LTM by measurement prediction

We think that AI-based RLM/BFD/CBD and LTM by measurement prediction would have the advantage of enabling faster decision-making.

Proposal 7: AI/ML based RLM/BFD/CBD, LTM by measurement prediction use case is worth studying in RAN4.

Use case index 10. AI/ML based SCell related enhancement by measurement prediction

We think the measurement window required for SCell activation is likely shorter than the window needed to obtain sufficiently accurate AI/ML prediction results. Even if it were longer, the difference would probably be insignificant. Furthermore, if the goal is to reduce the SCell activation delay using AI/ML, the accuracy of the predicted values might degrade. Therefore, we think that SCell-related enhancements achieved through AI/ML would provide little benefit.

Proposal 8: AI/ML based SCell-related enhancements use case is expected to provide little benefit.

Use case index 11. AI-based prediction for enhanced s-measure mechanism

If the s-measurement mechanism can be effectively optimized through AI, improvements in UE mobility performance can be expected. Enhancing UE mobility performance through an optimized s-measurement mechanism is a reasonable goal, and it is understandable that s-threshold tuning may be needed depending on the UE’s environment. Although the conditions are complex, this makes it a suitable use case for applying AI/ML.

Proposal 9: AI-based prediction for enhanced s-measure mechanism is a suitable use case for applying AI/ML.

Use case index 12. AI/ML based AGC through temporal/frequency domain prediction

Since AGC requires only a small number of samples, and measurements would still need to be performed for AI/ML-based prediction, it is unlikely that AI/ML would offer significant time-saving benefits. Furthermore, predicting AGC solely based on side conditions without additional measurements is not realistic. For example, AGC can vary significantly depending on whether the environment is LOS or NLOS, and also based on the interference level in the frequency band being measured. Acquiring all such contextual information to perform AGC using AI/ML would be inefficient, especially considering that conventional AGC operations are not burdensome in terms of complexity or processing time. Therefore, we believe that AI/ML-based AGC does not provide a meaningful advantage.

Proposal 10: AI/ML-based AGC use case is not expected to provide a meaningful advantage.

Use case index 13. AI/ML based DL sync through temporal/frequency domain prediction

Similar to AGC, it is difficult to predict synchronization (synch) information without knowing whether the environment is LOS or NLOS. If synch information is obtained through AI prediction and turns out to be incorrect, decoding will be based on the wrong synch, and any decoding failure would introduce a new challenge: distinguishing whether the error was due to incorrect synchronization or poor channel conditions. Given the high penalty associated with such errors, we do not support AI-based prediction for synch detection.

Proposal 11: We would like to deprioritize AI/ML-based DL synchronization, as it is unlikely to deliver high performance and is expected to incur a significant penalty for false alarms.

Use case index 14. AI abstraction of the wireless environment to facilitate data scheduling and network power saving

We believe the issue is somewhat unclear. It is not evident how wireless environment information around the base station, obtained through network-based AI, would directly impact data scheduling for the UE. When scheduling data for the UE, the UE’s own channel conditions are critical, and such information can only be obtained through UE-side measurements. Therefore, it is reasonable that data scheduling should be based on the UE’s channel environment. Based on this view, we believe the issue should either be clarified further or deprioritized.

Proposal 12: “AI abstraction of the wireless environment to facilitate data scheduling and network power saving” issue should either be clarified further or deprioritized.

Based on our analysis for each use case, we have summarized our view in the table below. And our preferences are highlighted in yellow.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((d, e), as RAN1-led study), Apple (c, d, e) | No benefits |  |  |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (c, d, e) | MG reduction | No dependency | Not sure. Looks like NW sided model. Cluster based measurement for prediction. Need to check the feasibility. |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) | MG reduction | No dependency | Not sure. Looks like NW sided model. Cluster based measurement for prediction. Need to check the feasibility. |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung | MG reduction | No dependency |  |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple | Optimized measurement procedure, power saving, MG reduction | No dependency |  |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi | Measurement delay reduction | No dependency |  |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | OPPO, vivo (AI for RLM) | Early decision making | No dependency |  |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO | Little benefit |  |  |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | vivo | Better mobility performance | No dependency |  |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE | No benefits |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE | No benefits |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT | Need to clarify more |  |  |

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s WF [1], and we propose

Proposal 1: RAN4 to deprioritize the L1 beam-level measurements prediction use cases.

Proposal 2: To avoid overlapping research with other WGs, it would be good to discuss frequency-domain prediction in non-collocated cell scenarios for L3 beam measurement.

Proposal 3: To avoid overlapping research with other WGs, it would be good to discuss frequency-domain prediction in non-collocated cell scenarios for L3 cell measurement.

Proposal 4: Multi-Domain L3-level prediction use case is highly valuable for research in RAN4.

Proposal 5: Dynamic Adaptation of Measurement Procedure use case is highly valuable for research in RAN4.

Proposal 6: Spatial domain RX beam sweeping reduction use case is worth studying in RAN4.

Proposal 7: AI/ML based RLM/BFD/CBD, LTM by measurement prediction use case is worth studying in RAN4.

Proposal 8: AI/ML based SCell-related enhancements use case is expected to provide little benefit.

Proposal 9: AI-based prediction for enhanced s-measure mechanism is a suitable use case for applying AI/ML.

Proposal 10: AI/ML-based AGC use case is not expected to provide a meaningful advantage.

Proposal 11: We would like to deprioritize AI/ML-based DL synchronization, as it is unlikely to deliver high performance and is expected to incur a significant penalty for false alarms.

Proposal 12: “AI abstraction of the wireless environment to facilitate data scheduling and network power saving” issue should either be clarified further or deprioritized.

Reference

[1] R4-2514637, “WF on 6G AI”, Samsung