3GPP TSG-RAN4 Meeting #117	R4-2520737

Dallas, US, 17th – 21st November, 2025

Agenda item:			7.8.3

Source:	vivo

Title:	Discussion on LBCA R20

Document for:	Discussion

- Introduction

In RAN4#116 bis meeting, the WF [1] captures the agreements in RAN4#116bis meeting, based on it we provide some views in this paper.

Issue 1: Work scope clarification - Do not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19 - Restrict the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier Issue 2: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - The main path antenna is shared across Case 1 and Case 3 switching states - The diversity path antenna is shared across Case 1 and Case 3 switching states- UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states - An example of the front end filter architecture for the case of adjacent bands is provided below for information Issue 3: Open issues related to the UE reference architecture - Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation - Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination - Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above - Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated Issue 4: Switching period for switching between Case 1 and Case 3 - Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations - Switching period capability values {35us, 70us, 140us} - The capability shall be per band pair per band combination Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3 - Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching Issue 6: Open issues related to the switching time mask for switching between Case 1 and Case 3 - Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3 - Whether confirmation by RAN1 is needed on any assumptions made by RAN4 - How to verify for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests Issue 7: Possible co-existence issues - The preliminary analysis of possible co-existence issues is summarized in the table below for information only (the intention is to give companies a starting point for their analysis) - Companies are encouraged to provide their analysis of the issue in the future meetings to allow RAN4 to confirm the co-existence scenarios and to make progress toward identifying possible mitigation - It is recommended to arrange future discussion of this issue in the Rel-19 maintenance agenda for the NR_LBCA_Sw work item Scenario; Type of interference; CA_n12-n29; CA_n28-n67. UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation); Co-channel; UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, it may be feasible to mitigate the situation thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)). Blocking; Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out. BS to UE / UE to BS; Given that co-location is assumed, all those wouldn’t have an issue.. BS to BS; Co-channel interference and blocking; Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out.. Issue 8: Impact on RX requirements - Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking Issue 9: Related to configuration - Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching

- Discussion

## 2.1 Discussion on frequency ranges

Issue 3: Open issues related to the UE reference architecture - Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation - Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination - Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above - Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated

Restriction of band operation

Currently, there is a potential consensus regarding the implementation of shared channel selection filters for downlink two adjacent bands in CA_n12A-n29A and CA_n28A-n67A configurations. The corresponding reference architecture is shown in Figure 1 below[1]. Based on this shared filter architecture and the relative frequency relationship between the two adjacent downlink bands in each band combination, considerations need to be given to imposing restrictions on the frequency scheduling range of the downlink bands.

Fig 1. An example of the front-end filter architecture[1]

Take CA_n28A-n67A as an example, the cons of full band n28 implementation is very obvious:

The full band n28 implementation need a wider Rx filter with the relative bandwidth as high as 8.4%, which is much stricter than the current worst filter of n105, the design is extremely challenging. Even though the architecture would switch between n28UL+ n28DL and n28DL+n67 SDL and it is more like TDD implementation, the UE-UE coexistence issue is unignorable. The n28UL range overlaps with n67DL so the UL transmits signals from other adjacent UEs would aliase with the SDL receiving signals. And the high-power signal of n28UL leaking directly into the filter would cause the LNA to saturate and causing significant interference to n28DL. The baseband section is unable to address the blocking interference that has occurred in the RF stage.

If restricting the operation of n28 to Region 1 range 703-733/758-788 MHz, the filter bandwidth of the DL filter has become relatively narrower, requiring only 50M. The relative bandwidth is only 6.55%, similar to that of the n105UL filter.[2] This significantly reduces the design complexity. There is a 5M gap between the range of DL and UL and could prevent the power of UL from directly leaking into the range of DL, thereby improving the performance of devices such as LNA, ADC. At the same time, there are also similar implementations in the current spec, such as the 5.5A configurations for CA in 38101-1:

For a higher order band combination of which CA_n20-n28 is a subset, the frequency range in band n28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

For CA_12A-n29A, it is also needed to restrict the operation range of n29DL as the gap between it and n12UL is only 1M and the impact of UL signal is more unignorable.

Proposal 1: For LBCA switching for the dual DL scenario, considering sufficient isolation of UL and DL the operation of n29 and n28 should be restricted to a certain range,

- For CA_n28A-n67A, restrict n28 operation to Region 1 range 703-733/758-788 MHz as a starting point.

- For CA_12A-n29A, further study is needed.

## 2.2 Discussion on Rx requirements & architecture

Issue 2: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - The main path antenna is shared across Case 1 and Case 3 switching states - The diversity path antenna is shared across Case 1 and Case 3 switching states- UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states - An example of the front end filter architecture for the case of adjacent bands is provided below for information Issue 8: Impact on RX requirements - Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking

Impact on REFSENS

The extent of sharing in the front-end RF components needs to be discussed. If the two DL CCs are separated after the LNA and before the LO, the impact on performance metrics is relatively minor. The primary issue arises from the increased interference introduced by the shared wideband channel filter.

On one hand, a wider filter may introduce higher insertion loss, which increases the overall noise figure of the system. Additionally, the wider bandwidth filter allows more thermal noise to pass through, further degrading the SNR. Although independent LO and baseband filters may provide some compensation, the noise has already been amplified in the RF front-end.

With introduction of the wider filter, the Tx rejection of shared Rx chain would be much worse. Not only SDL carrier would still impacted by UL Tx, FDD DL carrier may also be impacted due to worse filter rejection ratio toward UL Tx transmission. It is foreseen the DL AGC would need to be adjusted to tolerate larger Tx leakage portion thus the gain setting would become lower that brings noise level raised up for the two DL CCs.

Other RF requirements such as ACS, IBB and OBB are all related to REFSENS, therefore they could be further evaluated based on the update of REFSENS.

Proposal 2: Considering the limitation of wider DL filters in suppressing closer UL, the REFSENS would be impacted and should be firstly studied, the performance of ACS, IBB and OBB could be further analysed based on relaxation of REFSENS.

The impact of further sharing of RF front-end components

When considering further device sharing for DL branch such as LO, image issues between the two DL CCs will also arise.

Below is the calculation formula of IRR[3]: Clearly, if there was no amplitude imbalance present (ΔA = 0) between the I and Q branches of LO, the power ratio would be infinite. Due to the finite amplitude balance, a fundamental image rejection IRRfund is present. Furthermore, if the SCC (or any interference signal at the frequency area reserved for SCC) has ΔP dB larger received power compared to PCC, the total image rejection is decreased by the amount of ΔP. Likewise, reception of SCC is corrupted if PCC has larger reception power.

From the analysis above it can be seen that, IRR is related to the fundamental frequency dependent quadrature accuracy of the receiver analog parts, LO signal generation etc. If the LO is rather ideal, IRRfund would be higher. At the same time, if power imbalance of the two CC (i.e., ΔP) is very small, IRR would also be high enough to guarantee the rejection effect to image signal.

According to the WID, the co-located and synchronized network deployment for both carriers could be implemented, therefore a relative low PSD difference between the two DL CCs could be expected. Referring to the study of R19 fragmented carrier SI , as long as the PSD difference between the two DL CCs under the collocated condition does not exceed 6dB, it is not a big problem to reuse the IRR of 25dBc under the LO sharing state.

Therefore, the upper limit of two DL CC received PSD imbalance should be further studied and restricted. At the same time, IRR is a LTE requirement and not inherited to 5G. Whether to specify a new requirement or test case to measure the REFSENS degradation when image issues exist needs further discussion.

Observation 1: If the PSD difference between the in-gap interference and the target CC is sufficiently low, and the LO is ideal enough, then image rejection will not be a big issue.

Proposal 3: For the architecture of LO sharing between two DL CCs, the upper limit of two DL CC received power spectral density imbalance should be studied.

Proposal 4: When the LO is shared between two DL CCs, further discussion is needed on whether to define a new requirement or test case for REFSENS degradation due to image interference.

When the receiving architecture is shared from the LO to LPF and ADC, it is similar to the ‘one RX RF chain architecture’ in R19-fragmented carrier topic while in-gap interference is avoided. Filtering can only rely entirely on digital filters, leading to further degradation in reception performance. Increased Tx leakage entering the baseband via the LPF compresses the ADC's dynamic range. To maintain signal integrity, the AGC must proactively lower the gain, which consequently raises the system noise floor.

As a reference, during the discussion of R19-FC topic, some evaluation of REFSENS deterioration for CA_n26(2A) is excerpted as follows[4]. Compared with traditional partially shared RX RF chain of intra-band NC CA, the sharing of mixer, LPF and ADC would cause ΔRIBNC of SCC deteriorating by 1dB, and ΔRIBNC of PCC deteriorating by 13.9dB. The gap between n26 UL PCC and DL SCC is 10MHz, and the aggregated DL CBW is as wide as 35MHz.

Compared to CA_n28A-n67A with restriction allocation of n28, the gap between n28UL and n67DL is narrower, at 5M, and the possible largest aggregated DL CBW is 50MHz, for some allocation cases a worse REFSENS performance could be foreseen. Therefore, it is recommended to separate the RX chains for the two DL CCs after LNA to guarantee the receiving performance.

Figure 2: Illustration for CA_n26(2A) when sharing LO between the two DL CCs

Table 7.1.7-1: Proposed ΔRIBNC for example bands on Rx RF chain model [4]

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 13.9 | 26.2 | FDD |

Table 7.3A.2.2-1: Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink. [5]

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |

Observation 2: The sharing of other RF front-end components including the LO, LPF and ADC will further deteriorate the performance of the received signal, larger relaxation of REFSENS is needed.

Proposal 5: RAN4 requirement development should only consider to separate the RX chains for the two DL CCs after LNA.

- Conclusion

Proposal 1: For LBCA switching for the dual DL scenario, considering sufficient isolation of UL and DL the operation of n29 and n28 should be restricted to a certain range,

- For CA_n28A-n67A, restrict n28 operation to Region 1 range 703-733/758-788 MHz as a starting point.

- For CA_12A-n29A, further study is needed.

Proposal 2: Considering the limitation of wider DL filters in suppressing closer UL, the REFSENS would be impacted and should be firstly studied, the performance of ACS, IBB and OBB could be further analysed based on relaxation of REFSENS.

Observation 1: If the PSD difference between the in-gap interference and the target CC is sufficiently low, and the LO is ideal enough, then image rejection will not be a big issue.

Proposal 3: For the architecture of LO sharing between two DL CCs, the upper limit of two DL CC received power spectral density imbalance should be studied.

Proposal 4: When the LO is shared between two DL CCs, further discussion is needed on whether to define a new requirement or test case for REFSENS degradation due to image interference.

Observation 2: The sharing of other RF front-end components including the LO, LPF and ADC will further deteriorate the performance of the received signal, larger relaxation of REFSENS is needed.

Proposal 5: RAN4 requirement development should only consider to separate the RX chains for the two DL CCs after LNA.

- References

- R4-2515156, NR_LBCA_Sw_Enh WF, Moderator (Apple), RAN4#116bis.

- R4-2513810, Feasibility of LB switching FDD+SDL example combinations, Skyworks Solutions, Inc., RAN4#116bis.

- R4-102976, Considerations on retuning for deactivated SCell and related implications, Nokia Corporation, RAN4 #56

- TR38755, Study on NR FR1 DL fragmented carriers.

- TS38.101-1.