3GPP TSG-RAN WG4 Meeting # 117	R4-2520743

Dallas, US, Nov. 17th – 21st, 2025

Source: 	vivo

Title: 	Discussion on FDD HPUE duty cycle

Agenda Item:	10.2.2

Document for:	Approval

Introduction

In RAN4#116, a WF [1] has been submitted. The related issue has been submitted into several other agendas. A related CR was also submitted into [2].

Background:

TS 38.101-1 has a conflict: the maximum output power and configured maximum output power requirements specify a power reduction for PC2 UEs when default duty cycle of 50% is exceeded, unless support for larger duty cycle is signalled. However, no RMC with reduced duty cycle is specified resulting in test condition being in conflict from core specification.  A specification update is necessary to align core specification and testing.

There are different interpretations on what SI conclusion in TR 38.861 means. However, implementations always follow normative specifications independent of TR contents.

Way forward: In future meetings to ensure ran4 specification is correct, choose one of the following alternatives

-	Option 1: Introduce new RMC with up to 50% duty cycle for PC2 FDD, starting point is R4-2511382

-	Option 2: Restrict duty cycle applicability only to TDD, changes needed in 6.2.1 and 6.2.4 and RAN2 capability descriptions. Inform RAN5 that test procedures shall ensure that UE maintains PC2 power level throughout the conformance tests.

Although this WF was not officially agreed, this do provide some background information, and we would like to discuss some history of this issue, and some of the possible way forward.

Discussion

Status & History

Although a background part was provided in the WF, it was still a bit controversial. So some status and history was presented here.

Status:

There is a very widely used condition used in UE RF spec for PC2 UE

“…the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame)…”

In this condition, there is the following behavior:

“-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;”

There are also some capabilities such as “maxUplinkDutyCycle-PC2-FR1” defined, but it seems that this has not been implemented up to now.

Meanwhile, the testing spec such as 38.521-1 never really consider this especially for FDD case.

History Observation:

A brief history has been summarized below, to give a picture why such dispute of PC2 FDD duty cycle related beavior would be emerged in such late stage of 5G NR.

Observation 1. There are following stages before the current dilemma.

Stage 0: Simple in LTE

In LTE, the hpue was defined to have power backoff for certain UL/DL configuration which have larger than 50% duty cycle. It is easy to be controlled by UL/DL configuration, and this is clearly a TDD only feature.

In addition, since the typical UL/DL configuration deployed already satisfy < 50%, the power back off is unlikely to happen. Let alone the fact that the HPUE feature is basely used.

Stage 1: Introduce general term in NR, though only TDD at the time and have ambiguities

In NR R15, it was believed similar back off is needed as LTE.

Since there are flexible symbols and too many configurations in 5G, the current scheme of direct counting duty cycle was used after discussion.

Though at that time only TDD bands were considered, the terms are general and not explicitly preclude FDD. There is also ambiguities of the evaluation period since no conclusion on that can be made.

Stage 2: Standardization of FDD PC2, generally not accept duty cycle solution but wording not changed

During the standardization of FDD PC2, the dispute of whether this duty cycle scheme should be allowed rises. Only P-MPR is agreed as a SAR solution at that time. However, the original wording was never changed.

Maybe because the commercialization of FDD PC2 is slow, this was still not regarded as a serious problem. There was also a maintenance CR at later stage to add TDD restriction on these wordings, but was rejected since may not be necessary.

Furthermore, there was even more discussion to remove all duty cycle related descriptions, but still failed after several meetings dicussion.

Stage 3: Dispute rise again recently since commercialization approaching

With the approaching of commercialization in this year, the problem was raised again. The reference [1][2] are most recent documents for discussion.

Now it seems even more difficult to reach conclusion.

With the above mentioned history, the problem we introduce in the start of NR, now still a problem in the starting of 6G era.

Possible Guideline:

One of the new reasons is that it was found that the implementation of PC2 FDD duplexer is not as easy as expected, particularly for the thermal effect. The introduction of some allowance of duty cycle based power back off may be helpful to ease some implementation concerns, thus can be helpful for the commercialization of FDD HPUE. Of course, this would not be aligned with some long holding understanding that this scheme was not designed for FDD.

Still, since this is the late stage of 5G NR, it might be better to give more allowance to various implementations for these 5G features to facilitate quick deployments.

Proposal 1: It is preferred to give more allowance to various implementations for 5G features to facilitate quick deployments.

Analysis

In the previous WF, two options were provided. Here we listed four options, and did some analysis.

Observation 2: There are four options to treat this issue.

Option A: Restrict the current duty cycle related backoff explicitly to TDD.

(Note: This is the option 2 in previous noted WF.)

Pros: This is aligned with earlier scenario, and would explicitly exclude FDD from utilizing the whole scheme.

Cons: However, his is a stricter way and may posing some challenges for quick deployment of FDD PC2. In addition, this was tried some time before but not agreed. It is doubted that this could be agreed.

Option B: Accept the current duty cycle related backoff can be applied to FDD, while make the backoff optional.

Pros: This scheme has also been mentioned in Rel-19. This can also ease some implementation, while keeping the flexibility that is almost comparable to P-MPR, to alleviate thermal effects etc.

Cons: However, this is a quite significant change, and it is extremely doubtful whether possible to be agreed in such late stage of NR.

Option C: Increase the scope of P-MPR to cover more scenarios, such as for thermal effects.

Pros: This might be the most simple/flexible solution from spec point of view, and don’t have to touch the long contentious duty cycle.

Cons: There should be a discussion on whether thermal effects can be applied to conformance tests or not. If conformance tests are also a bottleneck, further extend the P-MPR to conformance tests might be

Option D: Add a dedicated UL RMC for FDD PC2 that have less than 50% duty cycle.

(Note: This is the option 1 in previous noted WF.)

Pros: This would ensure unified behaviour in conformance tests, no matter what the understanding is, since this is a more relaxed scenario. The thermal effects can also be alleviated. A way might be suitable to current late 5G stage.

Cons: This only tries to circumvent the different understanding, but not to solve it. May not be extendable to 6G and different scheme might have to be considered in the future.

Based on the analysis, the option C and D are preferred from our point of view.

Proposal 2: Consider to use the following two options:

- Increase the scope of P-MPR to cover more scenarios, such as for thermal effects. (Option C)

- Add a dedicated UL RMC for FDD PC2 that have less than 50% duty cycle. (Option D)

Conclusion

This paper discussed the possible way forward to treat FDD PC2 duty cycle issues, and has the following key observations and proposals:

Observation 1. There are following stages before the current dilemma.

Stage 0: Simple in LTE

Stage 1: Introduce general term in NR, though only TDD at the time and have ambiguities

Stage 2: Standardization of FDD PC2, generally not accept duty cycle solution but wording not changed

Stage 3: Dispute rise again recently since commercialization approaching

Proposal 1: It is preferred to give more allowance to various implementations for 5G features to facilitate quick deployments.

Observation 2: There are four options to treat this issue.

Option A: Restrict the current duty cycle related backoff explicitly to TDD.

Option B: Accept the current duty cycle related backoff can be applied to FDD, while make the backoff optional.

Option C: Increase the scope of P-MPR to cover more scenarios, such as for thermal effects.

Option D: Add a dedicated UL RMC for FDD PC2 that have less than 50% duty cycle.

Proposal 2: Consider to use the following two options:

Increase the scope of P-MPR to cover more scenarios, such as for thermal effects. (Option C)

Add a dedicated UL RMC for FDD PC2 that have less than 50% duty cycle. (Option D)

- References

[1] R4-2511777, WF on FDD PC2 duty cycle and measurement channel, RAN4#116

[2] R4-2511382, (NR_PC2_UE_FDD-Core) CR to TS 38.101-1: Addition of missing RMC, RAN4#116