3GPP TSG-RAN WG4 Meeting #117			                                              R4-2520758

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.7.4

Source: 	CMCC

Title: 	Discussion on UE RF core requirements for UAV

Document for:	Approval

# 1. Introduction

The last meeting did not conduct much discussion on UAV RF part, and only reached some general agreements. From our perspective, some of these agreements need to be further clarified. This document will focus on UAV UE RF requirements.

# 2. Discussion

## 2.1. General aspects

Last meeting, we have the following agreement:

- Agreements: - The high-power UAV UE applies for the listed example bands in Rel-20 WID. - RF requirements need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors. - The same UE can support either omni-directional antenna and phase array antenna.

For the first bullet, it mentions the example band of high-power UAVs. Generally, this sentence is correct. However, as the research progresses, it cannot be ruled out that operators may have new deployment request during the study in Rel-20. Therefore, in the future, the request from operators in other frequency bands should also be considered.

Proposal 1: The example band should also take into account the new requests raised by operators during R20.

## 2.2. Maximum output power

- Agreements: - RAN4 to specify maximum output power for UAV higher power UE considering the following two options: - Option 1: Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG) - Option 2: Introduce a new power class for UAV UE for up to 33dBm max total conductive output power for the example bands (Similar as legacy TN UE power class definition) - Option 3: Other options are not precluded.

There are still some discussions regarding the UAV maximum output power. The main point lies in the output power reporting method. Without changing the power limit of 33dBm, since the current maximum output power of UE PC1 is 31dBm, introducing a new capability or a new power class is two relatively good solutions. We prefer to introduce a new capability. Firstly, as ATG is another terminal with an output power exceeding 31dBm, the introduction of its output power capability can serve as a reference for UAV. Secondly, in R20, there are also discussions about 32dBm high-power terminals and PC1. If a new power class is introduced for UAV at this time, the situation will become even more complicated. In conclusion, we recommend choosing option 1.

Observation 1: Discussions about high-power terminal 32dBm and PC1 are also underway in R20.

Proposal 2: It is suggested to Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG).

## 2.3. UAV UE Rx requirements

- Agreements: - FFS on how to specify ACS requirements

Unlike the UAV Tx requirements, in this WI, the main objective is the impact of increasing Tx power on the UAV. Therefore, the current legacy Rx requirement for handheld UE could be used as the starting point. It is worth noting that it is necessary to clearly define the analysis scenarios and related RF requirements in the coexistence part. Once the scenarios and related RF requirements are defined, the further discuss on this topic.

Proposal 3: Legacy Rx requirement for handheld UE could be used as the starting point, and postpone the discussion on ACS until the coexistence part reaches a conclusion.

# 3. Conclusions

In this contribution, UAV UE RF requirements are discussed with following proposals:

Proposal 1: The example band should also take into account the new requests raised by operators during R20.

Observation 1: Discussions about high-power terminal 32dBm and PC1 are also underway in R20.

Proposal 2: It is suggested to Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG).

Proposal 3: Legacy Rx requirement for handheld UE could be used as the starting point, and postpone the discussion on ACS until the coexistence part reaches a conclusion.

# 4. Reference

[1] R4-2515159, Way Forward for [116bis][319] NR_UAV, Qualcomm Incorporated, CMCC