3GPP TSG-RAN WG4 Meeting #117		                           R4-2520760

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.6.4.1

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on Ku-band HD-FDD VSAT

Document for:	Discussion

1	Introduction

In RAN#109, a new WID was approved on enhanced requirements for NR NTN and IoT NTN phase 2 [1]. And its objective is as follows.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

This contribution provides our views on Phase 1 of this objective.

2	Discussion

2.1	HD-FDD requirements

In RAN4#116bis, RAN4 discussed to introduce HD-FDD for VSAT types 2, 3, 5, 6. And regarding EIRP, EIS requirements, the following WF was agreed [2].

Issue 1-1-5: EIRP requirements for HD-FDD - WF - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point - FFS on whether to modify. Issue 1-1-6: EIS requirements for HD-FDD - WF - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point - FFS on whether to modify.

Generally, Tx and Rx signals are divided by switch in HD-FDD systems, and its front-end insertion loss is reduced compared to FD-FDD systems which use a duplexer. So we could expect to improve EIRP and EIS by introducing HD-FDD. Nevertheless, in case of NTN VSAT, this principle may not necessarily conform due to the following reasons.

- Many NTN VSATs have separate Tx and Rx antennas, so they do not use (or require) a duplexer. Then the assumption that performance improvement can be expected by removing duplexer is not reasonable.

- Compared to the FD-FDD single antenna panel, removing duplexer may improve front-end insertion loss. However, this applies only when filters are not utilized in HD-FDD systems. If filters are installed, the performance difference compared to the duplexer-equipped FD-FDD becomes small.

Due to the above reason, In the case of VSAT, any performance differences between FD-FDD and HD-FDD are unlikely to be substantial enough to require changes to the EIRP and EIS requirements.

Observation 1:	In the case of VSAT, any performance differences between FD-FDD and HD-FDD are unlikely to be substantial enough to require changes to the EIRP and EIS requirements.

Proposal 1:	To reuse existing FDD VSAT EIRP and EIS requirement for HD-FDD VSAT type 2, 3, 5, 6.

2.2	VSAT with small antennas (e.g., 20x20cm)

Regarding the assumptions for VSAT with small antenna, the following WF was agreed [2].

Issue 1-2-2: Assumptions for VSAT with small antenna - WF - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects: - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO.

This paper provides our views on antenna size, number of antenna elements, antenna gain and Tx power per element as follows. First, the following table shows the assumption of Ku-band VSAT.

Table 2.2-1: The assumption of Ku-band VSAT

|  | Type5 | Type6 |
| --- | --- | --- |
| Minimum Peak EIRP [dBm] | 70 | 60 |
| Max TRP [dBm] | 47 | 40 |
| Maximum EIRP [dBm] | 88.2 | 76.6 |
| Antenna array configuration | 78 x 78 elements | 45 x 45 elements |
| # of ANT elements | 6,084 | 2,025 |
| Element gain [dBi] | 3.5 | 3.5 |
| Array gain [dB) | 37.8 | 33.1 |
| Realized antenna gain [dBi] | 41.3 | 36.6 |
| Array Ohmic loss [dB) | 2.0 | 2.0 |
| Conducted power (before Ohmic loss) per antenna element [dBm] | 9.0 | 6.9 |
| # of ch | 6,084 | 2,025 |
| Tx Transmit power [dBm] | 46.8 | 40.0 |
| EIRP [dBm] | 86.2 | 74.6 |

Even for LEO-only Type 6, the antenna dimensions are approximately 60 cm × 60 cm for downlink and 48 cm × 48 cm for uplink. In contrast, the target size for a small VSAT antenna is approximately 20 cm × 20 cm. In an HD-FDD system, where the antenna is shared between transmitter and receiver, arranging the antenna elements at half-wavelength spacing corresponding to 12.6 GHz (the center frequency for both UL and DL) results in a 17 × 17 element array. This configuration yields an overall antenna size of approximately 20.2 cm × 20.2 cm, which closely matches the target dimension.

In this case, considering only the number of antenna elements, a 45 × 45 element array is reduced to a 17 × 17 element array, resulting in an array gain reduction of 8.5 dB—from 34.6 dB to 24.1 dB. This represents a significant performance degradation due to VSAT miniaturization. Furthermore, not only is the array gain reduced, but the number of transmit channels also decreases proportionally with the number of antenna elements, leading to an additional 8.5 dB reduction in transmit power.

However, with respect to transmit power, the 6.9 dBm per channel used in the 45 × 45 element array is too low. This value was constrained by the need to meet the 40 dBm upper limit for TRP in that configuration. Currently available beamformer ICs are capable of higher output levels, with 15 dBm per channel being relatively common. Therefore, although somewhat conservative, Table 2.2-2 presents the estimated EIRP for an array size close to 20 cm, assuming a transmit power of 13 dBm per channel.

Table 2.2-2: EIRP estimation for VSAT with small antennas.

| Array configuration | 12 x 12 | 14 x 14 | 16 x 16 | 17 x 17 | 18 x 18 |
| --- | --- | --- | --- | --- | --- |
| # of elements | 144 | 196 | 256 | 289 | 324 |
| Antenna size [cm] | 14.3 | 16.7 | 19.0 | 20.2 | 21.4 |
| Element gain [dBi] | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| Array gain [dB] | 21.6 | 22.9 | 24.1 | 24.6 | 25.1 |
| Realized antenna gain [dBi] | 25.1 | 26.4 | 27.6 | 28.1 | 28.6 |
| Array Ohmic loss [dB] | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 |
| Tx Transmit power [dBm] | 34.6 | 35.9 | 37.1 | 37.6 | 38.1 |
| EIRP[dBm] | 57.7 | 60.3 | 62.7 | 63.7 | 64.7 |

With a 17 × 17 element array, TRP is 37.6 dBm and EIRP is 63.7 dBm. When designing a VSAT using commercially available beamformer ICs, a single IC typically supports either 4 or 8 antenna elements. From this perspective, a 16 × 16 array offers better implementation flexibility than a 17 × 17 array. Even with a 16 × 16 array, the EIRP reaches 62.7 dBm.

However, this EIRP value corresponds to an elevation angle of 90 degrees, i.e., when the antenna beam is aligned with the boresight. Since VSAT is required to support elevation angles as low as 35 degrees, it must be considered.

As presented by LG, the attenuation characteristics with respect to steering angle can be modeled using the Cosⁿ(θ) function [3]. Our measurements of patch antennas indicate attenuation characteristics that are close to those modeled by n = 2. Accordingly, the EIRP at an elevation angle of 35 degrees was evaluated assuming n = 2. The evaluation results are summarized in Table 2.2-3.

Table 2.2-3: EIRP estimation for VSAT with small antennas.

| Array configuration | 12 x 12 | 14 x 14 | 16 x 16 | 17 x 17 | 18 x 18 |
| --- | --- | --- | --- | --- | --- |
| # of elements | 144 | 196 | 256 | 289 | 324 |
| Antenna size [cm] | 14.3 | 16.7 | 19.0 | 20.2 | 21.4 |
| Element gain [dBi] | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| Array gain [dB] | 21.6 | 22.9 | 24.1 | 24.6 | 25.1 |
| Realized antenna gain [dBi] | 25.1 | 26.4 | 27.6 | 28.1 | 28.6 |
| Array Ohmic loss [dB] | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 |
| Tx Transmit power [dBm] | 34.6 | 35.9 | 37.1 | 37.6 | 38.1 |
| EIRP[dBm] @90deg. | 57.7 | 60.3 | 62.7 | 63.7 | 64.7 |
| EIRP[dBm] @35deg. | 52.8 | 55.5 | 57.8 | 58.9 | 59.9 |

Even for the largest 18 × 18 array, the estimated EIRP is 59.9 dBm, which falls short of the Type 6 minimum peak EIRP requirement of 60 dBm. Applying the Type 6 requirement to VSAT with small antennas is challenging; therefore, it is necessary to consider relaxed requirements that align with antenna miniaturization.

On the other hand, 16 × 16 and 17 × 17 arrays have the potential to achieve EIRP values close to the Type 6 minimum peak EIRP of 60 dBm. Accordingly, rather than applying excessive relaxation, it is more appropriate to determine the EIRP and EIS requirements for small VSAT antennas based on the performance needed in link budget evaluations.

Observation 2:	Expected EIRP is 58.9 dBm with 17 x 17 array, 57.8 dBm with 16 x16 array at 35 degree elevation angle. Considering the design flexibility 16 x 16 array would be more preferred.

Observation 3:	It is challenging to apply the requirement of Type6 to VSAT with small antenna, it is necessary to consider relaxed requirements that align with antenna miniaturization.

Proposal 2:	It is more appropriate to determine the EIRP and EIS requirements for small VSAT antennas based on the performance needed in link budget evaluations.

3	Conclusion

This contribution provides our views on HD-FDD requirements and VSAT with small antennas.

Observation 1:	In the case of VSAT, any performance differences between FD-FDD and HD-FDD are unlikely to be substantial enough to require changes to the EIRP and EIS requirements.

Proposal 1:	To reuse existing FDD VSAT EIRP and EIS requirement for HD-FDD VSAT type 2, 3, 5, 6.

Observation 2:	Expected EIRP is 58.9 dBm with 17 x 17 array, 57.8 dBm with 16 x16 array at 35 degree elevation angle. Considering the design flexibility 16 x 16 array would be more preferred.

Observation 3:	It is challenging to apply the requirement of Type6 to VSAT with small antenna, it is necessary to consider relaxed requirements that align with antenna miniaturization.

Proposal 2:	It is more appropriate to determine the EIRP and EIS requirements for small VSAT antennas based on the performance needed in link budget evaluations.

4	References

RP-252962, New WID: Enhanced requirements for NR NTN and IoT NTN Phase 2, moderator (Qualcomm)

R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek

R4-2513711, On Ku-band VSAT Half-Duplex RF requirements with FR1-Numerology, LG Electronics