3GPP TSG-RAN WG4 Meeting#117		R4-2520763

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.2.3

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G channel bandwidth

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about RAN4 6G system parameters was approved. In this contribution, we want to continue to provide some views on 6G channel bandwidth and others.

Discussion

Channel bandwidth

Max channel bandwidth, FFT and numerology

- Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

For FDD, max CBW is 50MHz in 5G, the largest spectrum available for n7 is 70MHz, the largest spectrum available for n65 is 90MHz, there are no FDD bands can support 100MHz CBW. In 6G, according to the guidance from RAN4, we need to consider FFT size, max CBW and numerology as a frame work to evaluate. For FDD, we need to keep a single SCS. Compared to 30 KHz SCS, 15 kHz can have more spectrum utilization. We prefer to define 15 kHz for FDD. Regarding the max CBW, if the FFT size is 8192, the max CBW with 15 kHz can be 100MHz. However, there are no bands can go up to 100MHz for FDD. We prefer to 50MHz for FDD, then we can saving power consumption if the FFT size is 4096, and it is applicable to most of FDD bands.

Proposal 1: We prefer to define 50MHz for FDD max CBW, 15 kHz SCS with 4096 FFT size for FDD bands in 6GR.

For TDD, max CBW is 100MHz in 5G, the largest spectrum available for n77 is 900MHz. In 6G, new candidate spectrum can be higher. The advantage of TDD band is its large bandwidth and high throughput, hence, we can define bigger maximum CBW for TDD. According to the agreements, we can compare two options between 200MHz and 400MHz from an implementation perspective. We can analyse it from the following four aspects. Firstly, from baseband processing, the complexity calculation of 400MHz will be increased significantly compared to 200MHz. Secondly,  from ADC and DAC design, generally, in order to obtain better performance, the sampling rate needs to be 2 times, 4 times, or even higher than CBW. Thirdly, from RF front-end design, especially for flatness, including output power and EVM, it is a challenge for UE implementation. Fourthly, PAPR is higher than 0.5~1dB compared to 400MHz. lastly, the power consumption will be increased from 60% to 100% compared to 200MHz. In addition, the most important is user experience data rate rather than peak data rate according to the IPR in 6G SID. Thus, For TDD bands in FR1 and around 7GHz, we prefer to define 200MHz as max CBW. For SCS, the existing SCS is 30 kHz and 60 kHz in sub-6GHz. We have simulated and results are shown in Figure 2 for around 7GHz .It is observed that 30 kHz exhibits better BLER performance. In addition, 30 kHz SCS has higher spectrum utilization compared to 60 kHz SCS. Therefore, we prefer to 30 kHz for TDD bands and around 7GHz. For 30 kHz with 8192 FFT size, it can be up to 200MHz.

Figure1: PAPR for 200MHz and 400MHz

Figure 2: Comparison between 30 kHz SCS and 60 kHz SCS for BLER performance

Proposal 2: For TDD bands in FR1 and around 7GHz, we prefer to define 200MHz as max CBW, maximum FFT size is 8192 and SCS is 30 kHz.

If 400MHz needs to be considered and evaluated, agreement has provided some options as follows.

Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

Based on the above analysis, we prefer option3. For option3, there is no significant difference for spectrum utilization between option1/2 and option3. And the ADC/DAC/FFT design can be reduced in complexity. In addition, we can consider to support different NW and UE max CBW. Different NW and UE max CWB exists in 5G, such as Redcap UE. We can support NW for 400MHz max CBW and UE max CBW for 200MHz.

Proposal 3: If 400MHz needs to be studied, 1x8k_FFTx2CC can be supported. In addition, we can support NW for 400MHz max CBW and UE max CBW for 200MHz.

In order to simplify the design of 6G system, one single numerology and SCS per band/frequency range is necessary.

Proposal 4: One single numerology and SCS per band/frequency range is necessary.

Min channel bandwidth

- Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

In 5G, the min channel bandwidth is 3MHz which is introduced in later release. From 6G perspective, we need to consider as following two aspects.

Firstly, for initial access. Min CBW is related with SSB design. If we need to keep the same principle of sync raster like 5G in 6GR, min CBW decides the sync raster granularity. Min CBW is bigger, the sync raster is sparser.

Lastly, for spectrum allocation, it is smaller than 5MHz for operators’ spectrum. From operators’ perspective, we need to allow 3MHz cell to be accessed to the 6GR.

Based on the above analysis, 5MHz as min CBW could be considered as a general baseline, for 3MHz in particular bands, we can consider punching or re-tuning schemes. It can be pending on RAN1’s progress.

Proposal 5: 5MHz could be considered as a general baseline min CBW, while 3MHz is allowed for particular bands.

Spectrum utilization

- Agreement - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

It is observed that spectrum utilization of smaller bandwidth is less than 95%. How to optimize spectrum utilization? There are some advanced confinement techniques like windowing or filtering at the transmitter or receiver [2]. These techniques are all applied to waveforms. Out-of-band emission can be optimized by them. Figure 2 shows that a time domain windowing to the CP-OFDM. Figure3 shows that the filtering is applied to a subband consisting of a group of subcarriers. In signal processing and digital communications, it is commonly known that the use of a matched filter is optimal for maximizing the SNR in the presence of additive stochastic noise. A receiver matched filter is equivalent to the conjugated complex time-reversed version of the filter which has been applied at the transmitter side. To be able to apply a matched filter at the receiver side, the filter impulse responses of all possible filters either have to be specified leading to a huge look-up table and high complexity or have to be signalled to receiver side leading to a huge signalling overhead. We need to consider the trade-off between spectrum utilization and UE/BS complexity. Of course, these techniques are needed to coordinate with RAN1 on different waveforms.

Figure2: window-OFDM block diagram                                          Figure3: filtering-OFDM block diagram

Observation1: Advanced confinement techniques like windowing and filtering will cause high implementation of UE and BS side and signalling overhead.

Before evaluating and studying the SU enhancement techniques, a set of common simulation assumptions for SU evaluation is needed. Including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). PA models may need to be updated with new PA models section. In addition, No matter how to achieve optimization, we should ensure that the out-of-band emission and REFSENS will not deteriorate.

Proposal 6: Out-of-band emission and REFSENS will not deteriorate are the prerequisites of spectrum utilization enhancement.

Proposal 7: We can postpone the discussion about spectrum utilization until new PA models and RAN1 progress on waveform.

Table1: CBW and spectrum utilization

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| SU | 90.00% | 93.60% | 94.80% | 95.40% | 95.76% | 96.00% | 96.69% | 97.20% | 96.80% | 97.20% |  |  |  |  |  |  |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| SU | N/A | 79.20% | 86.40% | 91.20% | 91.80% | 93.60% | 93.60% | 94.63% | 95.40% | 94.40% | 95.76% | 97.20% | 97.20% | 97.65% | 98.00% | 98.28% |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| SU | N/A | N/A | 79.20% | 86.40% | 86.40% | 89.28% | 91.20% | 90.51% | 91.80% | 92.80% | 93.60% | 94.80% | 95.66% | 96.30% | 96.80% | 97.20% |

Asymmetric channel bandwidth

- Agreement - Study the need and feasibility to enable asymmetric CBW

Asymmetric channel bandwidth exists in 5G, which comes from the operator’s request. Asymmetric channel bandwidth is applied to some specific FDD bands. The purpose of asymmetric channel bandwidth is to meet different throughput demand for UL and DL. At the same time, it can save power consumption to a certain extent. However, the default TX-Rx frequency separation will be changed. There is no fixed and only one TX-RX frequency separation. And it will impact the performance of duplex filter. If we want to save power consumption, BWP is a good choice. RAN4 can define based on specific request and specific bands if the operator has some request about asymmetric channel bandwidth, there is no need to study to enable asymmetric channel bandwidth in 6G SID.

Proposal 8: There is no need to study to enable asymmetric channel bandwidth in 6G SID. RAN4 can define based on specific bands if the operator has requests about asymmetric channel bandwidth.

Irregular channel bandwidth

- Agreement - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths - Other options not presented in this meeting are not precluded

Irregular channel bandwidths come from operators’ requests to fully utilize spectrum resources. Due to the scarcity of spectrum resources, which causes the spectrum fragmentation of many operators’ spectrum. Operator’s spectrum will not be the same as the CBW defined in RAN4. Based on the TR 38.844, there are four solutions as follows. Solution1: using the next large channel bandwidth than licensed BW, this solution is to use the next large channel bandwidth for BS and irregular channel bandwidth allocation for UE. Wider channel filter for BS is applied. Channel filter for UE is up to implementation. Due to the wider channel filter, there is performance loss. And UE have to support asymmetric bandwidth. Solution2:  Overlapping UE CBWs from NW perspective, NW has to support irregular bandwidth from NW perspective. Each UE supports the licensed bandwidth from UE perspective. There is no change for UE implementation. Solution3: Combined UE CBW, studied spectrum blocks covered by “main RF carrier” and “additional RF carrier”, from UE perspective, supported in downlink only. Solution4: overlapping CA from NW perspective and UE perspective. Solution3 and Solution4 will increase the complexity for UE. In summary, how to fully use the irregular channel bandwidth and avoid the UE implementation complexity and test burden in 6GR? In our understanding, from NW perspective, which can support irregular channel bandwidth with different solutions based on above to fully use the irregular channel bandwidth to guarantee the system capacity. From UE perspective, we can define scalable channel bandwidth and scalable guard band based on existing licensed channel bandwidth for irregular channel bandwidth. UE channel filter based on next large channel bandwidth to reduce the UE implementation complexity rather than irregular channel filter.

Proposal 9: From UE perspective, we can define scalable channel bandwidth and scalable guard band based on existing licensed channel bandwidth for irregular channel bandwidth.

Proposal 10: UE channel filter based on next large channel bandwidth can be assumption for defining UE RF requirements.

Conclusions

In this paper, we provide our views on 6G channel bandwidth, we have made the following proposals and observation.

Proposal 1: We prefer to define 50MHz for FDD max CBW, 15 kHz SCS with 4096 FFT size for FDD bands in 6GR.

Proposal 2: For TDD bands in FR1 and around 7GHz, we prefer to define 200MHz as max CBW, maximum FFT size is 8192 and SCS is 30 kHz.

Proposal 3: If 400MHz needs to be studied, 1x8k_FFTx2CC can be supported. In addition, we can support NW for 400MHz max CBW and UE max CBW for 200MHz.

Proposal 4: One single numerology and SCS per band/frequency range is necessary.

Observation1: Advanced confinement techniques like windowing and filtering will cause high implementation of UE and BS side and signalling overhead.

Proposal 6: Out-of-band emission and REFSENS will not deteriorate are the prerequisites of spectrum utilization enhancement.

Proposal 7: We can postpone the discussion about spectrum utilization until new PA models and RAN1 progress on waveform.

Proposal 8: There is no need to study to enable asymmetric channel bandwidth in 6G SID. RAN4 can define based on specific bands if the operator has requests about asymmetric channel bandwidth.

Proposal 9: From UE perspective, we can define scalable channel bandwidth and scalable guard band based on existing licensed channel bandwidth for irregular channel bandwidth.

Proposal 10: UE channel filter based on next large channel bandwidth can be assumption for defining UE RF requirements.

- Reference

- R4-2514650, WF on 6GR system parameter, Huawei, HiSilicon. TSG-RAN4#116bis meeting

- Jamal Bazzi_, Katsutoshi Kusume_, Petra Weitkempery, Kazuaki Takedaz, Anass Benjebbourz “Transparent spectral confinement approach for 5G”.