3GPP TSG-RAN WG4 Meeting#117		R4-2520764

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.2.4

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G channel arrangement

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about RAN4 6G system parameters was approved. In this contribution, we want to continue to provide some views on 6G channel arrangement.

Discussion

Channel arrangement

Channel raster

- Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

The purpose of channel raster is to place the channel. In 5G, channel raster definitions include 100 kHz, 10 kHz and SCS-based. The definition of 100 kHz is to address the co-existence problems. And the definition of 10 kHz is to address the issue of odd RBs in BS and even RBs in UE side and vice versa. SCS-based channel raster can be flexible for wider channel bandwidth configuration. If no defined channel raster in 6GR, operators will not place the channel based on specific rules, the channel will be free, which causes interference between operators. Similarly, in 6GR, 10 kHz is also needed to address the issue of odd RBs in BS and even RBs in UE side and vice versa. For above 3GHz, SCS-based channel raster to address the co-existence problems.

Proposal 1: It is necessary to define channel raster, for re-farming bands with 10 kHz channel raster, using 10 kHz channel raster in 6GR. For other re-farming bands and new bands, SCS-based channel raster can be adopted in 6GR.

Sync. Raster

- Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

In LTE, channel raster and sync raster keep the same. In NR, decouple the channel raster and sync raster to be more flexible deployment and saving cell search time. Similarly, decoupling the channel raster and sync raster is also needed in 6GR. Meanwhile, when one cell is deployed in one channel raster, SSB can be deployed in one sync raster. Mapping relationship needs to be guaranteed between channel raster and sync raster.

Proposal 2: Decouple the channel raster and sync raster to be more flexible deployment. Meanwhile, one cell is deployed in one channel raster, SSB can be deployed in one sync raster in this cell.

In order to save power from the network, many companies think that SSB can be extended (e.g., 80ms/160ms). Along with the expansion of SSB period mentioned, if the sync raster is unchanged, then the cell search time of UE will be longer. User experience in 6GR cannot be worse than 5G, thus, a more sparse sync raster compared to NR for 6GR is necessary. In addition, a more sparse sync raster will save energy consumption and decrease complexity in the cell search of UE. Therefore, 6GR should consider a more sparse Sync raster design, especially for low-capability device types.

However, the principle in NR for sync raster is to guarantee at least one SSB in one CBW. The formula is that sync raster ≤ min CBW- SSB CBW+ channel raster. In 5G, we have defined so many sync raster due to the principle. Nevertheless, according to the survey of the network deployments, operators only use a part of these sync rasters in practically. From this point of view, whether the principle in NR for sync raster to be kept in 6GR needs to be discussed. If we should follow the principle, increasing min CBW or decreasing SSB CBW (depending on RAN1’s design) can make the sync raster be sparser. If we didn’t follow the principle, maybe we could define fixed and limited sync raster to achieve sparse sync raster. We can wait for RAN1’s progress on the definition principle of sync raster and SSB design.

How to develop a flexible framework to assess different sync raster? From RAN4 perspective, we can quantify the time of cell initial search.

Proposal 3: More sparse sync raster design can be considered in 6GR. Whether the principle in NR for sync raster should be followed in 6GR needs to be discussed. We can wait for RAN1’s progress on the definition principle of sync raster and SSB design.

Conclusions

In this paper, we provide our views on 6G system parameters, we have made the following proposals.

Proposal 1: It is necessary to define channel raster, for re-farming bands with 10 kHz channel raster, using 10 kHz channel raster in 6GR. For other re-farming bands and new bands, SCS-based channel raster can be adopted in 6GR.

Proposal 2: Decouple the channel raster and sync raster to be more flexible deployment. Meanwhile, one cell is deployed in one channel raster, SSB can be deployed in one sync raster in this cell.

Proposal 3: More sparse sync raster design can be considered in 6GR. Whether the principle in NR for sync raster should be followed in 6GR needs to be discussed. We can wait for RAN1’s progress on the definition principle of sync raster and SSB design.

- Reference

- R4-2514650, WF on 6GR system parameter, Huawei, HiSilicon. TSG-RAN4#116bis meeting