3GPP TSG-RAN WG4 Meeting#117		R4-2520765

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.2.5

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G device types

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about RAN4 6G system parameters was approved. In this contribution, we want to continue to provide some views on 6G device types.

Discussion

Number of Tx and Rx

- Agreement - Study the framework of spec impact with different number of Tx/Rx - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective. - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design - Co-ordinate with RAN and RAN1 for number of Tx/Rx

Tx/Rx MIMO improves the capacity, coverage and reliability of communication systems exponentially without increasing spectrum bandwidth and transmission power. It is one of the critical technologies of 5G NR. Similarly, it is still one of the critical technologies of 6GR. Regarding the cost and limitation of the form factor, different numbers of Tx/Rx for different device types. Different frequency with different number of Tx/Rx.

As everyone knows, with the increase of frequency, the antenna size becomes smaller, but its free loss will also increase gradually. Hence, with the increase of frequency, the number of Tx/Rx increase to compensate the free loss and increase the throughput. In addition, device size and form factors will limit the number of Tx/Rx and antenna isolation, which impacts the SNR of UL/DL MIMO.

In 5G, handheld UE and 2Tx/4Rx is defined for handheld UE in 5G. Like other types, 1Tx/2Rx for XR and (e) Redcap UE. 1Tx/1Rx for IoT devices in 5G. Regarding around 7GHz and/or around 15GHz will be supported in 6GR, UE architecture will be more complex, and we can reuse the NR for Tx/Rx MIMO as the baseline for 6GR. We can take Table3 as the number of Tx/Rx in 6GR.

Table1: The number of Tx/Rx in 6GR

| Devices | Frequency range | Tx/Rx |
| --- | --- | --- |
| IoT | NA | 1T1R |
| Redcap/XR UE | NA | 1T2R |
| Handheld UE | Sub-1GHz | 1T2R |
|  | Sub-3GHz | 2T4R |
|  | Sub-6GHz | 2T4R |
|  | U6G and around 7GHz | 2T4R or 3T6R |
| FWA UE | Sub-1GHz | 1T4R |
|  | Sub-3GHz | 2T4R |
|  | Sub-6GHz | 2T4R or 3T6R |
|  | U6G and around 7GHz | 3T6R or 4T8R |

Proposal 1: Adopt the Table 1 for Tx/Rx MIMO as the baseline for 6GR.

Device types

- Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

Design Principe

Firstly, with the new devices with urgent market demands (e.g., Redcap and 2RX XR) were introduced in 5G NR later release, which causes network deployment to be more complex. 5G NR cannot provide a good eco-environment for those UE types introduced in later releases.

Secondly, flexible UE capability report mechanism was adopted in 5G NR to support various features/use cases. However, statistically speaking, just only 70% of features are commercialized. Which means various features are over designed.

Lastly, there are various device types applied in 5GNR, which may cause market fragmentation, some device types haven’t formed a large-scale market.

Based on the above analysis, we propose the following design principles.

    - A limited set of device types to avoid market fragmentation

    - Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitations (e.g. form factor)

    - Scalable and forward-compatible design

      - In order to achieve scalable and forward-compatible design, we have defined three kinds of capabilities.

      - A common basic mandatory function set is defined for all device types;

      - On top of that, a different additional mandatory capability set is defined for each device type;

      - Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

Proposal 2: we proposed the design principle for device types as follows.

    - A limited set of device types to avoid market fragmentation in 6G day1

    - Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitations (e.g. form factor)

    - Scalable and forward-compatible design

      - -  A common basic mandatory function set is defined for all device types;

      - -  on top of it, a different additional mandatory capability set is defined for each device type;

      - - Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

6G device types

Based on the above design principles, device types are categorized based on two main usage scenarios, including eMBB and massive IoT, in addition, different form factors with different communication capabilities. Therefore, we propose to define three kinds of device types, including type A/B/C. Type A and type B belong to the eMBB usage scenario, and type C belongs to the massive IoT usage scenario. In order to achieve scalable and forward-compatible design to avoid market fragmentation, we have defined three kinds of capability/feature. A common basic mandatory function set is defined for all device types, such as waveform, frame structure, channel coding, initial access procedure, DL and UL control channel design, scheduling /HARQ operation and other capability/features. On top of that, a different additional mandatory capability set is defined for each device type, further, regarding new usage scenario or new services that will be introduced in the future, and optional additional capability sets are defined for each device type.

Based on the design principle and above analysis, we can take table 5 to define device types as a starting point.

Table 2: 6GR UE/Device type example

| 6G communication usage scenarios | Example device type | Scalable and forward-compatible design to avoid market fragmentation |  |  |  |  | Example devices/use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A common basic mandatory function set for all device types | A different additional mandatory capability set for each device type | Optional capability/feature sets for different new usage scenarios or new services. |  |  |  |
| eMBB | TypeA | - Waveform - Frame structure - Channel coding - Initial access design - DL and UL control channel design - Scheduling/HARQ operation - BS/UE energy saving features etc. | - 200M - 2T/4R - Modulation: 256 QAM DL/UL - Power class: 3 | - Optional set A1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Power class | Set A2 … 4T/8R | … | Smart phone, CPE/FW, VUE, VSAT, Robots… |
|  | Type B |  | - 100M/200M - 1T2R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set B1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Power class Access control, etc. | Set B2 … | … | 2RX XR, High-end watch |
| Massive IoT | Type C |  | - 20M - 1T1R - Modulation:64 QAM DL/UL Power class: 3 or 5 | Optional set C1 - Processing time - Power class - Access control Scaling factor, etc. | Set C2 … | … | Wearables, Industrial sensors, Low-end watch, Video surveillance … |

Proposal 3: The Table 2 could be considered as the starting point to achieve scalable and forward-compatible design.

Conclusions

In this paper, we provide our views on 6G system parameters, we have made the following proposals.

Proposal 1: Adopt the Table 1 for Tx/Rx MIMO as the baseline for 6GR.

Table1: The number of Tx/Rx in 6GR

| Devices | Frequency range | Tx/Rx |
| --- | --- | --- |
| IoT | NA | 1T1R |
| Redcap/XR UE | NA | 1T2R |
| Handheld UE | Sub-1GHz | 1T2R |
|  | Sub-3GHz | 2T4R |
|  | Sub-6GHz | 2T4R |
|  | U6G and around 7GHz | 2T4R or 3T6R |
| FWA UE | Sub-1GHz | 1T4R |
|  | Sub-3GHz | 2T4R |
|  | Sub-6GHz | 2T4R or 3T6R |
|  | U6G and around 7GHz | 3T6R or 4T8R |

Proposal 2: we proposed the design principle for device types as follows.

    - A limited set of device types to avoid market fragmentation in 6G day1

    - Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitations (e.g. form factor)

    - Scalable and forward-compatible design

      - -  A common basic mandatory function set is defined for all device types;

      - -  on top of it, a different additional mandatory capability set is defined for each device type;

      - - Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

Proposal 3: The Table 2 could be considered as the starting point to achieve scalable and forward-compatible design.

Table 2: 6GR UE/Device type example

| 6G communication usage scenarios | Example device type | Scalable and forward-compatible design to avoid market fragmentation |  |  |  |  | Example devices/use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A common basic mandatory function set for all device types | A different additional mandatory capability set for each device type | Optional capability/feature sets for different new usage scenarios or new services. |  |  |  |
| eMBB | TypeA | - Waveform - Frame structure - Channel coding - Initial access design - DL and UL control channel design - Scheduling/HARQ operation - BS/UE energy saving features etc. | - 200M - 2T/4R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set A1 - #MIMO layers - Processing time - Spectrum aggregation capabilities Power clas | Set A2 … 4T/8R | … | Smart phone, CPE/FW, VUE, VSAT, Robots… |
|  | Type B |  | - 100M/200M - 1T2R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set B1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Power class Access control, etc. | Set B2 … | … | 2RX XR, High-end watch |
| Massive IoT | Type C |  | - 20M - 1T1R - Modulation:64 QAM DL/UL Power class: 3 or 5 | Optional set C1 - Processing time - Power class - Access control Scaling factor, etc. | Set C2 … | … | Wearables, Industrial sensors, Low-end watch, Video surveillance … |

- Reference

- R4-2514650, WF on 6GR system parameter, Huawei, HiSilicon. TSG-RAN4#116bis meeting