3GPP TSG-RAN WG4 Meeting #117	R4-2520769

Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on co-existence for HPUE

Agenda Item:	7.1.3

Document for:	Discussion

- Introduction

In RAN#109 plenary meeting, a new WID on UE RF requirement enhancements for the deployment in NR FR1 was approved [1], and it is noted New co-existence study to evaluate the ACLR requirement would be necessary since the legacy ACLR requirement for PC1 may not be suitable for typical UE implementations under 4x26 dBm PA configuration assumption.

RAN4#116bis meeting kicked off the first meeting for the discussion of the co-existence issue, and some simulation assumptions are agreed for urban macro and rural scenario[2]. In this contribution, we continue to provide some views from our side and show some primary simulation results based on the agreements.

- Discussion

2.1 Simulation assumption

In last meeting, both urban and rural scenarios were discussed for further co-existence evaluation as below:

Table 1 lists the co-existence study scenario for FWA HPUE with 4Tx. Table 1: Co-existence study scenario Usage scenario; Aggressor; Victim; Direction; Simulation frequency; Deployment Scenario; Note. eMBB; NR, 100MHz; NR, 100MHz; UL to UL; 2.6GHz; [Urban macro, Rural macro]. 3.5GHz, 4.9GHz, 7GHz; Urban macro.

In our opinion, HPUE is mainly used in a scenario with broad coverage. Theoretically, to perform a comprehensive study on the co-existence of HPUE with up to 32dBm maximum output power, rural macro scenario should also be considered.

Besides, in TR 37.829, we can see that rural macro scenario is included in both band n41 and band n77, thus it is proposed to consider rural scenario for both 2.6GHz and 3.5GHz.

- Proposal 1: For co-existence study scenario, consider an updated table1 for RMA as:

Table 1: Co-existence study scenario

| Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 3.5GHz | Urban macro, Rural macro |  |
|  |  |  |  | 4.9GHz, 7GHz | Urban macro |  |

Network layout model for urban has been agreed in last meeting, but the parameters for rural scenario are missing. It is suggested to complete the simulation assumptions for rural macro as well.

Proposal 2: Complete the simulation assumptions for rural macro scenarios as well.

2.2 Primary co-existence simulation result for UMa

In this section we provides some primary simulation results based on the agreements of urban scenario, it  is noted that max output power is set as 32 dBm for HPUE, and 23dBm for normal UE :

- 2.6G urban ISD = 750m, BS height = 45m, user number = 1

| ACIR | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss | 26.7509 | 22.5582 | 18.8397 | 15.5969 | 12.7966 | 10.4182 | 8.4144 | 6.7457 | 5.3685 | 4.2414 | 3.3233 |  |  |  |  |
| 5%-ile throughput loss | 100.0000 | 84.8959 | 80.0090 | 72.8715 | 64.5455 | 56.4222 | 48.1892 | 40.5898 | 31.4564 | 24.6453 | 19.8971 | 13.1729 | 9.1994 | 7.9762 | 4.8393 |

- 3.5G urban ISD = 500m, BS height = 45m, user number = 1

| ACIR | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss | 37.1856 | 31.9609 | 27.1525 | 22.8075 | 18.9476 | 15.5807 | 12.6847 | 10.2291 | 8.1735 | 6.4708 | 5.0774 | 3.9491 |  |  |  |  |  |
| 5%-ile throughput loss | 100 | 100 | 88.6335 | 83.3706 | 77.0641 | 69.2862 | 61.1094 | 53.2106 | 43.6898 | 34.1338 | 28.0701 | 21.1634 | 16.8694 | 13.4566 | 9.9386 | 6.0721 | 4.5130 |

- 4.9G urban ISD = 600m, BS height = 25m, user number = 1

| ACIR | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss | 28.5031 | 24.1795 | 20.3184 | 16.9280 | 13.9854 | 11.4653 | 9.3268 | 7.5313 | 6.0402 | 4.8093 |  |  |  |  |  |  |  |  |
| 5%-ile throughput loss | 100 | 100 | 100 | 100 | 67.9752 | 62.4557 | 53.4172 | 45.1920 | 35.9560 | 26.4284 | 21.8573 | 17.3625 | 15.1928 | 10.8529 | 7.9689 | 7.6728 | 5.0276 | 4.8803 |

- 7G urban ISD = 600m, BS height = 25m, user number = 1

| ACIR | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss | 16.1408 | 13.3893 | 11.0327 | 9.0311 | 7.3504 | 5.9524 | 4.7932 |  |  |  |  |  |  |  |  |
| 5%-ile throughput loss | 80.8627 | 74.9589 | 69.1259 | 61.4553 | 53.1560 | 46.2453 | 36.3257 | 29.2211 | 23.7253 | 21.6374 | 17.4015 | 13.4177 | 12.6802 | 10.2601 | 9.6995 |

- Taking 2.6GHz for example, we can derive the 5% loss point of 5%-ile throughput loss is: ACIR = 47.6616dB

- As , considering the ACS of gNB is 45dB, then we can get ACLR = 43.1197 dB.

- Observation 1: When max output power is 32 dBm for aggressor UE, ACLR need about 43 dB to meet the performance metric.

- Conclusion

In this contribution, we have the following proposals on the HPUE co-existence:

- Proposal 1: For co-existence study scenario, consider an updated table1 for RMA as:

Table 1: Co-existence study scenario

| Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 3.5GHz | Urban macro, Rural macro |  |
|  |  |  |  | 4.9GHz, 7GHz | Urban macro |  |

- Proposal 2: Complete the simulation assumptions for rural macro scenarios as well.

- Observation 1: When max output power is 32 dBm for aggressor UE, ACLR need about 43 dB to meet the performance metric.

- Reference

- RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109.

- R4-2515085 Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence.