# 3GPP TSG-RAN WG4 Meeting #117	R4-2520801

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.10.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RRM performance requirements of fast SCell activation for UE supporting Rel-18 EMR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 2-1-2: Applicability rules and number of test cases Agreements: - Further discussion applicability rules, taking the table containing TCs for EMR based SCell activation as reference. TC No. (CR No.); Scenario. 1; Normal SCell activation + FR1 + Configuration 1. Normal SCell activation + FR1 + Configuration 2. Normal SCell activation + FR1 + Configuration 3. 2; Normal SCell activation + FR2 + Configuration 1. Normal SCell activation + FR2 + Configuration 2. Normal SCell activation + FR2 + Configuration 3. 3; Direct SCell activation at SCell addition + FR1 + Configuration 1. Direct SCell activation at SCell addition + FR1 + Configuration 2. Direct SCell activation at SCell addition + FR1 + Configuration 3. 4; Direct SCell activation at SCell addition + FR2 + Configuration 1. Direct SCell activation at SCell addition + FR2 + Configuration 2. Direct SCell activation at SCell addition + FR2 + Configuration 3. 5; PUCCH SCell activation + FR1 + Configuration 1. PUCCH SCell activation + FR1 + Configuration 2. PUCCH SCell activation + FR1 + Configuration 3. 6; PUCCH SCell activation + FR2 + Configuration 1. PUCCH SCell activation + FR2 + Configuration 2. PUCCH SCell activation + FR2 + Configuration 3. Way forward: - Discuss the following applicability rules as a starting point: - Agree the following testing applicability rules: - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped. - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped. - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped. - Other applicability rules are not precluded. - Note: Any agreed testing applicability rules will be captured in clause A.3.X in 38.133.

In our views, if UE support multiple configurations or scenarios, in order to limit the numbers of test cases that UE is required to pass, UE can be allowed to pass one of the test cases. Therefore, it’s proposed to define applicability rules for the designed test cases and agree the following testing applicability rules.

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

Proposal 1: Agree the following testing applicability rules.

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

- Conclusion

The contribution presents our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: Agree the following testing applicability rules.

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

- References

- R4-2514801, WF on NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2513511, Topic summary for [116bis][205] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.