# 3GPP TSG-RAN WG4 Meeting #117	R4-2520803

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.15.2.1

Source:	China Telecom

Title:	Discussion on RRM performance requirements for On-demand SSB SCell operation

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RRM performance requirements for On-demand SSB SCell operation were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for On-demand SSB SCell operation captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 1-1-3: General – Different OD-SSB and AO-SSB relation in Case 2 Agreement: RAN4 to define TCs for Alt Time-C1. Meanwhile, - Option 1: Not define TC for Alt Time-C2 - Option 2: Select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped. Further discuss in one test case or separate test cases.

It was agreed to define TCs for Alt Time-C1, meanwhile, for Alt Time-C2, considering the workload and not typical use case, we prefer not to define TC for Alt Time-C2. However, if there is no consensus for Alt Time-C2 and in order to make progress, we can also compromise to select one test for Alt Time-C2.

Proposal 1: Not define TC for Alt Time-C2.

Status in the WF [1]:

Issue 1-2-1: Test case list FFS the following test cases. - SCell activation No.; Test category; Test scenario; Test configuration; Test purpose. FFS; SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1. - Direct SCell Activation No.; Test category; Test scenario; Test configuration; Test purpose. FFS; Direct SCell activation at SCell addition based on OD-SSB; Direct SCell activation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2a FR1. - Deactivated SCell L3 measurement No.; Test category; Test scenario; Test configuration; Test purpose. FFS; Intra-frequency measurement(deactivated SCell) based on OD-SSB; Deactivated SCell measurement; Interruption during deactivated SCell measurement; OD-SSB periodicity 20ms AO-SSB periodicity 20ms; OD-SSB Case 2 FMW FFS FR1.

For SCell activation, the TC4 for SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB Case 2 Alt Time-C1 and the TC1 for SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB Case 1 were agreed to be defined in last meeting, in our views, no need to define test case for SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB Case 1.

For Direct SCell Activation, the TC6 for Direct SCell activation at SCell addition based on OD-SSB Case 2 Alt Time-C1 was agreed to be defined in last meeting, in our views, no need to define test case for Direct SCell activation at SCell addition based on OD-SSB Case 1.

For Deactivated SCell L3 measurement, the TC8 for Intra-frequency measurement (deactivated SCell) based on OD-SSB Case 2 in FR2 was agreed to be defined in last meeting, in our views, no need to define test case for Intra-frequency measurement (deactivated SCell) based on OD-SSB Case 2 in FR1.

Proposal 2: No need to define the following test cases.

- SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB Case 1

- Direct SCell activation at SCell addition based on OD-SSB Case 1

- Intra-frequency measurement (deactivated SCell) based on OD-SSB Case 2 in FR1

- Conclusion

The contribution presents our views on RRM performance requirements for On-demand SSB SCell operation, with the following proposals:

Proposal 1: Not define TC for Alt Time-C2.

Proposal 2: No need to define the following test cases.

- SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB Case 1

- Direct SCell activation at SCell addition based on OD-SSB Case 1

- Intra-frequency measurement (deactivated SCell) based on OD-SSB Case 2 in FR1

- References

- R4-2514808, WF on Netw_Energy_NR_enh_RRM_Part1, Ericsson, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2513514, Topic summary for [116bis][208] Netw_Energy_NR_enh_RRM_Part1, Ericsson, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.