# 3GPP TSG-RAN WG4 Meeting #117	R4-2520806

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.6

Source:	China Telecom

Title:	Discussion on RAN4 6G RRM requirements

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RAN4 6G RRM requirements were discussed and the WF [1] was approved.

In this paper, we present our views on RAN4 6G RRM requirements captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 8: Spectrum aggregation and CA related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify which of the following topics can be starts directly in RAN4 RRM with less RAN1/2 and RAN4 RF session dependency: - SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations (MTK(SCell activation), QC, vivo(activation), Ericsson, Nokia) - RRM conditions and requirements for Single Cell Multi-Carriers (MTK, vivo, Samsung, OPPO) - RRM impacts of DL and UL decoupling (Samsung, Ericsson) - Carrier switch enhancements for UL and DL (MTK) - RRM impacts of realistic SCS for spectrum (Samsung) - relaxation of the requirement on timing alignment between carriers (Ericsson) - RRM for MRSS (Samsung) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics if there are sufficient conclusions from other WGs and RAN4 RF session - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics once there are sufficient conclusions from other WGs and RAN4 RF session

For Spectrum aggregation and CA related RRM, the requirement on SCell activation/deactivation is one of the important parts for 5G specification, a lot of scenarios, conditions, enhancements for the delay on SCell activation/deactivation are defined in 5G specification. In our views, since the 6G UEs are the new UEs with  high-performance, the enhanced requirements on SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations shall be supported in 6G Day1.

In 6GR, one of the trends for Spectrum aggregation is that multiple carriers with small frequency separation are aggregated into a single cell, although the Spectrum aggregation and CA are also discussed in RAN1/2 and RAN4 RF session, Single Cell Multi-Carriers is an important framework for 6G Spectrum aggregation and CA, we think the RRM conditions and requirements for Single Cell Multi-Carriers shall be discussed simultaneously in RAN4 RRM session.

Considering the multiple carriers with small frequency separation are aggregated into a single cell, the DL and UL for each carrier can be decoupled to achieve higher performance, RRM impacts of DL and UL decoupling should be discussed in RAN4 RRM.

Proposal 1: For Spectrum aggregation and CA related RRM, the following topics can be started directly in RAN4 RRM.

- SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations

- RRM conditions and requirements for Single Cell Multi-Carriers

- RRM impacts of DL and UL decoupling

Status in the WF [1]:

Issue 7: RRM related energy efficiency [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify candidate topics can be studied in RAN4 directly: - Network energy saving: - RRM for new SSB design(e.g., SSB periodicity extension, OD-SSB/OD-SIB1) (vivo, Ericsson, Nokia, ZTE) - SSB-less based RRM (Ericsson, Nokia, ZTE) - UE power saving: - UE type/state based RRM relaxation (Apple, CMCC, Ericsson(scalable set of measurement requirement), Nokia) - LR based solutions for UE power saving (Sony, vivo, Ericsson) - DRX/eDRX based measurement (Ericsson) - Energy sensing based RRM strategy (CATT) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other energy efficiency related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other energy efficiency related RRM topics once there are sufficient conclusions from other WGs.

For RRM related energy efficiency, the energy efficiency and energy saving including network and UE are crucial for 6GR study, which affects network deployment and commercial use. The energy efficiency and energy saving for network and UE need to be considered in a coordinated manner, in order to keep the balance on energy saving gains for NW/UE and impacts on UE/NW.

For Network energy saving, many features are introduced in 5G NR, e.g., SSB-less NES cell, OD-SSB SCell, OD-SIB1, SSB adaptation, etc., in our views, the solutions on network energy saving shall be supported in 6G Day1. Although the new SSB design is discussed in RAN1, we think the RRM related solutions with new SSB design shall be discussed simultaneously in RAN4 RRM session, e.g., RRM related SSB periodicity extension, OD-SSB, OD-SIB1, SSB-less, etc.

For UE power saving, the solutions on RRM relaxation, LP-WUR, DRX/eDRX, etc., are defined in 5G NR, we can further discuss whether these solutions can be applied in 6G Day1.

Proposal 2: For RRM related energy efficiency, the following topics can be started directly in RAN4 RRM.

- Network energy saving

  - RRM for new SSB design (e.g., SSB periodicity extension, OD-SSB/OD-SIB1)

  - SSB-less based RRM

- UE power saving

  - UE type/state based RRM relaxation

  - LR based solutions for UE power saving

  - DRX/eDRX based measurement

Status in the WF [1]:

Issue 6: Mobility related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

For Mobility related RRM, Mobility is crucial in wireless network. In RRC_CONNECTED state mobility, several handover requirements are defined in 5G NR, and features on latency and/or interruption reduction for mobility are also specified in 5G NR, e.g., DAPS Handover, CHO, LTM, Conditional LTM, etc.

In our views, considering the new 6G UE implementations with high-performance, latency and/or interruption reduction for mobility shall be studied and defined in 6G Day1. For example, latency reduction on RAN4-defined components with beam sweeping reduction, unified solution with L1/L3 measurement, scenarios/conditions for latency and/or interruption reduction, etc.

Besides, the solution on LTM can be considered as the reference, e.g., pre-sync can be operated for candidate target cells while maintaining connection with the source cell. Therefore, early RRC decoding, DL/UL sync, early T/F tracking for mobility can be considered in 6GR. In addition, if needed, it is better to unify the definition of intra frequency/inter frequency and known/unknown for different cases.

Proposal 3: For Mobility related RRM, the following topics can be started directly in RAN4 RRM.

- Latency and/or interruption reduction for mobility through RAN4-defined components

  - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc.

  - discuss the scenarios/conditions for such reduction (known, unknown, or other status)

- Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility

Status in the WF [1]:

Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics. [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Measurement capability - measurement capability for number of cells, beams and frequency layers (MTK, OPPO, Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - RF retuning time, baseband processing time for typical use cases - UE reference architecture for 6G spectrums - Baseline assumptions of RRM requirements for different UE device types - Measurement delay/overhead - Searcher number for enhanced simultaneous measurements (OPPO, HW, Samsung, ZTE, vivo, Ericsson) - Rx beam sweeping factor reduction (QC, Ericsson) - Identification/measurement/tracking/reporting delay reduction (QC) - RRM with NW aided measurement priority (Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - Unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes. (Nokia) - SSB evaluation (Samsung) - Intra and inter-frequency definition - Unified measurements - United/integrated cross-layers measurement and/or report between L1 and L3 (Samsung, OPPO, CMCC, LGE, Xiaomi) - United/integrated cross-functions measurement and/or report for L1 (e.g., integration of MIMO and LTM, or integration of RLM/BFD/CBD)(OPPO, CMCC, Xiaomi) - RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement. Unification in terms of reusing L3 measurements for L1 before certain point (e.g., before TCI state activation) (Ericsson) - FFS: - For other WG driven topics which have RRM impacts: - Option 1: RAN4 to set check points to check whether or not starting discussion on other RRM framework related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - If the topic is driven by RAN4, RAN4 can take into account the agreements from other WGs as early as possible. - Option 2: RAN4 to check whether or not starting discussion on other RRM framework related RRM topics once there are sufficient conclusions from other WGs.

It was agreed in last meeting that RRM framework: measurement capability/ delay/ overhead/ accuracy/ quantities/ unified measurement, is agreed as part of RAN4 RRM 6G study, and the detailed scope will be further decided. In our views, RRM framework including measurement capability, measurement delay/overhead, unified measurements can be firstly discussed.

For measurement capability, the number of cells, beams and frequency layers need to be further investigated with new UE implementations in 6GR.

For measurement delay/overhead, with new UE implementations, the searcher number for enhanced simultaneous measurements and Rx beam sweeping factor reduction shall be considered in 6GR, which is beneficial for RRM measurement delay/overhead.

For unified measurements, RAN4 can investigate whether L1 and L3 measurement and reporting can be defined as the unified framework and how to define the unified measurement requirements.

Proposal 4: For RRM framework, the following topics can be started directly in RAN4 RRM.

- Measurement capability

  - measurement capability for number of cells, beams and frequency layers

- Measurement delay/overhead

  - Searcher number for enhanced simultaneous measurements

  - Rx beam sweeping factor reduction

- Unified measurements

  - United/integrated cross-layers measurement and/or report between L1 and L3

- Conclusion

The contribution presents our views on RAN4 6G RRM requirements, with the following proposals:

Proposal 1: For Spectrum aggregation and CA related RRM, the following topics can be started directly in RAN4 RRM.

- SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations

- RRM conditions and requirements for Single Cell Multi-Carriers

- RRM impacts of DL and UL decoupling

Proposal 2: For RRM related energy efficiency, the following topics can be started directly in RAN4 RRM.

- Network energy saving

  - RRM for new SSB design (e.g., SSB periodicity extension, OD-SSB/OD-SIB1)

  - SSB-less based RRM

- UE power saving

  - UE type/state based RRM relaxation

  - LR based solutions for UE power saving

  - DRX/eDRX based measurement

Proposal 3: For Mobility related RRM, the following topics can be started directly in RAN4 RRM.

- Latency and/or interruption reduction for mobility through RAN4-defined components

  - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc.

  - discuss the scenarios/conditions for such reduction (known, unknown, or other status)

- Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility

Proposal 4: For RRM framework, the following topics can be started directly in RAN4 RRM.

- Measurement capability

  - measurement capability for number of cells, beams and frequency layers

- Measurement delay/overhead

  - Searcher number for enhanced simultaneous measurements

  - Rx beam sweeping factor reduction

- Unified measurements

  - United/integrated cross-layers measurement and/or report between L1 and L3

- References

- R4-2514644, WF on 6G RRM, Apple, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2514512, FL summary for [116bis][105]6G RRM, Apple, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.