3GPP TSG-RAN WG4 Meeting #117	R4-2520809

Dallas, US, November 17th – 21st, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance requirements for CSSF optimization

Agenda item:	6.10.2.2

Document for:	Discussion

Introduction

In this contribution we are discussing the leftover issues on RRM performance requirements for FR2-1 SSB based L3 measurement delay reduction, considering CSSF optimization.

Discussion

In last meeting, it was not concluded if we are defining the test case when one inter-RAT E-UTRAN Cell is configured.

Issue 2-1-3: Cell configurations in TCs for CSSF solution 3

<Way Forward>:

- Continue discussions on the test case when one inter-RAT E-UTRAN Cell configured and consider the impact to core requirements.

Herein we would like to recite the agreement on E-UTRAN inter-RAT measurements from RAN4#116 meeting as below. We understood legacy searcher is assumed for the inter-RAT MOs without MGs, but this does not prevent applying 3 searchers to the measurements on intra-frequency carriers.

Observation 1: It was agreed not to apply 3 searchers to EUTRAN inter-RAT MOs without MG, but this does not prevent applying 3 searchers to the measurements on intra-frequency carriers.

Issue 1-2-1: applicability limitation for solution 3

Agreement:

For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG.

When checking the enhanced CSSF table as cited below, taking FR1 only CA as an example, the optimized CSSF is applied to the measurement on FR1 SCC while legacy CSSF is applied to the measurement on E-UTRA inter-RAT MO without MG. Hence optimized measurement delay is still required on the intra-frequency carriers while E-UTRA inter-RAT MO is also configured.

Observation 2: In enhanced CSSF table, the optimized CSSF is applied to the intra-frequency measurements on e.g. FR1 SCC while E-UTRA inter-RAT MO without MG is also configured.

Table 9.1.5.1.2-2: Enhanced CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA Note12 | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | 0.5×(NSCC_SSB +2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) +Y+Z | N/A | N/A | N/A | 0.5×(NSCC_SSB +2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) +Y+Z | (NSCC_SSB +Y+Z+2x NSCC_CSIRS |

Therefore, we can define a test case to verify the measurement delay on the intra-frequency carriers based on enhanced CSSF table in 9.1.5.1.2-2 while inter-RAT MOs without MG is also configured. As the measurement of inter-RAT MOs is not impacted, there is no need to verify the measurement delay on the inter-RAT MOs.

RAN4 to define a test case to verify the optimized measurement delay on the intra-frequency carriers based on enhanced CSSF table in 9.1.5.1.2-2 while inter-RAT MOs without MG is also configured.

Conclusion

The following Observations and Proposals were made:

Observation 1: It was agreed not to apply 3 searchers to EUTRAN inter-RAT MOs without MG, but this does not prevent applying 3 searchers to the measurements on intra-frequency carriers.

Observation 2: In enhanced CSSF table, the optimized CSSF is applied to the intra-frequency measurements on e.g. FR1 SCC while E-UTRA inter-RAT MO without MG is also configured.

- RAN4 to define a test case to verify the optimized measurement delay on the intra-frequency carriers based on enhanced CSSF table in 9.1.5.1.2-2 while inter-RAT MOs without MG is also configured.