3GPP TSG-RAN4 Meeting #117	R4-2520816

# Dallas, USA, November 17th – 21st, 2025

Source:	LG Electronics

Title: 	(6G system parameters) Waveform

Agenda Item:	8.2.1

Document for:	Discussion

Introduction

RAN4 made WF [1] for following 6G system parameters in RAN4#116bis.

- Waveform

  - Framework study for waveform

  - PA model

- Modulation

- Channel bandwidth (CBW)

  - Max CBW

  - Min CBW

  - FFT size

  - Numerology

  - Spectrum utilization

  - Asymmetric CBWs

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Irregular channel bandwidth

- Number of Tx and Rx

- Device types

We would like to provide our views on 6G waveform.

Discussion

Framework study for waveform

RAN4 Agreement

RAN4#116bis - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

RAN1 agreement

RAN1#122 CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink RAN1#122bis - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

According to RAN1 agreement, basis and potential additional waveforms, and net gain are as follows.

- Waveforms

  - Basis

    - UL : CP-OFDM and DFT-s-OFDM

    - DL : CP-OFDM

  - Potential additional waveform

    - DL : DTF-s-OFDM

- Net gain for UL

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

RAN1 net gain for UL can be also considered in RAN4.

And, different Low-PAPR techniques for CP-OFDM and DFT-s-OFDM is under discussion in RAN1.

- Low-PAPR for CP-OFDM

  - Tone Reservation (TR)

  - Selected Mapping (SLM)

- Low-PAPR for DFT-s-OFDM

  - Crest Factor Reduction (CFR) with spectrum extension

  - DFT precoder extension

  - Flexible frequency-domain mapping of the DFT output to the spectrum allocation,

  - Flexible time-domain resource mapping of the DFT

  - Frequency domain spectrum shaping (FDSS)

  - FDSS with spectrum extension

  - GMSK approximation

  - I/Q-offset DFT-s-OFDM

  - Interpolation-based π/2-BPSK and QAM DFT-s-OFDM

  - O-QPSK and O-QAM modulated DFT-s-OFDM

  - Pruning QAM (e.g. 32QAM) modulated DFT-s-OFDM

  - Selected mapping (SLM)

  - Tone Reservation (TR)

  - Subcarrier truncation for π/2-BPSK modulated DFT-s-OFDM

  - Multi-Tx enhancements: different transmit ports transmit over different frequency domain allocations.

Primary purpose of RAN4 study on waveform is to evaluate the followings.

- candidate waveforms

- potential PAPR reduction techniques based on agreements and inputs from RAN1

Here, in our understanding, candidate waveform is DL DFT-s-OFDM.

For evaluation of potential PAPR reduction, RAN4 needs to wait for RAN1 agreement/input on CP-OFDM and DFT-s-OFDM.

Proposal 1: RAN4 needs to evaluate candidate waveform, DL DFT-s-OFDM.

Proposal 2: RAN4 needs to wait RAN1 agreement/input on Low-PAPR techniques for CP-OFDM and DFT-s-OFDM.

Proposal 3: Consider Net gain in RAN1 as evaluation metric for UL.

- Net gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

PA model

RAN4 Agreement

RAN4#116bis - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

RAN4 has used several different models to model the non-linearities and other imperfections of the PA and/or transmit system when making comparisons between options and when defining the RF requirements. In addition to simulations, measurements have also been used when possible, as it is very difficult to create a model that correctly takes into account all the needed aspects, but simulations, when used properly are fast, relatively accurate and straightforward way to get started and enable analysis of wide parameter sets that are close to impossible to cover with measurements.

In this contributions we compare the ACLR, EVM, SEM and IBE performance with different transmit signals of following memory less and RF bandwidth agnostic PA-models.

- AM Clip (AM-AM)

- Tanh (AM-AM)

- Rapp (AM-AM)

- GMP#1 (AM-AM & AM-PM from earlier RAN4 studies)

- GMP#2 (AM-AM & AM-PM) (R4-2514637)

The AM-AM and AM-PM characteristics for these models are shown in Figure 1.

Figure 1. AM-AM and AM-PM characteristics for the models

The test signal waveforms are selected to represent signals that are known to be problematic for one or more of the above listed RF performance parameters, have transmission bandwidth configuration and modulation as in Table 1 and MPR as indicated in Figure 2.

Table 1. Transmission bandwidth configuration and modulation for test waveforms 1 to 5

| Case | Modulation |  | Allocation | Nrb | Lcrb | Start RB | End RB | Ptx | PAPR 99.9% | QM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DFTS | QPSK | Inner | 272 | 136 | 68 | 203 | 26 | 6.8 | 7.3 |
| 2 | DFTS | QPSK | Outer | 272 | 268 | 3 | 270 | 25 | 6.8 | 7.3 |
| 3 | DFTS | QPSK | Edge | 272 | 1 | 1 | 1 | 25 | 3.9 | 6.0 |
| 4 | CP | QPSK | Outer | 272 | 268 | 3 | 270 | 23 | 11.1 | 9.8 |
| 5 | CP | 64QAM | Outer | 272 | 268 | 3 | 270 | 23 | 11.1 | 9.8 |

Test signal has SCS= 30kHz, CBW= 100MHz and maximum SU= 98% with NRB,MAX= 272 (Max 3264 sub-carriers and Max transmission BW of 97.92MHz).

Figure 2. Test signal waveforms and 5G MPRs.

The operating point for each PA model is selected in a way that ACLR= 31dB for Case 2 signal with 1dB MPR (DFTS-OFDM, QPSK, Outer allocation).

Annex 5 lists all the simulated cases that are used in the comparison and also shows the more detailed simulation results for the model number 5 i.e. for GMP#2 with parameters copied from R4-2514637.

Simulations results are shown and compared in Figure 3, Figure 4, Figure 5, Figure 6 and Figure 7:

Observation 1: Different PA-models show very similar performance for ACLR, EVM, SEM and IBE in all analysed cases.

Proposal 4: Different PA-models seem to show very similar performance for ACLR, EVM, SEM and IVE in all analysed cases and this should be taken into consideration when discussing the PA modelling for 6G waveform analysis and for RAN4 6G study.

Figure 3. Case 1 ACLR and EVM

Figure 4. Case 2 ACLR and EVM

Figure 5. Case 3 SEM margin

Figure 6. Case 4 ACLR and EVM

Figure 7. Case 5 ACLR and EVM

Conclusions

This document discusses 6G waveform including PA model, and provides our view.

[Waveform]

Proposal 1: RAN4 needs to evaluate candidate waveform, DL DFT-s-OFDM.

Proposal 2: RAN4 needs to wait RAN1 agreement/input on Low-PAPR techniques for CP-OFDM and DFT-s-OFDM.

Proposal 3: Consider Net gain in RAN1 as evaluation metric for UL.

- Net gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

[PA]

Observation 1: Different PA-models show very similar performance for ACLR, EVM, SEM and IBE in all analysed cases.

Proposal 4: Different PA-models seem to show very similar performance for ACLR, EVM, SEM and IVE in all analysed cases and this should be taken into consideration when discussing the PA modelling for 6G waveform analysis and for RAN4 6G study.

References

[1] R4-2514650, WF on 6G system parameter, TSG-RAN4 Meeting #116bis, October 2025

Annex

| PA | Case | Mod | Mod | Alloc |
| --- | --- | --- | --- | --- |
| Clip | 1 | DFTS | QPSK | Inner |
| Clip | 2 | DFTS | QPSK | Outer |
| Clip | 3 | DFTS | QPSK | Edge |
| Clip | 4 | CP | QPSK | Outer |
| Clip | 5 | CP | 64QAM | Outer |
| Tanh | 1 | DFTS | QPSK | Inner |
| Tanh | 2 | DFTS | QPSK | Outer |
| Tanh | 3 | DFTS | QPSK | Edge |
| Tanh | 4 | CP | QPSK | Outer |
| Tanh | 5 | CP | 64QAM | Outer |
| Rapp | 1 | DFTS | QPSK | Inner |
| Rapp | 2 | DFTS | QPSK | Outer |
| Rapp | 3 | DFTS | QPSK | Edge |
| Rapp | 4 | CP | QPSK | Outer |
| Rapp | 5 | CP | 64QAM | Outer |
| GMP #1 | 1 | DFTS | QPSK | Inner |
| GMP #1 | 2 | DFTS | QPSK | Outer |
| GMP #1 | 3 | DFTS | QPSK | Edge |
| GMP #1 | 4 | CP | QPSK | Outer |
| GMP #1 | 5 | CP | 64QAM | Outer |
| GMP #2 | 1 | DFTS | QPSK | Inner |
| GMP #2 | 2 | DFTS | QPSK | Outer |
| GMP #2 | 3 | DFTS | QPSK | Edge |
| GMP #2 | 4 | CP | QPSK | Outer |
| GMP #2 | 5 | CP | 64QAM | Outer |

Figure 8. PA model#5, Case 1

Figure 9. PA model#5, Case 2

Figure 10. PA model#5, Case 3

Figure 11. PA model#5, Case 4

Figure 12. PA model#5, Case 5