3GPP TSG-RAN WG4 Meeting #117							   R4-2520824

Dallas, USA, 17th – 21st November 2025

Agenda Item: 	7.1.3

Source: 	MediaTek Inc.

Title: 	Initial coexistence evaluation results for TN FWA 4x26dBm device in Rel-20

Document for: 	Discussion & Decision

# Introduction

In the last RAN WG4 # 116BIS Meeting, RAN4 approved the WF [2] on TN FWA 4x26dBm coexistence assumptions to determine FWA ACLR/ACS requirements. In this paper we propose our initial coexistence system level simulation results based on the agreed the detail coexistence deployments scenarios and RF parameters for TN FWA high power UEs.

The following topics are Rel-20 TN UE RF enh. Phase 5 main discussion topics.

- 4x26dBm (up to 32dBm) high Power UE for FWA only

- 2x26dBm PC1.5 UE in FDD band for both HHUE and FWA

- Feasibility study from 4Q 2026 as following two sub-topics (start 4Q 2026)

  - PC1.5 1Tx (1x29dBm) for TDD bands for FWA UE

  - PC1 2Tx (2x29dBm) for TDD bands for FWA UE

# Coexistence evaluation for 4x26dBm FWA UE

## Coexistence study for High power 4x26dBm (up to 32dBm) FWA in TN

In last RAN4 Meeting, RAN4 agreed the coexistence scenarios and detail system assumptions and RF parameters for TN FWA high power UEs [2].

Table 1: Co-existence study scenario

| Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- |
| eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz | [Urban macro, Rural macro] |
|  |  |  |  | 3.5GHz, 4.9GHz, 7GHz | Urban macro |

In the previous agreements in [2], the UE antenna characteristic is just assumed the iso isotropic with 0dBi (TR 38.921 [3]). But, in TS38.901[5], the latest UE antenna characteristics are updated to consider directional antenna pattern as follow

For consumer premise equipment (CPE) devices, the following properties apply:

-	Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. Note that a candidate antenna location (e.g., 9) can be used for multiple antennas.

-	Device dimensions for UT antenna modelling are (0 cm, 20 cm, 20 cm)

Table 7.3-2: Radiation power pattern of a single antenna element for handheld UT

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with θ3dB=125°,SLAV=22.5dB and θ″∈0°, 180° |
| Horizontal cut of the radiation power pattern (dB) | AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with ϕ3dB=125°, Amax=22.5 dB and ϕ″∈-180°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax |
| Maximum directional gain of an antenna element GE,max | 5.3 dBi |
| NOTE: For UT antenna modelling of handheld devices, optional antenna imbalance can be modelled. If modelled, randomized loss is applied per UT antenna port and randomized loss can be applied independently for the UL and DL directions. No imbalance is modelled by default. |  |

So, we recommend updating FWA antenna characteristics based on the TR38.901[3] with directional antenna system.

Proposal 1: RAN4 can consider the realistic FWA antenna system with directional antenna pattern based on TR38.901 for the SLS coexistence study for 4x26dBm FWA UE.

## TN FWA 4x26dBm UE coexistence evaluation results

In Table 2, we shared the network layout model for urban macro based on approved WF [2].

Table 2: Network layout model for urban macro

| Parameters |  | Value |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and [TR 38.921, 600m] for other frequencies) ISD = 750m for 2.6GHz (UMa) |
| Multi operators layout |  | Uncoordinated |
| BS antenna height |  | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and TR 38.922 for other frequencies) |
| UE location | Outdoor/indoor | Outdoor and indoor |
|  | Indoor UE ratio | 20% (TR 38.921) |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 (TR 38.921) |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |
| Channel model |  | UMa |

To derive adjacent channel coexistence evaluation, RAN4 considered just 1 active UE with whole CBW within the NR general deployment parameters in among Table 1,2 for FWA 4x26dBm UE.

Table 3: Other detail simulation assumptions for SLS coexistence evaluation

| Parameters | Value |
| --- | --- |
| Frequency reuse | 1x3x1 |
| The number of active UE (UL) | 1 or 3 |
| Scheduled channel bandwidth per UE (UL) | 98.28 MHz (1 user) 32.76 MHz (3 user) |
| Link Level Performance Model | TR 38.921 |
| Received power model | TR 38.921 |
| Traffic model | Full buffer |
| UE max TX power in dBm | 23 (as reference), 32 |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 6 (@7 GHz) 5 (@below 6 GHz) |
| UE Noise figure in dB | 9 or 12 (Note 1) |
| ACLR of UE | 30 dB +X |
| ACS of BS | 45 dB (@below 6 GHz) 42 dB (@7 GHz) |
| Handover margin | 3 dB |
| Note 1: NF of 12 dB is for band n104. For bands below 6 GHz, NF is 9dB. |  |

Table 4. Required ACIR levels (dBc) for 32dBm FWA at 5% UL throughput loss

| ACLR requirements (5%-tile edge UE) | Indoor scenarios | Outdoor Scenarios |
| --- | --- | --- |
| PC3 FWA | 17.42 | 17.02 |
| 4x26dBm FWA | 15.25 | 15.08 |

From the initial evaluation results, we can observe:

- 4x26dBm FWA UE’s ACLR is higher for indoor scenarios compared to outdoor scenarios. In here, we excluded the outage (Bad) UEs which even with no T-put since very lower SINR (< -10dB)

- Based on the assumption of outage UEs, the worst case ACLR is less than 18 dB for PC3 UE.

The original ACLR level is 30dBc for PC3 UE, but this simulation assumed the enhanced AAS in gNB side, the enh. schemes will be reduced the ACI levels in SLS. So small ACLR was observed for PC3 UE.

- All UL RSRP, ICI and ACI are higher 4x26dBm FWA UE compared to PC3 UE.

Less UL SINR degradation with high power UE (32 dBm) than PC3.

Observation 1: At around 2.6GHz, the required ACLR level is 15.5 dBc for 4x26dBm FWA.

RAN4 encourage to merge the coexistence simulation results to finalize UE ACLR and ACS level for 4x26dBm FWA UE. So, based on the coexistence simulation results from interested companies, RAN4 will further discuss the detail ACLR and ACS requirements for 4x26dBm FWA UE.

Proposal 2: RAN4 need to merge the coexistence evaluation results from interested companies and RAN4 will further discuss the detail ACLR and ACS requirements based on the coexistence evaluation results.

- PC3 UE’s Tx power CDF curve            (b) 4x26dBm FWA UE’s Tx power CDF curve

Figure 1. Tx power CDF curves between PC3 UE and 4x26dBm FWA UE

- PC3 UE’s T-put loss as reference            (b) 4x26dBm FWA UE’s T-put loss

Figure 2. Uplink Throughput loss comparison between PC3 UE and 4x26dBm FWA UE

# Summary

In this paper, we provide our views on the coexistence evaluation initial results and related coexistence detail scenarios and parameters for FWA 4x26dBm (up to 32dBm) device. Based on the above analysis, we propose as following proposals in paper.

Proposal 1: RAN4 can consider the realistic FWA antenna system with directional antenna pattern based on TR38.901 for the SLS coexistence study for 4x26dBm FWA UE.

Observation 1: At around 2.6GHz, the required ACLR level is 15.5 dBc for 4x26dBm FWA.

Proposal 2: RAN4 need to merge the coexistence evaluation results from interested companies and RAN4 will further discuss the detail ACLR and ACS requirements based on the coexistence evaluation results.

# References

- [1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5,” Moderator (vivo)

- [2] R4-2515085, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence” Huawei

- [3] TR38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”

- [4] TR38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”

- [5] TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz