3GPP TSG-RAN WG4 Meeting #117							   R4-2520860

Dallas, USA, 17th – 21st November 2025

Agenda Item: 	10.2.2

Source: 	MediaTek Inc.

Title: 	MPR evaluation results for n104 relaxed ACLR level

Document for: 	Approval

# Introduction

In the previous RAN4 #116 Meeting, RAN4 approved the WF on MPR reduction for n104 with relaxed ACLR level with 26dBc which will be treated as Rel-17 maintenance [1] as following work scope. In this paper we share our initial measurement results to derive MPR requirements for n104 with relaxed ACLR level with 26dBc.

<Topic 1: Scope>

<Way forward/Agreement>

-	RAN4 introduces a band specific ACLR of 26 dB to n104 with a dedicated MPR under the following conditions.

- PC3 for n104 and CA_n104C and Band Combinations (BCs) including n104

- 1Tx and 2Tx are considered

<Topic 2: technical aspects>

## Sub-topic 2-1: How to introduce the ACLR

<Way forward/Agreement>

- Below approaches can be further discussed

- FFS on if a band specific ACLR for n104 and CA_n104C is treated with NS signaling like other bands or not.

- Other approaches are not precluded

## Sub-topic 2-2: MSD

<Way forward/Agreement>

- FFS on MSD for a new MSD value for CA_n78-n104 with respect to cross band isolation

<Topic 3: Procedure>

<Way forward/Agreement>

-	This topic is treated under Release 17 maintenance as correction

# MPR evaluation for relaxed ACLR with 26dBc for n104

## MPR evaluation assumption and detail parameters

To derive MPR level for n104 UE with relaxed ACLR 26dBc, we measured the MPR levels based on this PA assumption and detail parameters as follow.

- PA assumption

  - Operates from 6GHz to 10GHz

  - 25dBm typical transmit state PSAT

  - 22dB typical transmit state small signal gain

  - 18dB typical receive state small gain

  - 2.5dB typical receive state noise figure

  - Coupled PA output for power detection

MPR simulation assumptions

- Transmit IQ image :- 28dBc

- Transmit carrier leakage: -28dBc

- Transmit CIM3 : -60dBc

- Transmit CIM5 : -70dBc

- PA calibration points: 30dB ACLR at PC3 with 1dB MPR on 20MHz CBW, 15kHz SCS and DFT-s-OFDM 100RB0 waveform

We refer the current MPR requirements with ACLR 30dBc in TS38.101-1 as follow

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

## Required MPR requirements for n104 UE with ACLR 26dBc

Based on our measurement results, we propose the required MPR Table for n104 UE with relaxed ACLR 26dBc as follow

Table 1. Simulation assumptions of TN NR deployment parameters

| Modulation |  | MPR (dB), Edge and Outer RB |  |  |
| --- | --- | --- | --- | --- |
|  |  | ACLR = -26dBc | 3GPP Spec | Limiting Factor |
| DFT-s-OFDM | QPSK | 0.0 | <=1 | ACLR -26dBc |
|  | 16QAM | 0.5 | <=2 | ACLR -26dBc |
|  | 64QAM | 1.0 | <=2.5 | ACLR -26dBc |
|  | 256QAM | 4.5 | <=4.5 | EVM 3.5% |
| CP-OFDM | QPSK | 2.0 | <=3 | ACLR -26dBc |
|  | 16QAM | 2.0 | <=3 | ACLR -26dBc |
|  | 64QAM | 3.5 | <=3.5 | EVM 8% |
|  | 256QAM | 6.5 | <=6.5 | EVM 3.5% |

From the measurement results in Table 1, we can propose the required MPR for n104 UE in Table 2.

Table 2 Maximum power reduction (MPR) for power class 3 for n104 with ACLR 26dBc

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.0 |  |  |
|  | QPSK | ≤ 0.0 |  |  |
|  | 16 QAM | ≤ 0.5 |  | ≤ 0.0 |
|  | 64 QAM | ≤ 1.0 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 2.0 |  | ≤ 0.5 |
|  | 16 QAM | ≤ 2.0 |  | ≤ 1.0 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Proposal 1: For n104 PC3 UE with relaxed ACLR 26dBc, the required MPR levels in Table 2 can be considered for revised MPR for PC3 n104 UE in TS38.101-1.

# Summary

In this paper, we provide our views on the coexistence evaluation necessity and related coexistence detail scenarios and parameters for FWA 4x26dBm (up to 32dBm) device. Based on the above analysis, we propose as following proposals in paper.

Proposal 1: For n104 PC3 UE with relaxed ACLR 26dBc, The required MPR  levels in Table 2 can be considered for revised MPR for PC3 n104 UE in TS38.101-1.

# References

- [1] R4-2511799, “Way forward for MPR reduction for n104,” Huawei, OPPO, Samsung, CATT

- [2] TR38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”

- [3] TR38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”

- [4] TR38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects