3GPP TSG-RAN WG4 Meeting #117            								                      R4-2520882

Dallas, USA, Nov. 17-21, 2025

Source: 	Xiaomi

Title: 	The remaining issues for R20 AIoT co-existence simulation assumption

Agenda item:	7.11.3

Document for:	Discussion

- Introduction

This contribution provides our analysis for the remaining co-existence simulation assumptions. Based on our preliminary simulation results, some modification proposals are provided.

- Discussion

  - Common assumptions for standalone and In-band mode

The following cases are proposed to be calibrated between companies.

Table 1: Evaluation cases

| Operation mode | Device type | Evaluation case number | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | Reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | Device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | Reader | D2R |
|  |  | 7 | Reader | NR DL | R2D |
|  |  | 8 | NR DL | Device | R2D |

Case 7 is identical with case 3 because the AIoT reader and NR UE assumptions are the same for the two cases, and this link has no relationship with AIoT device. So case 7 can be removed.

Proposal 1: Case 7 is removed because it’s identical with case 3.

When the calibration simulation is conducted, we found some simulation assumption may need to be revised.

AIoT device power control

For the AIoT device, no power control is used in the calibration. In our preliminary simulation results shown in the following table, severe AIoT D2R to NR UL is observed.

Table 2: NR UL thput loss results when no AIoT device power control

| Case |  | Baseline thput (bit/Hz) | Thput with ACI (bit/Hz) | Thput loss(%) |
| --- | --- | --- | --- | --- |
| Case 1 Device 2b -> NR UL | 5% tile | 0.2781 | 0.2687 | 3.3779 |
|  | 50% tile | 0.9192 | 0.8856 | 3.6579 |
| Case 5 Device C -> NR UL | 5% tile | 0.2804 | 0.2243 | 20.008 |
|  | 50% tile | 0.9178 | 0.8171 | 10.9737 |

When the device is near NR BS, if there’s no power control, the adjacent channel noise is very large that interferes the small received signal in NR Rx side. On the other hand, from the energy saving aspect, power control for AIoT device is also necessary. We propose to use power control in the co-existence simulation.

Proposal 2: Power control is used for AIoT device D2R signal.

For the power control model, NR UE power control model can be a candidate, i.e.

However, if this model is used, the device MOP and minimum output power should be decided, and also CLx-ile should be derived. Usually, the minimum output power is derived from system analysis and considering the device RF capability. MOP can use the same value in the simulation. The derivation of CLx-ile needs SNR target to calculate the maximum CL. A temperate solution can be that R19 SNR assumption in co-existence simulation is used as the starting point to continue the simulation. It can be revisited and aligned offline if some RAN1 references can be found.

Proposal 3: The following power control model for device D2R signal is used for RAN4 co-existence simulation.

Proposal 4: R19 SNR assumption in co-existence simulation is used as the starting point to continue the discussion and simulation. It can be revisited if there’s any RAN1 agreements.

NR UE ACLR model

1 step flat ACLR model is assumed in the calibration that any frequency ranges outside the wanted signal suffer the same PSD level interference with the adjacent channel. This model introduces more interference than the real UE performance because the 2nd adjacent channel emission can be lower than the 1st adjacent channel. According to our initial simulation results, using the 1 step flat ACLR model, more than 1 dB SINR degradation is observed at AIoT Rx side. If 2 step ACLR model is used, the SINR degradation is decreased.

Table 3: AIoT SINR degradation results for two different NR UE ACLR model

| Case |  | SINR_baseline(dB) | SINR with ACI (dB) | SINR_degradation(dB) |
| --- | --- | --- | --- | --- |
| Case 2 （NR UL->Reader NR UE 1 step flat ACLR model） | 5% tile | -16.1823 | -16.6635 | 0.48122 |
|  | 50% tile | 6.9015 | 5.5244 | 1.3771 |
| Case 2（NR UL->Reader NR UE 2 step ACLR model） | 5% tile | -15.7437 | -16.0492 | 0.30556 |
|  | 50% tile | 6.8166 | 6.3249 | 0.49172 |

Based on the above assumption, we propose to use 2 step ACLR model for the NR UE. The following model is copied from TR 36.942. It can be taken as a reference.

Figure 1: 2-step ACLR model, copied from TR 36.942

Proposal 5: 2-step ACLR model is used for NR UE.

Another observation from Table 3 is that the baseline SINR is very low for 5% tile, the reason may be that device MOP is too low to reach the large coverage BS. This may need some discussion that if the ISD assumption needs to be revisited.

Observation 1: ISD assumption may need to be revisited according to the Reader Rx SINR.

Device ACLR model

For device ACLR model, currently in the calibration, the 2nd adjacent channel is assumed no emission at all. This is too optimised that no device can achieve this kind of performance. If more than one device is assumed, the ACLR model outside of the 1st adjacent channel impacts the whole emission level. So we propose to use 2-step ACLR model like NR UE.

Proposal 6: 2-step model is used for the device ACLR.

Device ACS model

In the Rx Digital Down Conversion (DDC) function, usually multi-stage antialiasing filters are used. For the stages except last stage, the sampling rate is higher, so the filter can have a smooth transient band, and the rejection on aliasing frequency range can be higher enough to ignore the aliasing noise at Rx band. For the last stage, as the sampling rate is not that high, only the guard band can be used as the transient band for the filter, the aliasing rejection ratio is assumed as ACS level. So for the device ACS model, the BW scaling should be used for the emission.

Proposal 7: BW scaling should be used for the emission fall into AIoT device Rx band, i.e. only the interference of AIoT BW needs to be taken into account.

Number of active AIoT device

In the calibration, only one AIoT device is assumed for UL 15kHz. This assumption may lead to too relaxed ACLR requirements because the BW can be 180kHz and 12 devices are scheduled then the emission can be 12 times higher than one device. So we propose to simulate multiple devices in order to derive reasonable ACLR requirements.

Proposal 8: Multiple AIoT devices are assumed in the co-existence simulation to derive reasonable ACLR requirements.

  - Specific assumptions for standalone mode

AIoT reader output power

In the initial simulation results, we also observed severe interference from AIoT reader as shown in Table 3.

Table 3: NR DL thput loss for two AIoT reader power assumptions

| Case |  | Baseline thput (bit/Hz) | Thput with ACI (bit/Hz) | Thput loss(%) |
| --- | --- | --- | --- | --- |
| Case3 （43 dBm Reader -> NR DL） | 5% tile | 0.4373 | 0.3770 | 13.7837 |
|  | 50% tile | 1.8943 | 1.8251 | 3.6524 |
| Case3 （38 dBm Reader -> NR DL） | 5% tile | 0.4522 | 0.4201 | 7.1088 |
|  | 50% tile | 1.9095 | 1.8734 | 1.8931 |

The possible problem may be that the AIoT reader is 17 dB PSD higher than NR BS. But the ACLR/ACS assumption is the same with NR system, there’s no that big margin for the ACIR requirement. Because more stringent ACIR requirement means more implementation effort for BS/UE, so from co-existence simulation point of view, too much margin is not reasonable. Thus if the PSD is much higher than NR BS, severe interference will happen. The AIoT reader power can be reduced in our understanding because R2D link is not the limited link in AIoT system.

Observation 2: AIoT reader power in standalone mode may need to be reduced to mitigate the severe interference to NR DL.

  - Specific assumptions for In-band mode

Co-site AIoT reader in-band power

In the last meeting, there’s no agreement for AIoT in-band power after the on-line/offline discussion. The argument is PSD limited or not limited. As there’s no clear assumption in SID if shared hardware for AIoT and NR BS is used, it is hard to say which position is correct. So we have the same proposal as in last meeting that a relative higher power can be assumed to see if there’s co-existence issue. If severe interference is found, it means the lower power should be set.

Proposal 9: 38 dBm co-site AIoT reader in-band power is assumed as the starting point for the co-existence simulation. If severe interference is found, the lower power level should be used.

Orthogonal or non-orthogonal for AIoT R2D and NR DL

In theory, orthogonality can be assumed for AIoT R2D and NR DL if they’re from the same equipment or the clock can be perfectly aligned. However, there’s some comment in the offline that if the clock is not perfectly aligned, non-orthogonality should be considered. This is a technically correct comment from our understanding, so it’ll be safe to do some analysis on this scenario. As the reader and NR BS are co-site, link level analysis is sufficient because other cells interference is much lower than the interference in same cell coverage.

For the link of reader interfering NR DL, all of the reader transmitted signal and the emission fall into the NR UE Rx band. Reader ACLR is assumed 45 dBc then the emission part interference can be ignored. If 10 dB PSD larger for AIoT reader than NR BS, assuming 45 dBc ACLR, the AIoT interference to the NR wanted signal is 35 dB PSD lower. This interference is too small. The transmitted signal part impacts the NR UE Rx dynamic range but may not need to be cared about much because the time domain power level is maximum 3 dB larger.

For the link of NR DL interfering AIoT device, it’s a little different that ACLR and ACS, i.e. ACIR should considered. Still assuming AIoT R2D signal is 10 dB larger than NR DL signal, if AIoT DL necessary SNR is 10 dB, then the AIoT device ACS can be very relaxed, ~ 10 dB is sufficient.

From the above analysis, it’s not necessary to simulate the links of NR DL <-> AIoT R2D even orthogonality can’t be meet for AIoT reader and NR BS.

Observation 3: Link level simulation can be conducted to check the interference between NR DL and AIoT R2D.

Observation 4: For the link of reader interfering NR DL, the interference is negligible assuming AIoT signal PSD is 10 dB higher than NR signal.

Observation 5: For the link of NR DL interfering reader, AIoT device ACS can be very relaxed, ~ 10 dB, assuming AIoT signal PSD is 10 dB higher than NR signal.

Proposal 10: No co-existence simulation is needed for the interference between NR DL and AIoT R2D.

- Conclusion

This contribution provides our analysis for the remaining co-existence simulation assumptions. We have the following observations and proposals for the simulation assumptions.

For the common assumption,

Proposal 1: Case 7 is removed because it’s identical with case 3.

Proposal 2: Power control is used for AIoT device D2R signal.

Proposal 3: The following power control model for device D2R signal is used for RAN4 co-existence simulation.

Proposal 4: R19 SNR assumption in co-existence simulation is used as the starting point to continue the discussion and simulation. It can be revisited if there’s any RAN1 agreements.

Proposal 5: 2-step ACLR model is used for NR UE.

Observation 1: ISD assumption may need to be revisited according to the Reader Rx SINR.

Proposal 6: 2-step model is used for the device ACLR.

Proposal 7: BW scaling should be used for the emission fall into AIoT device Rx band, i.e. only the interference of AIoT BW needs to be taken into account.

Proposal 8: Multiple AIoT devices are assumed in the co-existence simulation to derive reasonable ACLR requirements.

For the specific assumptions for standalone mode

Observation 2: AIoT reader power in standalone mode may need to be reduced to mitigate the severe interference to NR DL.

For the specific assumptions for In-band mode

Proposal 9: 38 dBm co-site AIoT reader in-band power is assumed as the starting point for the co-existence simulation. If severe interference is found, the lower power level should be used.

Observation 3: Link level simulation can be conducted to check the interference between NR DL and AIoT R2D.

Observation 4: For the link of reader interfering NR DL, the interference is negligible assuming AIoT signal PSD is 10 dB higher than NR signal.

Observation 5: For the link of NR DL interfering reader, AIoT device ACS can be very relaxed, ~ 10 dB, assuming AIoT signal PSD is 10 dB higher than NR signal.

Proposal 10: No co-existence simulation is needed for the interference between NR DL and AIoT R2D.

References

[1] R4-2515152, “Way Forward for [116bis][321] Study of A-IoT for outdoor”, RAN4#116b, LGE