3GPP TSG-RAN WG4 Meeting #117	R4-2520888

# Dallas, USA, Nov. 17-21, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Ku-band NTN with HD-FDD

Agenda item:	7.6.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on HD-FDD Ku-band based on the WF [1].

Discussion

Requirements for HD-FDD VSAT

In the last RAN4 meeting, there were high level discussion and agreements for HD-FDD Ku-band with FR1 numerology.

Agreement - Define the HD-FDD VSAT with FR1 numerology RRM requirements based on the Rel-19 Ku-band NTN FDD requirements, and incorporate updates to address HD-FDD impacts, using the RedCap NTN approach as the baseline. - The following RRM requirements scope for HD-FDD VSAT in Ku band with FR1 numerology - Cell reselection, including paging reception - Handover and CHO - Satellite switching - Random access - UE UL timing accuracy - RLM, including scheduling availability and L1 indication - intra/inter-frequency measurements, including scheduling availability - L1-RSRP measurement, including scheduling availability

RAN4 ageed that RedCap NTN approach can be baseline to address HD-FDD in Ku-band NTN. Even if the UE type in the HD-FDD Ku-band scenario is VSAT, the impacts introduced by HD-FDD operation in RedCap NTN are applicable to VSAT UEs. Therefore, for following RRM requirements for HD-FDD VSAT in Ku-band, the existing NTN requirements in Ku-band can be reused by adding the condition that at least one SSB is available within a given periodicity (e.g., SMTC).

- Cell reselection, including paging reception

- Handover and CHO

- Satellite switching

- Random access

- Proposal 1: For following RRM requirements for HD-FDD VSAT in Ku-band, the existing NTN requirements in Ku-band can be reused by adding the condition that at least one SSB is available within a given periodicity (e.g., SMTC).

  - Cell reselection, including paging reception

  - Handover and CHO

  - Satellite switching

  - Random access

For UL timing accuracy, there is no impact on HD-FDD VSAT in Ku-band, so existing NTN requirement can be reused.

- Proposal 2: For UL timing accuracy, there is no impact on HD-FDD operation in Ku-band.

For the requirements for RLM/link recovery/L1-RSRP, including scheduling availability and L1 indication, the condition of collision cases and scheduling restriction have been defined in RedCap NTN as below:

For RedCap NTN with HD-FDD operation (e.g., BFD requirements) - Minimum requirements - For HD-FDD UE indicating CollisionHandlingOfHDFDDOperation, the requirements in clause 8.5E.3.2 shall also apply, provided that the following conditions are met: - there is no collision between the CSI-RS resources for BFD measurements and UL transmission, or - in case of collision between the CSI-RS resources for BFD measurements and UL transmission except PRACH, and ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision is not indicated by network. - In case of collision between the CSI-RS resources to be measured and PRACH, if UE indicates CollisionHandlingOfHDFDDOperation, it is up to HD-FDD UE implementation to either receive the CSI-RS or transmit the PRACH, according to TS 38.213. - Minimum requirement for L1 indication - In addition, for HD-FDD UE, the conditions and requirements for TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period. - Scheduling availability of UE during BFD - For HD-FDD UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during beam failure detection, except when UE indicates CollisionHandlingOfHDFDDOperation and is indicated that UL transmission is prioritized over BFD evaluation via ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision in case of collision between CSI-RS resources to be measured and UL transmission.

For HD-FDD in Ku-band, the same approaches captured above can be reused for RLM/link recovery/L1-RSRP since the collision rules are the same. However, collision rule related UE capabilities in HD-FDD operatio in NTN are only applicable for RedCap NTN. Therefore, RAN4 needs to discuss how to extend these UE capabilities to apply HD-FDD VSAT UE in Ku-band NTN.

- Proposal 3: For the requirements for RLM/link recovery/L1-RSRP in Ku-band NTN with HD-FDD operation, RAN4 to reuse the conditions for collision cases and scheduling availability as defined in RedCap NTN, and further discussion is required on how to extend and apply the UE capabilities related to collision rules, which are only applicable for RedCap NTN, to Ku-band VSAT.

For intra/inter-frequency measurements, including scheduling availability, it also reuse same approaches for RedCap NTN. Therefore, scheduling availability should be added for HD-FDD VSAT in Ku-band NTN as below.

- Scheduling availability of HD-FDD UE performing measurement for Intra-frequency measurement w/o gap

    - When the HD-FDD UE performs intra-frequency measurements in a FR1 numerology, the following restrictions apply due to SS-RSRP or SS-SINR measurement

      - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

      - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

    - When the HD-FDD UE performs intra-frequency measurements in a FR1 numerology, the following restrictions apply due to SS-RSRQ measurement

      - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

      - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

- Scheduling availability of HD-FDD UE performing measurement for Inter-frequency measurement w/o gap

    - When UE performs inter-frequency measurements without measurement gaps in a HD-FDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

      - UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration.

    - When UE performs inter-frequency measurements without measurement gaps in a HD-FDD band, the following restrictions apply due to SS-RSRQ measurement

      - UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration.

- Proposal 4: For intra/inter-frequency measurements, including scheduling availability in Ku-band NTN with HD-FDD operation, following scheduling availability can be defined

- Scheduling availability of HD-FDD UE performing measurement for Intra-frequency measurement w/o gap

    - When the HD-FDD UE performs intra-frequency measurements in a FR1 numerology, the following restrictions apply due to SS-RSRP or SS-SINR measurement

      - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

      - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

    - When the HD-FDD UE performs intra-frequency measurements in a FR1 numerology, the following restrictions apply due to SS-RSRQ measurement

      - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

      - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

- Scheduling availability of HD-FDD UE performing measurement for Inter-frequency measurement w/o gap

    - When UE performs inter-frequency measurements without measurement gaps in a HD-FDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

      - UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration.

    - When UE performs inter-frequency measurements without measurement gaps in a HD-FDD band, the following restrictions apply due to SS-RSRQ measurement

      - UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration.

TA reporting

In last RAN1 meeting, RAN1 had conclusion for TA reporting enhancement for RedCap HD-FDD below.

Conclusion Rel-19 NTN doesn’t support TA report enhancement for HD-FDD (e)RedCap.

To minizie TA mismatch issue between actual TA used by the UE and assumed TA for the UE at the gNB in HD-FDD operation, TA reporting is beneficial, but it is optional UE capability. If UE does not support TA reporting, network should handle the TA mismatch and UE receive and transmit signal based on collision rule, so UE throughput might be degraded. RAN1 already agreed not to enhance TA reporting and introduce other solution for th TA mismatch for HD-FDD operation in NTN, RAN4 needs to discuss whether TA reporting should be considered as mandatory.

- Proposal 5: RAN4 needs to consider TA reporting as mandatory if no discussion to resolve TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB in HD-FDD operation is discussed in RAN1.

Conclusion

In this contribution, we provide our views on Rel-20 HD-FDD Ku-band NTN with FR1 numerology, and we propose

- Proposal 1: For following RRM requirements for HD-FDD VSAT in Ku-band, the existing NTN requirements in Ku-band can be reused by adding the condition that at least one SSB is available within a given periodicity (e.g., SMTC).

  - Cell reselection, including paging reception

  - Handover and CHO

  - Satellite switching

  - Random access

- Proposal 2: For UL timing accuracy, there is no impact on HD-FDD operation in Ku-band.

- Proposal 3: For the requirements for RLM/link recovery/L1-RSRP in Ku-band NTN with HD-FDD operation, RAN4 to reuse the conditions for collision cases and scheduling availability as defined in RedCap NTN, and further discussion is required on how to extend and apply the UE capabilities related to collision rules, which are only applicable for RedCap NTN, to Ku-band VSAT.

- Proposal 4: For intra/inter-frequency measurements, including scheduling availability in Ku-band NTN with HD-FDD operation, following scheduling availability can be defined

- Scheduling availability of HD-FDD UE performing measurement for Intra-frequency measurement w/o gap

    - When the HD-FDD UE performs intra-frequency measurements in a FR1 numerology, the following restrictions apply due to SS-RSRP or SS-SINR measurement

      - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

      - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

    - When the HD-FDD UE performs intra-frequency measurements in a FR1 numerology, the following restrictions apply due to SS-RSRQ measurement

      - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

      - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

- Scheduling availability of HD-FDD UE performing measurement for Inter-frequency measurement w/o gap

    - When UE performs inter-frequency measurements without measurement gaps in a HD-FDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

      - UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration.

    - When UE performs inter-frequency measurements without measurement gaps in a HD-FDD band, the following restrictions apply due to SS-RSRQ measurement

      - UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration.

- Proposal 5: RAN4 needs to consider TA reporting as mandatory if no discussion to resolve TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB in HD-FDD operation is discussed in RAN1.

Reference

[1] R4-2515082, “Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2,” MediaTek