3GPP TSG-RAN WG4 Meeting #117	R4-2520893

# Dallas, USA, Nov. 17th – 21st, 2025

Source:	LG Electronics

Title:	Discussion on 6G RAN4 RRM Study

Agenda item:	8.6

Document for:	Discussion

Introduction

In last RAN4 meeting, many issues related 6G RRM were proposed and discussed, and it was agreed that measurement gap and RRM framework are as part of RAN4 RRM 6G study [1]. However, the detailed scope should be discussed.

Agreement Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. Agreement RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

Also, in this meeting, it is necessary to determine the priority of the listed RRM issues below for discussion within the RRM SI. Furthermore, the organization of issues according to the check-point settings discussed in the previous meeting should be considered in order to facilitate the smooth progress of the 6G RAN4 RRM SI.

- Mobility related RRM - RRM related energy efficiency - Spectrum aggregation related RRM - MIMO and mTRP operation related RRM - NTN related RRM - Initial access related RRM - Other PHY signal/channel/procedure related RRM

In this contribution, we provide our views on 6G RRM related issues based on the WF [1].

Discussion

Measurement Gap

In RAN4#116bis meeting, RAN4 trive to identify the feature, which is less impacted by other WGs. Based on the discussion, measurement gap including gap-less measurement will be studied as part of RAN4 RRM 6G study. In agreed WF for 6GR [1], following was captured regarding measurement gap.

Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. [Way forward] - FFS followings in RAN4#117 (other sub-topics are not precluded) - based on the majority views from companies, RAN4 RRM to discuss which MG related sub-topics can be prioritized from the following candidate list, and then discuss the solutions for the selected topics in 6G SI: - MG pattern reduction from 5G (10 companies: Apple, MTK, QC, OPPO, Sony, Xiaomi, vivo, Ericsson, Samsung, Nokia) - Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain. - Unified MG (7 companies: Apple, OPPO, LGE, Xiaomi, ZTE, Ericsson, QC), e.g., - Unified MG configuration - Unified MG and scheduling restriction - Unified MG for different feature related measurements - Adapative MG operation, e.g., activation/deactivation/cancellation/skipping (7 companies: Apple, OPPO, CMCC, Xiaomi, ZTE, Ericsson, LGE) - Using which 5G MG enhancement features to 6G day 1 (OPPO, CMCC, Xiaomi, vivo) - E.g., needforGap, NCSG, concurrent MG, preconfigured MG and etc. - MG sharing(Apple, OPPO, vivo), e.g., - among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement) - UE assisted MG configuration (Apple, QC) - e.g., MG requesting by UE (Apple, QC) - Multi-CC measurements in MG (Apple, CMCC, ZTE(searcher related)) - Optimization on MGL and RF tuning/retuning (Apple, MTK, QC) - MG applicability for per-UE, per-FR, per-CC, or per-CC group (OPPO, Xiaomi, ZTE(granularity))

According to WF, RAN4 will disuss which MG related sub-topics can be prioritized from the following candidate list. Below, we provide our view on the sub-topics to be prioritized during RAN4 RRM 6G study.

MG pattern reduction from 5G

In 5G NR, the measurement gap (MG) configuration is defined by a combination of Measurement Gap Length (MGL), and Measurement Gap Repetition Period (MGRP). The current NR specification supports a variety of gap patterns, where each MGL value {1.5, 3, 3.5, 4, 5.5, 6} ms is paired with multiple MGRP values {20, 40, 80, 160} ms. This results in a comprehensive set of predefined gap patterns.

However, based on the views shared by companies during previous RAN4 meetings, it has been observed that not all defined gap patterns are actively used in commercial deployments. Instead, only a subset of the available patterns is commonly utilized. Since the actual usage of measurement gap patterns is determined by implementation and deployment preferences, it is reasonable to consider reducing the number of unused patterns in the specification.

However, reduction of gap aptterns without considering on going 6GR discussion may lead to unnecessary limitation for 6G MG. For example, in the ongoing 6GR study, RAN1 is discussing the possibility of extending the default SSB periodicity up to 160 ms. If such an extension is adopted, it may be necessary to introduce longer MGRP values (e.g. 160 ms or beyond) for inter-frequency measurements to ensure alignment with the extended SSB periodicity.

Furthermore, majority of companies in other WG consider OD-SSB, which allows the network or UE to trigger additional SSB transmissions at shorter periodicity when needed, as the compensatory mechanism for measurement and mobility caused by the increased default SSB periodicity. Therefore, maintaining shorter MGRP values such as 20, 40, and 80 ms remains important to support flexible measurement operations.

Another point of consensus among companies is that measurement gaps inherently limit overall throughput. In principle, measurement gaps should only be configured and used when strictly necessary. This principle suggests that 6GR MG may benefit from more finer granularality over MGL, potentially introducing shorter MGL than those currently defined in 5G NR.

In conclusion, rather than aiming to reduce the number of measurement gap patterns uniformly, it is more appropriate to study which patterns are realistically needed based on current deployment practices and anticipated requirements from ongoing 6GR studies. This approach ensures that the specification remains both efficient and forward-compatible with future RRM enhancements.

- Proposal 1: RAN4 to prioritize MG pattern reduction from 5G

  - Study of realistic gap pattern based on current deployment practices and progress from ongoing 6GR studies

Gap-less measurement and its side conditions

The introduction of MG is primarily motivated by the limitation that a UE equipped with a single RF chain cannot simultaneously perform data transmission/reception and measurements on different frequencies. However, in order to support mobility procedures, the UE is required to perform inter-frequency or inter-RAT measurements, which inevitably leads to throughput degradation due to MG.

In scenarios where the UE has additional available RF chains or when mobility-related measurements are not required, the configuration of MG becomes unnecessary. In previous meeting, majoirity of companies share the same view on studying the gap-less measurement and scenario/condition. We also open to prioritize gap-less measurement for 6GR study. There are scenario and conditions to be discussed in this topic.

One scenario is that the UE can perform gap-less measurement when the UE has spare RF chain for measurement. This scenario has been discussed and introduced in 5G, e.g. needForGAP and/or NCSG. Current 5G solutions can be starting point for MG-less measurement. Another scenario is that UE can perform gap-less mesausrement without sapre RF chain. We are open to investigate gap-less measurement without spare RF chain.

In conclusion, since gap-less measurement fundamentally addresses the limitations of MG, it is essential to further investigate and discuss this feature in 6GR study. We would like to suggest that the 5G gap-less feature is starting point, and further investigate condition of gap-less with RRM relaxation issue.

- Proposal 2: RAN4 to prioritize study of gap-less measurement

  - 5G gap-less solution, e.g. needForGAP and/or NCSG can be starting point

Unified MG

In 5G NR, various MG enhancement features such as NCSG, concurrent MG, preconfigured MG, positioning GAP, and MUSIM GAP have been introduced. These features were specified at different stages of NR releases, resulting in fragmented requirements and lack of a unified configuration framework. When multiple MG features are configured simultaneously, additional requirements are needed, leading to complex operational behavior.

For 6G NR study, a simplified MG configuration mechanism is desirable. In WF[1], unified MG configuration, which integrates all existing MG types (e.g. NCSG, concurrent MG, preconfigured MG, positioning GAP, and MUSIM GAP) under a single framework is proposed. The principle of unified MG configuration is to support flexible activation and deactivation of various MG types across different services, which can enabe dynamic and efficient measurement operations.

At least from our understanding, as illustrated in Figure 1, the unified MG configuration can be an extension of the existing concurrent MG configuraion, and extended to encompass all MG types. Within the MG occssion, the UE can autonomously perform various measurement operations including intra-frequency, inter-frequency, inter-RAT measurements, as well as RRM, MUSIM, and positioning measurements, through MG sharing.

Regarding including scheduling restriction as part of MG, we think that the motivation is to simplify the measurement procedure, and increase the throughput. Since scheduling restriction, and MG has similar UE behaviour, how to minimize the restriction can be studied during the study phase.

In conclusion, we believe that unified MG is necessary, and should be prioiritized in 6GR study.

- Proposal 3: RAN4 to prioritize unified MG

Figure 1 Example of unified MG configuration

Adapative MG operation, e.g., activation/deactivation/cancellation/skipping

In 6G, network energy saving is expected to be a critical KPI, and OD-SSB is considered as basis of the overall measurement framework. As mentioned beofre, the introduction of OD-SSB, enabled by increasing the default SSB periodicity, is highly likely to be applied to both serving cell and neighbor cell measurements.

If OD-SSB is adopted from Day 1 in 6G, the SSB periodicity may change dynamically. To align with such dynamic behavior, the periodicity of the MG must also be adaptable. Additionally, based on specific conditions such as high throughput demands or the availability of spare RF chains, it may be necessary to cancel or update the MG configuration. When mobility based on MG is not neede, for example, if a UE reports that the UE is in the stationary sate, the UE would perform less frequent inter-frequency/inter-RAT measurements. NW can deactive MG or increase MGRP at this moment to accommodate UE higher throughput. Regarding adaptive MG operation, as illustrated in Figure 2, several MG configurations can be dynamically activated or deactivated.

Therefore, it is necessary to prioritize adaptive MG operaition as part of 6G RRM.

- Proposal 4: RAN4 to prioritize adaptvie MG operation

Figure 2 Example of adaptive MG configuration

MG applicability for per-UE, per-FR, per-CC, or per-CC group

In 5G NR, MG can be configured either per-UE or per-FR. The per-UE applies uniformly across all frequency layers, whereas the per-FR allows independent measurement gap patterns for each frequency range. 5G NR MG inherited from LTE MG, and introduced coarse granuarility to decrease implementation complexity. We believe that 6G UE or device can handle more complex implementation than those of 5G, so finer granularity for MG should be considered for 6G RRM strudy, e.g. per-CC, or per-CC group. This finer granularity would enable higher thoughput for CA operation. Therefore, it is essential to prioritize the discussion on MG applicability for 6G RRM study

- Proposal 5: RAN4 to prioritize MG applicability for per-UE, per-FR, per-CC, or per-CC group

Using which 5G MG enhancement features to 6G day 1

In 5G NR, multiple enhancements have been introduced to improve MG functionality, including NeedForGap, NCSG, concurrent MG, preconfigured MG, positioning GAP, and MUSIM GAP. It is evident that 6G MG should be further developed based on these 5G MG enhancements.

As the 6G study is still in its early phase, it is premature to select which features should be included or precluded. At this stage, it is recommended to keep the scope open to all potential features. Decisions can be made during the normative work phase after study phase.

- Proposal 6: RAN4 to deprioritize selection of which 5G MG enhancement features to be used for 6GR study

MG sharing

Regarding MG sharing, several companies’ view is that MG can be shared across intra-frequency, inter-frequency, and inter-RAT measurements, including both L3 and L1 measurements. For 6G handover, majority of company expecte that LTM will be supported from Day 1. Since LTM relies on L1 based measurements to detect and evaluate neighboring cells, it is essential that L1 measurements can be included in the scope of MG sharing.

The vision for 6G is to enable seamless operation of diverse services under a unified system framework. In MG-based measurement operations, the UE must be capable of supporting multiple MG configurations to accommodate various service requirements. While 5G NR did not support configurations that include multiple MGs simultaneously, the 6GR study is exploring the possibility of a unified MG configuration that can support multiple services concurrently. We believed that MGs required for different services can coexist through MG sharing, allowing legacy mobility measurements and new service-specific measurements to operate in unfied frameworks.

For example, if MG is required for positioning or MUSIM operations overlaped with those used for conventional mobility measurements, MG sharing can be applied to optimize resource usage. Therefore, it is reasonable to consider that MG sharing should not be limited to L1 and L3 measurements alone, but also include specific service features.

In conclusion, we propose that MG sharing can be discussed including L1, L3 measurements, and specific service measurements. Furthermore, this topic can be discussed under the unifid MG topics.

- Proposal 7: RAN4 to merge MG sharing into unified MG for different feature related measurements

UE assisted MG configuration

Regarding UE assisted MG configuration, it can be discussed, once 6G MG configuration is clear. Therefore, we think this topic can be deprioritized.

- Proposal 8: RAN4 to deprioritize UE assisted MG configuration

Multi-CC measurements in MG

Regarding Multi-CC measurement in MG, RAN4 need to focus single-CC measurements in MG first. Therefore, we think this topic can be deprioritized.

- Proposal 9: RAN4 to deprioritize multi-CC measurements in MG

Optimization on MGL and RF tuning/retuning

Regarding optimization on MGL and RF tuning/retuning, this topic may related to UE RF switching, which needs more time to discuss. Therefore, we think this topic can be deprioritized.

- Proposal 10: RAN4 to deprioritize optimization on MGL and RF tuning/retuning

RRM framework

During the previous RAN4 meeting, it was agreed that RAN4 would initiate a study on the RRM framework which address multiple aspects including measurement capability, delay, overhead, accuracy, measurement quantities, and unified measurement principles. Proposed topics from companies in the last meeting [1] are listed below

- Measurement capability - measurement capability for number of cells, beams and frequency layers (MTK, OPPO, Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - RF retuning time, baseband processing time for typical use cases - UE reference architecture for 6G spectrums - Baseline assumptions of RRM requirements for different UE device types - Measurement delay/overhead - Searcher number for enhanced simultaneous measurements (OPPO, HW, Samsung, ZTE, vivo, Ericsson) - Rx beam sweeping factor reduction (QC, Ericsson) - Identification/measurement/tracking/reporting delay reduction (QC) - RRM with NW aided measurement priority (Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - Unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes. (Nokia) - SSB evaluation (Samsung) - Intra and inter-frequency definition - Unified measurements - United/integrated cross-layers measurement and/or report between L1 and L3 (Samsung, OPPO, CMCC, LGE, Xiaomi) - United/integrated cross-functions measurement and/or report for L1 (e.g., integration of MIMO and LTM, or integration of RLM/BFD/CBD)(OPPO, CMCC, Xiaomi) - RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement. Unification in terms of reusing L3 measurements for L1 before certain point (e.g., before TCI state activation) (Ericsson)

These topics include both independent issues and interrelated aspects that require joint discussion. Furthermore, RRM-related considerations cannot be isolated from other feature discussions in other sections, such as energy efficiency and mobility. For example, L1 and L3 measurements should be addressed in conjunction with mobility RRM issues to ensure coherent specification development.

In past RAN4 specifications, each Release introduced new features and enhancements, resulting in increasingly segmented and detailed measurement requirements. While these requirements supported advanced functionalities, they also led to redundant measurement and reporting, increased measurement delays, and higher UE power consumption. This was an inevitable consequence of fragmented, feature-dependent measurement definitions. For 6GR, there is a clear need to study an integrated RRM measurement framework that reduces unnecessary measurements and reporting across layers and features/functionalites, thereby simplifying UE and network operation and lowering power consumption. Building on lessons learned from 5G NR, RAN4 should study a unified 6G RRM measurement framework that replaces fragmented, feature-dependent measurement operations with a cross-layer and cross-feature/function integrated approach. L3 measurements are defined as filtered results derived from L1 measurements. Since Layer-1 based received signal measurements such as BFD, RLM, and L1-RSRP are performed using the same or similar reference signals (e.g., SSB or CSI-RS), these reference signals can also be utilized or applied for L3 measurement purposes. Furthermore, the L1 measurement results can be reused and applied for different objectives, such as serving-cell link monitoring, beam failure detection, beam candidate identification, and L1-RSRP evaluation. Such an operation can mitigate measurement or scheduling restrictions caused by overlapping measurement resources, thereby reducing measurement delay and preventing potential degradation of data throughput. Of course, for this operation, aspects such as measurement resources alignment/configuration, measurement filtering, timing/reporting synchronization, and event triggering evaluation/reporting between cross-layer shall be further studied.

- Proposal 11: RAN4 to study feasibility and approaches of unified measurement between cross-layer and feature/function aiming to minimize redundant measurement operations, reduce UE power consumption, alleviate measurement and scheduling restrictions.

In the current specifications, measurement requirements are defined separately for L1 measurement, intra-frequency and inter-frequency measurements, and further distinguished based on whether a measurement gap is configured or not. While this structure provides flexibility for various deployment scenarios, the continuous introduction of new features has increased specification complexity due to repeated application of the same distinctions. For 6GR, it is considered beneficial to extend the current definition of intra-frequency measurement to cover broader cases, such as measurements based on reference SSBs within the same BWP. In addition, it is proposed to simplify the existing requirement framework by unifying the conditions of measurement with and without gaps. Furthermore, to efficiently support energy-saving features and to take advantage of OD-SSB structures discussed under energy efficiency studies, RAN4 can consider defining the measurement requirements between serving cell measurements and neighbor cell measurements separately.

Table 1 5G measurement requirements

| Type | Measurement | Target cell | Use case |
| --- | --- | --- | --- |
| L1 measurement | RLM | Serving cell | Cell level link monitoring |
|  | BFD/CBD | Serving cell | Beam level link monitoring |
|  | L1-RSRP | Serving cell | For various purpose (CSI, SCell activation, L1/L2 triggered mobility…) |
|  |  | Neighbor cell | L1/L2 triggered mobility |
| L3 measurement | Intra-frequency without gaps | Serving cell / neighbor cell | Mobility |
|  | Intra-frequency with gaps | Serving cell / neighbor cell | When reference SSB is not located within active BWP |
|  | Inter-frequency without gaps | Neighbor cell | When SSB is located within active BWP, but different frequency with reference SSB |
|  | Inter-frequency with gaps | Neighbor cell | Mobility |

Table 2 Example of 6GR measurement requirements

| Type | Measurement | Target cell | Use case | RS measurement |
| --- | --- | --- | --- | --- |
| L1 measurement | BFD/CBD | Serving cell | Beam level link monitoring | Share measurement results for cross-layer and cross-functions to avoid redundancy |
|  | L1-RSRP | Serving cell | For various purpose (CSI, SCell activation, L1/L2 triggered mobility…) |  |
|  |  | Neighbor cell | L1/L2 triggered mobility |  |
| L3 measurement | RLM | Serving cell | Cell level link monitoring |  |
|  | Intra-frequency without gaps | Neighbor cell | Mobility (includes the case when SSB is not within active BWP) |  |
|  | Inter-frequency with gaps | Neighbor cell | Mobility (includes the case when SSB is located within active BWP, but different frequency with reference SSB) |  |

- Proposal 12: RAN4 to study to redefine the current intra/inter-frequency measurement requirements structure for improved measurement efficiency such as the definition of intra-frequency or separate serving and neigbhor cell measurements requirements.

In 5G NR, a UE performed multiple measurements and reporting by network configuration, leading to power consumtion and measuremet/scheduling restriction. For 6GR, RAN4 should study UE-assisted information based measurement operations, where the UE provides contextual information to determine whether a measurement is required. This approach can prevent unnecessary measurement operations, reduce power consumption, and improve system efficiency, while assessing potential system-level gains and trade-offs.

- Proposal 13: RAN4 to study UE contextual information based measurement operation to prevent unnecessary measurement operations, reduce power consumption, and improve system efficiency

6GR will support diverse environments, including portable devices and vehicular communications. In scenarios where multiple devices experience similar channel conditions (e.g., within the same vehicle), each UE currently performs independent measurements and reporting. RAN4 should study the feasibility and benefits of a virtual UE group concept, enabling shared measurement and reporting among UEs in similar environments. This could significantly reduce measurement and reporting overhead in 6G systems.

- Proposal 14: RAN4 to study feasibility and benefit of virtual UE group measurement and reporting to reduce measurement and reporting overhead in 6GR

Mobility related RRM

In the last RAN2 meeting (113BIS), it was agreed that 6G mobility design should minimize interruption, enhance robustness, and improve energy efficiency while considering aspects such as early synchronization and conditional handover. Specifically, RAN2 captured the following requirements for 6G mobility design:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In parallel, RAN4 has also captured a Way Forward for mobility-related RRM in the last meeting [1], where the group agreed to start from mobility solutions with less RAN1/2-dependency.

Issue 6: Mobility related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

Mobility plays a fundamental role in ensuring service continuity in wireless systems. In 5G, mobility support is defined for RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states, with this section focusing on the connected mode where user data transmission is active. In this state, mobility procedures encompass handover, RRC re-establishment, redirection after release, and L1/L2-triggered inter-cell mobility.

The corresponding NR requirements specify delay and interruption performance for various handover schemes, along with RRC connection control functions such as re-establishment and random access.

Unified Mobility for 6G

L3/RRC-based Handover

The traditional L3-based handover procedure operates through the RRC layer. It relies on L3 measurement reporting (e.g., RSRP/RSRQ filtering and event-triggered measurement reports) sent by the UE to the network. Upon receiving these reports, the network performs handover decision-making and transmits a handover command containing the full configuration of the target cell. The UE then performs RRC decoding, downlink synchronization, random access (RACH), and uplink synchronization sequentially. This top-down process ensures robustness and full network control but inherently suffers from latency and signaling delay, as each step depends on RRC-level coordination. In practice, most interruption originates from DL synchronization and RACH access, which cannot begin until the UE receives the handover command.

L1/L2-triggered Mobility (LTM)

In contrast, LTM shifts the handover triggering and synchronization preparation to lower layers, the MAC layer. The UE can perform early DL/UL synchronization and candidate cell preparation before receiving the final handover command.

The trigger decision is based on L1 measurement results (e.g., RSRP or CSI-RS-based evaluations) and executed via MAC-CE signaling. This design minimizes handover interruption and allows the cell switch to occur with sub-20 ms interruption under favorable conditions. However, the benefit is realized only when strict timing relationships between measurement, synchronization, and command signaling are satisfied (e.g., TCI activation gap < 160 ms).

While both frameworks coexist in NR, they operate independently, with L1 and L3 maintaining separate measurement definitions, and triggering criteria.

This separation between measurement and mobility management across layers leads to several inefficiencies:

- Duplicated measurement operations: The UE performs both L1 and L3 measurements for the same candidate cells, increasing computational load and power consumption.

- Inconsistent timing budgets: L1-based pre-synchronization and L3-based command-driven handovers use different timing models, complicating cross-layer optimization.

A unified mobility is therefore essential in 6G to harmonize measurement, decision, and execution across layers. Integration would allow shared measurement results, synchronized timing references, and consistent control between UE and network —reducing duplication, simplifying UE implementation, and achieving faster yet more reliable handover operations. Such an approach aligns with the goal of minimizing interruption while maintaining energy efficiency and scalability across diverse mobility conditions.

Figure 3 Unified mobility for 6G

- Proposal 15: RAN4 to study a cross-layer alignment between L3/RRC-based and L1/L2-triggered mobility procedures to eliminate redundant measurements and define consistent timing and performance requirements for 6G.

Evolving Concept of Mobility in 6G

With the emergence of 6G, radio environments will become increasingly heterogeneous — combining dense cell deployments, Non-Terrestrial Network (NTN) extensions, and AI-assisted scheduling. In such conditions, mobility can no longer be treated as an isolated handover procedure, but should evolve into a continuously adaptive coordination of RRM behaviors responding to the UE’s motion and radio context.

In current NR, mobility control remains largely event-driven and network-triggered. The UE performs measurements and reports based on static thresholds and periodicities configured by the network. While this ensures deterministic control, it lacks adaptability: the same configuration is applied regardless of UE speed, beam stability, or serving-cell region, leading to inefficiencies in both robustness and energy consumption; for example, a high-mobility UE may require denser measurements and shorter synchronization windows to maintain reliability, whereas a stationary UE under stable beam and low-interference conditions could safely operate with relaxed periodicity to conserve energy.

To address these limitations, mobility in 6G should be redefined as a UE-triggered and tontext-aware RRM system, where the network and UE jointly manage mobility as a dynamic state rather than a discrete procedure. In this concept, all RRM behaviors — measurement, synchronization, reporting, and configuration — are cohesively coordinated in response to the UE’s context. The goal is to achieve context-aware and proactive adaptation, enabling seamless continuity, robustness, and energy efficiency across diverse mobility conditions.

Table 3 Dimensions of Mobility Adaptation in 6G

| Mobility Category | Meaning | Related keywords |
| --- | --- | --- |
| Temporal mobility | Variations in time budgets caused by UE mobility | Unified latency budget, HO latency, interruption |
| Spatial mobility | Adaptation of measurement and synchronization behavior as the UE moves across different spatial or coverage regions (e.g., cell-center/edge) | RSRP-based region detection, adaptive measurement |

For 6G, RAN4 should study a UE-triggered, context-aware mobility mechanism that enables the device to autonomously initiate or prepare mobility actions within network-defined boundaries.

- Proposal 16: RAN4 to study a UE-triggered and context-aware RRM mechanisms for seamless and energy-efficient mobility in 6G.

RRM related energy efficiency

In agreed WF for 6GR [1], following was captured regarding RRM related energy efficeiency.

Issue 7: RRM related energy efficiency [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify candidate topics can be studied in RAN4 directly: - Network energy saving: - RRM for new SSB design(e.g., SSB periodicity extension, OD-SSB/OD-SIB1) (vivo, Ericsson, Nokia, ZTE) - SSB-less based RRM (Ericsson, Nokia, ZTE) - UE power saving: - UE type/state based RRM relaxation (Apple, CMCC, Ericsson(scalable set of measurement requirement), Nokia) - LR based solutions for UE power saving (Sony, vivo, Ericsson) - DRX/eDRX based measurement (Ericsson) - Energy sensing based RRM strategy (CATT) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other energy efficiency related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other energy efficiency related RRM topics once there are sufficient conclusions from other WGs.

Meanwhile, in RAN1#122bis following agreement in 6G energy efficiency agenda has been achieved.

Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Unclustered provisioning with cell-common signaling, UE impact: - Cell search complexity and latency, including frequency search latency, - UE Power consumption, - Sync signal detection, coverage and tracking performance, - RRM, mobility, - Beam management, - Other properties are not precluded, - Improvements to address identified impact, e.g., - Additional sync signal needs, - Adaptation of sync signal transmission periodicity, - Sparser synch raster. Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities Agreement Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,: - On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP, - Network-triggered and UE-triggered on-demand sync signal(s), - Idle and/or connected modes, - Other mechanisms/aspects/signals/channels are not precluded. Agreement Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,: - Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios, - RRC states, - UE energy consumption and complexity, - Other mechanisms/aspects/signals/channels are not precluded.

According to the RAN1 agreement, agreements have been reached regarding the default SSB periodicity, DL WUS, OD-SSB, and SSB-less Cell. Our perspective on the related RAN1 agreements and the topics discussed in the RAN4 WF can be summarized as follows:

- UE type/state based RRM relaxation

  - Impact

    - RRM relaxation requirement

    - RRM relaxation UE type/condition

  - Whether above topic can be studied in RAN4 directly

    - Possible

    - Similar principle has already been introduced in 5G NR, e.g. RLM/BFD relexation, LP-WUS MR relaxation, and Measurement relaxation in RRC_IDLE/INACTIVE

    - RAN4 can study which UE type or condition can be applied

- DRX/eDRX based measurement

  - Impact

    - RRM requirement based on DRX/eDRX

  - Whether above topic can be studied in RAN4 directly

    - Not possible

    - RAN2 try to study the power saving framework that includes both Cell DTX/DRX and UE DRX/eDRX

    - More progress from other WG is needed

- Increase of default SSB periodicity

  - Impact

    - RRM, Mobility

    - Beam management

    - Timing requirement

  - Whether above topic can be studied in RAN4 directly

    - Not possible

    - Physical design of SSB and/or CSI-RS is not clear

    - Mobliity/Beam management mechanism is not clear

    - More progress from other WG is needed

- DL WUS of OFDM based sequence

  - Impact

    - LR Measurements and/or synchronization

    - LR neighbor cell measurement

  - Whether above topic can be studied in RAN4 directly

    - Not possible

    - Assumption of LP-SS is not clear

    - More progress from other WG is needed

- On-demand sync signal(s) mechanisms for single cell/carrier, multi-carrier/cell, multi-TRP

  - Impact

    - OD-SSB for PCell in RRC_IDLE/CONNECTED

    - NW triggerd and UE-triggered OD-SSB

  - Whether above topic can be studied in RAN4 directly

    - Not possible

    - All of the OD-SSB mechanism is not clear

    - Physical design of SSB and/or CSI-RS is not clear

    - More progress from other WG is needed

- Sync signal-less carriers/cells/TRPs

  - Impact

    - At least for intra-band and collocated inter-band multi-carrier/cell/TRPs

    - Extension to additional deployments and scenarios

  - Whether above topic can be studied in RAN4 directly

    - Possible

    - At least for intra-band and collocated inter-band multi-carrier/cell/TRPs can be studied, since this condition has already been supported in 5G

In conclusion, since there is currently no stable agreement on 6G energy efficiency, most of the related topics should be deprioritized except UE type/state based RRM relaxation and SSB-less cell, whose principle has already been adopted in 5G. The study on RRM relaxation and SSB-less cells appears to align closely with the current 5G specifications, so it may be feasible to proceed directly in RAN4.

- Proposal 17: RAN4 to prioritize UE type/state based RRM relaxation and SSB-less Cell for RRM related energy efficiency, and deprioritize other issues

- Proposal 18: RAN4 to set check-point in 2Q 2026 to discuss whether there are any RAN4 issues for RRM related energy efficiency after sufficient progress in RAN1.

Spectrum aggregation related RRM

For spectrum aggregation related RRM, following proposals were submitted in the last RAN4 meeting. As mentioned in the summary below, some of issues can be starts directly in RAN4 RRM without other WG, and for some issues, further progress in discussions from other WGs is still needed.

Proposals from RAN4#116bis - RAN4 to identify which of the following topics can be starts directly in RAN4 RRM with less RAN1/2 and RAN4 RF session dependency: - SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations (MTK(SCell activation), QC, vivo(activation), Ericsson, Nokia) - RRM conditions and requirements for Single Cell Multi-Carriers (MTK, vivo, Samsung, OPPO) - RRM impacts of DL and UL decoupling (Samsung, Ericsson) - Carrier switch enhancements for UL and DL (MTK) - RRM impacts of realistic SCS for spectrum (Samsung) - relaxation of the requirement on timing alignment between carriers (Ericsson) - RRM for MRSS (Samsung)

In every Release in 5G NR, various SCell activation scenarios, considering different UE capability configurations, have been evaluated, and enhancement approaches to reduce SCell activation delay have been proposed and defined in RAN4. For 6G, a lean design for SCell activation applicable to all RRC states should be considered based on the approaches proposed and defined in 5G. Furthermore, improvements to mitigate interruptions during the SCell activation procedure should be studied, taking into account practical UE behavior and implementation aspects.

- Proposal 19: RAN4 to study lean design for SCell activation/deactivation procedure and delay requirements considering proposed and defined in 5G NR, and improvement of interruptions during the SCell activation procedure considering practical UE behavior and implementation aspects.

In 6GR, new spectrum aggregation-related features such as single-cell multi-carrier and DL/UL decoupling, could be introduced as listed above and if these features are introduced, there would be RAN4 RRM issues. However, addressing the RRM issues for these features requires progress in other WGs and/or RF session. Therefore, RAN4 should set a check-point and reconsider when RAN4 will initiate discussion for these.

- Proposal 20: RAN4 to set check-point to initiate discussion on RRM topics related other WGs such as single cell multi-carrier and DL/UL decoupling.

MIMO and mTRP operation related RRM

In previous meeting, following WF[1] has been agreed.

Issue 9: MIMO and mTRP operation related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Proposal 1(Apple): - study RRM impact from mTRP on different carriers, if supported. - Proposal 2(Samsung): - In 6GR, for MIMO operation and multi-TRP, RAN4 RRM to discussion on following aspects: - Multiple Tx and Multiple Rx with/without simultaneously in transmission/reception - TCI states: RRM only support unified TCI states framework - Further harmonized “TR point” and “cell” for mobility and measurement - Proposal 3(vivo): - MIMO/mTRP related RRM requirements (e.g., measurement for beam management, TCI state switching) need be studied based on concrete RAN1 assumptions and progress, and early RAN4 evolvement is needed w.r.t UE implementation constraint (e.g., UE multi-panel, Rx/Tx timing difference). - Proposal 4(Nokia): - RAN4 to study how to enable robust multi TRP operation considering both beam pairs and deployments

MIMO has continuously evolved across all 5G NR releases and remains a highly important feature. However, MIMO is a topic with strong dependencies on other WG, making it challenging for RAN4 to conduct independent studies. MIMO topic will be discussed from Q1 2026 in RAN1. Therefore, it is proposed that RAN4 deprioritize MIMO-related study items until sufficient agreements are reached in the relevant WGs.

- Proposal 21: RAN4 to set check-point in 2Q 2026 to discuss whether there are any RAN4 issues related MIMO after sufficient progress in RAN1.

NTN related RRM

In last RAN1 meeting, very high level agreements for 6G NTN were made.

RAN1 Agreements - The aspects to consider for supporting NTN include, but not limited to - Initial access, including cell search and SSB periodicity - Coverage - Duplexing - Capacity - Signalling overhead - GNSS-less/resilient/based operation - Large/varying doppler and propagation delay - Beamforming / beam management / beam hopping

In RAN1, NTN discussions are expected to take place on the above items with the objective of achieving integration between TN and NTN systems, and these discussions may have a direct impact on RAN4. Since 6G has clearly set the goal of an integrated design for TN and NTN, it is important that RAN4 also engages jointly with RAN1 within the scope of the RAN4 6G SI, and the following RAN4 RRM impacts from integrated TN-NTN design were proposed during the last meeting.

Proposals from RAN4#116bis - Proposal 1: - RAN4 to discuss for harmonized 6G Radio design for TN and NTN, can start from these aspects: - Re-consider which procedures can be common for TN and NTN. - Integrated TN-NTN mobility support. - Re-consider the framework and mechanisms for the different NTN UE types. - Proposal 2: - Within the harmonized 6G Radio design for TN and NTN, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control, the measurement on TN carrier shall have higher priority as the default assumption. - Towards 6G, the baseline UE measurement capability shall be reconsidered, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G. - Proposal 3: - RAN4 shall ensure that NTN RRM requirements remain aligned with the TN RRM framework in 6G, while incorporating updates to address NTN-specific aspects (which have already been introduced in earlier releases or may be introduced in 6G). - RAN4 shall study valid and effective NTN RRM requirements remain under both GNSS-resilient and GNSS-less operation. - Proposal 4: - NTN should be considered as a day-1 feature in NR and not be relegated to separate subclauses. This includes the measurement framework. - RAN4 to study how to simplify the measurement requirements for NTN. RAN4 to inform RAN1/2 about the conclusions of the study. - Proposal 5: - For Space-Air-Ground Integrated Network, RAN4 to study the optimization of RLM and access performance with frequent and significant changes in propagation delay, which may involve multi-dimensional optimization at least including frequency layer and spatial layer.

However, as the specific aspects impacting RAN4 will become clearer following the progress of RAN1’s discussion, it may be more efficient to set a check-point in 2Q 2026 and initiate the RAN4 study thereafter.

In 6G, NTN use cases are expected to differ from the scenarios assumed in 5G NR NTN, with the constraints of TN–NTN isolation being relaxed. This will require seamless mobility between TN and NTN systems. For example, for vehicular UEs supporting NTN, service continuity should be ensured when moving outside TN coverage areas, and in some cases, even within TN coverage where NTN service operation is needed. In such situations, smooth TN–NTN mobility should be supported. Furthermore, in Rel‑20 NR NTN, GNSS‑resilient operation is under discussion, and in 6G, both extended GNSS‑resilient scenarios beyond those assumed in 5G and GNSS‑less operation are clearly within scope of 6G NTN. Therefore, RAN4 should initiate studies on these topics with RAN1.

- Proposal 22: RAN4 should first study the RRM impacts of the following issues, which are clearly introduced and required in 6G NTN. Other RRM issues could be further discussed after 2Q 2026 (check-point) according to RAN1’s discussions.

  - TN-NTN mobility under integrated TN-NTN system

  - Both extended GNSS‑resilient scenarios beyond those assumed in 5G and GNSS‑less operation

Initial access related RRM

Following proposals for initial access related RRM were submitted in the last RAN4 meeting. As RAN1 will begin discussions on initial access from Q1 2026, it is premature at this stage to discuss whether there are any RAN4 RRM issues related to initial access. Such issues can be addressed after sufficient progress has been made in RAN1’s discussions on initial access. Therefore, RAN4 should set a check-point in Q3 2026 to discuss whether there are any RAN4 issues related to initial access.

- Proposal 1(Samsung): - In 6GR, for initial access, RAN4 RRM to discussion on following aspects: - Whether to specify the RRM requirements for initial cell search. To consider on following aspects: - Whether can find the start point to define such RRM requirement like “power on” - Necessity to specify such RRM requirements if “UE is powered on” happened infrequently. - Part of UE performance in initial cell search can be ensured by other procedures like cell identification; sync raster - RRM requirements are needed for cell selection - Whether to specify the RACH RRM requirements as functionality as correct UE behavior and tests in RRM. - Proposal 2(vivo): - The cell detection should be studied based on 6G SSB design. In addition, RAN4 may discuss whether necessary to introduce cell selection delay requirement whereas the same principle as that of 4G/5G, i.e., no corresponding cell selection delay requirement, can be used as the base.

- Proposal 23: RAN4 to set check-point in 3Q 2026 to discuss whether there are any RAN4 issues related to initial access after sufficient progress in RAN1.

Conclusion

In this contribution, we provide our views on the 6G RRM SI topics, and propose

Measurement gap(MG) and interruption

- Proposal 1: RAN4 to prioritize MG pattern reduction from 5G

  - Study of realistic gap pattern based on current deployment practices and progress from ongoing 6GR studies

- Proposal 2: RAN4 to prioritize study of gap-less measurement

  - 5G gap-less solution, e.g. needForGAP and/or NCSG can be starting point

- Proposal 3: RAN4 to prioritize unified MG

- Proposal 4: RAN4 to prioritize adaptvie MG operation

- Proposal 5: RAN4 to prioritize MG applicability for per-UE, per-FR, per-CC, or per-CC group

- Proposal 6: RAN4 to deprioritize selection of which 5G MG enhancement features to be used for 6GR study

- Proposal 7: RAN4 to merge MG sharing into unified MG for different feature related measurements

- Proposal 8: RAN4 to deprioritize UE assisted MG configuration

- Proposal 9: RAN4 to deprioritize multi-CC measurements in MG

- Proposal 10: RAN4 to deprioritize optimization on MGL and RF tuning/retuning

RRM framework: Measurement capability/delay/overhead/accuracy/quantity/unified-measurement

- Proposal 11: RAN4 to study feasibility and approaches of unified measurement between cross-layer and feature/function aiming to minimize redundant measurement operations, reduce UE power consumption, alleviate measurement and scheduling restrictions.

- Proposal 12: RAN4 to study to redefine the current intra/inter-frequency measurement requirements structure for improved measurement efficiency such as the definition of intra-frequency or separate serving and neigbhor cell measurements requirements.

- Proposal 13: RAN4 to study UE contextual information based measurement operation to prevent unnecessary measurement operations, reduce power consumption, and improve system efficiency

- Proposal 14: RAN4 to study feasibility and benefit of virtual UE group measurement and reporting to reduce measurement and reporting overhead in 6GR

Mobility related RRM

- Proposal 15: RAN4 to study a cross-layer alignment between L3/RRC-based and L1/L2-triggered mobility procedures to eliminate redundant measurements and define consistent timing and performance requirements for 6G.

- Proposal 16: RAN4 to study a UE-triggered and context-aware RRM mechanisms for seamless and energy-efficient mobility in 6G.

RRM related energy efficiency

- Proposal 17: RAN4 to prioritize UE type/state based RRM relaxation and SSB-less Cell for RRM related energy efficiency, and deprioritize other issues

- Proposal 18: RAN4 to set check-point in 2Q 2026 to discuss whether there are any RAN4 issues for RRM related energy efficiency after sufficient progress in RAN1.

Spectrum aggregation related RRM

- Proposal 19: RAN4 to study lean design for SCell activation/deactivation procedure and delay requirements considering proposed and defined in 5G NR, and improvement of interruptions during the SCell activation procedure considering practical UE behavior and implementation aspects.

- Proposal 20: RAN4 to set check-point to initiate discussion on RRM topics related other WGs such as single cell multi-carrier and DL/UL decoupling.

MIMO and mTRP operation related RRM

- Proposal 21: RAN4 to set check-point in 2Q 2026 to discuss whether there are any RAN4 issues related MIMO after sufficient progress in RAN1.

NTN related RRM

- Proposal 22: RAN4 should first study the RRM impacts of the following issues, which are clearly introduced and required in 6G NTN. Other RRM issues could be further discussed after 2Q 2026 (check-point) according to RAN1’s discussions.

  - TN-NTN mobility under integrated TN-NTN system

  - Both extended GNSS‑resilient scenarios beyond those assumed in 5G and GNSS‑less operation

Initial access related RRM

- Proposal 23: RAN4 to set check-point in 3Q 2026 to discuss whether there are any RAN4 issues related to initial access after sufficient progress in RAN1.

Reference

[1] R4-2514644, “WF on 6G RRM,” Feature lead (Apple)