3GPP TSG-WG4 Meeting #117	R4-2520902

# Dallas, US, 17 – 21 November, 2025

# Title:	TP for PRD 01 on guidelines for near-miss harmonic mixing

# Agenda Item:	5.2.6

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #114 a new set of guidelines on MSD requirements due to near-miss Rx harmonic mixing were presented in [1], and these guidelines were followed in correction CR [2,3,4] and finalized at this meeting in [5]. This TP for TR proposes to capture these guidelines in the RAN4 PRD. We also propose to clarify how to distinguish a direct-hit from a near-miss case for MSD due to harmonic or harmonic mixing interference.

# References

- R4-2501987 On near-miss for Rx harmonic mixing, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc., Anritsu, Qualcomm France, Oppo.

- R4-2511688 [NR_newRAT-Core] CR to TS38.101-1-gn0: Correction to harmonic mixing MSD – CatF, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc., Anritsu, Qualcomm France

- R4-2511689 (NR_CADC_R17_2BDL_xBUL-Core) CR to TS38.101-1-hi0: Correction to harmonic mixing MSD – CatF, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc., Anritsu, Qualcomm France.

- R4-2511690 (NR_CADC_R18_2BDL_xBUL-Core) CR to TS38.101-1-ia0: Correction to harmonic mixing MSD – CatF, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc., Anritsu, Qualcomm France.

- R4-2520221 (NR_CADC_SUL_R19-Core) CR to TS38.101-1-j31: Correction and missing harmonic mixing MSD - Cat-F, 3GPP TSG-RAN WG4 Meeting #117, Dallas, US, Skyworks Solutions, Inc., Anritsu, Qualcomm France.

# Text Proposal

----- Start of TP -----

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aggregated channel bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.

Harmonic direct-hit: Case where there is at least one individual RE within the uplink transmission bandwidth of the (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of an affected (higher) band.

Harmonic near-miss: Only applicable when direct-hit requirements do not apply and only to UL2/DL1 (UL 2nd harmonic/DL fundamental) interference. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

Harmonic mixing direct-hit: Case where one of the DL affected band LO harmonics overlaps with one RE of the UL band fundamental carrier, so called “UL1”, or with one the UL band harmonics, so called “UL2” for 2nd harmonic, “UL3” for 3rd harmonic, etc..

Harmonic mixing near-miss: Only applicable when direct-hit requirements do not apply and only to UL1/DL3 (3rd harmonic of DL band LO) and to UL1/DL5 (5th harmonic of DL band LO) interference. Case where one of the odd order DL band LO harmonics overlaps with the ACLR1 of the UL band fundamental carrier, so called “UL1”.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

----- Unchanged sections are skipped -----

#### 5.5.2.1	Two-band Inter-band Downlink CA

#### 5.5.2.1.1	One Uplink Band

#### 5.5.2.1.1.1	One Uplink CC

----- Unchanged sections are skipped -----

#### Guidelines for MSD Test Points due to near-miss uplink harmonic interference.

- Applicability: For UL harmonic, near-miss MSD cases apply only to UL2/DL1 harmonic interference and only when the band combination does not meet the direct-hit criteria.

- Guideline for near-miss MSD test points:

  - Only one MSD requirement is specified for MSD due near-miss harmonic interference,

  - The MSD is specified with:

    - UL band CBW (lower band): smallest mandatory supported CBW, e.g. 5MHz for FDD bands,

    - UL band LCRB:  DFT-s-OFDM full allocation, e.g. 25RB for 5MHz CBW,

    - The Note 6 of Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1 is modified as follows:

NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

#### Guidelines for MSD Test Points due to near-miss harmonic mixing interference.

- Applicability: For harmonic mixing, near-miss MSD cases apply only to UL1/DL3 and UL1/DL5 interference and only when the band combination does not meet the direct-hit criteria.

- Guideline for near-miss MSD test points:

  - Only one MSD requirement is specified for MSD due near-miss harmonic mixing interference,

  - The MSD is specified:

    - UL band CBW (higher band): smallest CBW that ensures Rx harmonic mixing of the UL1 ACLR1 lower channel, denoted here “ACLR1_low” or harmonic mixing of the UL1 ACLR1 higher channel, denoted here “ACLR1_high”.

    - UL band LCRB: DFT-s-OFDM full allocation, e.g. 50RBs for 10MHz CBW

    - DL band CBW (lower band): smallest CBW.

  - The near-miss MSD requirements are verified with the following test conditions:

    - Case of near-miss harmonic mixing with UL1 ACLR1_low:

    - The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band,

    - Case of near-miss harmonic mixing with UL1 ACLR1_high:

The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band,

----- End of TP -----