3GPP TSG-RAN4 #117	R4-2520918

Dallas, USA, 17th Nov. – 21st Nov. 2025

Title:	Views on 6GR testability and OTA

Source:			Anritsu Corporation

Agenda Item:			8.11

Document for:			Approval

# 1.	Introduction

RAN4 started a discussion on 6GR testability and OTA from RAN4 #116-bis meeting in Prague and the associated way forward [1] was approved. In this contribution, we would like to discuss topic #2 (New test methodologies for new 6GR frequencies) and topic #6 (Harmonized testing for TN and NTN) in the WF especially from the viewpoint of the OTA test chamber.

# 2.	Discussion

# 2.1 Views on topic #2 (New test methodologies for new 6GR frequencies)

The agreement of sub-topic 2-1 in the WF [1] is extracted below for reference.

- Sub-topic 2-1 Testability for new 6GR frequency Issue 2-1-1: Testability for New Frequencies between FR1 and FR2 Agreements: RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered: - The frequency range definition will be discussed under the spectrum agenda. - For conducted test, use existing test method as a starting point - For OTA test - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

2.1.1 General

First, to determine test methodologies for new 6GR frequencies at the range between FR1 and FR2, in addition to a boundary of conducted testing and radiated testing, it is necessary to clarify an expected antenna structure in a UE. In FR1, the UE antenna was assumed to be a single element (omnidirectional), while in FR2, an array antenna (antenna aperture 5 cm) was assumed. An information on the UE antenna's directivity (beamwidth) is necessary for examining the TRP measurement grid width and calculating the measurement uncertainty (MU). Therefore, it is necessary to clarify the antenna structure used for UEs in the new frequency bands between FR1 and FR2.

Observation 1: In FR1, the UE antenna was assumed to be a single element (omnidirectional), while in FR2, an array antenna (antenna aperture 5 cm) was assumed. An information on the UE antenna's directivity (beamwidth) is necessary for examining the TRP measurement grid width and calculating the measurement uncertainty (MU). To determine test methodologies for new 6GR frequencies at the range between FR1 and FR2 (aka FR3), in addition to a boundary of conducted testing and radiated testing, it is necessary to clarify an expected antenna structure in the UE.

Proposal 1: Proponent companies are encouraged to clarify the following antenna structure for devices in between FR1 and FR2 (aka FR3).
- Directivity (Omni-directional or directional antenna)
- Maximum size of the antenna (Minimum beam width) and number of antenna elements if necessary
- Polarization (If there is a need to support the circular polarization)

Also, the group may prefer that the test system can be commonly used with both FR2 and the frequencies between FR1 and FR2 (aka FR3). However, we expect some technical issues to combine the test systems for those frequency ranges, and further studies are necessary in RAN4.

Observation 2: Additional studies are necessary to achieve the test system supporting both FR2 and the frequencies between FR1 and FR2 (aka FR3) in one system.

Expected issues are explained in the following sub-clause.

2.1.2 Technical issues to support both FR2 and frequencies between FR1 and FR2 in one system

Expected technical issues to support both FR2 and frequencies between FR1 and FR2 in one system are as follows.

Observation 3: Technical issues when considering the use of existing FR2 indirect far field (IFF) chamber in the new frequency band between FR1 and FR2 are as follows.

Issue 1) For quality of quiet zone (QoQZ), when comparing reflectors of the same size, performance deteriorates as the test frequency decreases. (The impact of QZ degradation due to scattering at the reflector edges increases as frequency decreases.) Therefore, consideration of measurement uncertainty (MU) relaxation is necessary.

Issue 2) To maintain a constant phase within the QZ, the measurement antenna must be placed at the focal point of the IFF reflector. This requires a directional antenna with a wider bandwidth than the current measurement antenna used for the FR2. If such an antenna cannot be provided, multiple measurement antennas must be switched at the focal position, necessitating a more complex measurement system.

Issue 3) Generally, as the frequency decreases (wavelength increases), the amount of electromagnetic wave absorption by radio wave absorbers diminishes. Consequently, within existing chambers, the use of radio wave absorbers is expected to worsen QoQZ due to increased electromagnetic wave disturbance inside the chamber. Furthermore, if radio wave absorbers are made thicker to match the operating frequency, the effective area within the chamber becomes smaller, potentially requiring a larger chamber.

In addition to the issues above, in a case we need to cover also FR1 in one OTA chamber like mentioned in topic #1 sub-topic 1-2 in the previous WF[1], the issue significantly increases.

Observation 4: The relevance of those issues significantly increases in the case FR1 needs also to be covered in one OTA chamber.

# 2.2 Views on topic #6 (Harmonized testing for TN and NTN)

The agreement of sub-topic 6-1 in the WF [1] is extracted below for reference.

- Sub-topic 6-1 Harmonized OTA test methodologies for TN and NTN Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN Agreements: - Targeting a harmonized test system/test method for TN and NTN devices for FR1. - The test complexity and cost should be considered.

2.2.1 Necessary installing area of hybrid OTA chamber for TN and NTN devices

When we talk about FR1 for NTN, the frequency range FR1-NTN includes the range which is higher than the TN case such as Ku band (10.7 – 14.5 GHz). Since it has an impact on other parameters such as QZ size and polarization of antenna, it is necessary to clarify the understanding of the group again.

Observation 5: FR1-NTN is defined as up to 14.5 GHz.

Proposal 2: Further clarification is necessary if the previous agreement of Issue 6-1-1 in WF [3(R4-2514820)] denotes sub-6GHz or it includes the frequency up to 14.5 GHz.

From a viewpoint of QoQZ in FR1, the existing MIMO OTA chambers for TN devices are assuming 30 cm cylindrical quiet zone in the chamber based on the description of the TR 38.762 [2]. On the other hand, as can be seen in the previous WF for UE OTA test enhancement [3] for Ka/Ku band, the QZ size larger than [63] cm is assumed as the smallest size. To cover both the TN and NTN devices in one chamber, there is a need to develop a larger size of an OTA chamber than the legacy FR1 MIMO OTA chamber, which may lead to a larger installing area (footprint) in a room.

Observation 6: A larger OTA chamber and an installing area are required to cover both TN and NTN devices in FR1 and FR1-NTN.

2.2.2 Polarization of FR1-NTN device

In the WF [3], there is a description of circular polarization for Ka and Ku band devices. Since the existing FR1 OTA chamber is assuming linear polarization antenna, there might be a need to equip the chamber with the cross-polarization antenna and the circular polarization antennae (LHCP and RHCP) if we try to cover both TN and NTN devices in one OTA chamber.

Observation 7: There might be a need to equip the chamber with the linear polarization antenna and the circular polarization antennae (LHCP and RHCP) if we try to cover both TN and NTN devices in one OTA chamber.

- Conclusion

In this contribution, we discussed the 6GR related testability especially from the OTA test chamber viewpoint.

Observation 1: In FR1, the UE antenna was assumed to be a single element (omnidirectional), while in FR2, an array antenna (antenna aperture 5 cm) was assumed. An information on the UE antenna's directivity (beamwidth) is necessary for examining the TRP measurement grid width and calculating the measurement uncertainty (MU). To determine test methodologies for new 6GR frequencies at the range between FR1 and FR2 (aka FR3), in addition to a boundary of conducted testing and radiated testing, it is necessary to clarify an expected antenna structure in the UE.

Proposal 1: Proponent companies are encouraged to clarify the following antenna structure for devices in between FR1 and FR2 (aka FR3).
- Directivity (Omni-directional or directional antenna)
- Maximum size of the antenna (Minimum beam width) and number of antenna elements if necessary
- Polarization (If there is a need to support the circular polarization)

Observation 2: Additional studies are necessary to achieve the test system supporting both FR2 and the frequencies between FR1 and FR2 (aka FR3) in one system.

Observation 3: Technical issues when considering the use of existing FR2 indirect far field (IFF) chamber in the new frequency band between FR1 and FR2 are as follows.

Issue 1) For quality of quiet zone (QoQZ), when comparing reflectors of the same size, performance deteriorates as the test frequency decreases. (The impact of QZ degradation due to scattering at the reflector edges increases as frequency decreases.) Therefore, consideration of measurement uncertainty (MU) relaxation is necessary.

Issue 2) To maintain a constant phase within the QZ, the measurement antenna must be placed at the focal point of the IFF reflector. This requires a directional antenna with a wider bandwidth than the current measurement antenna used for the FR2. If such an antenna cannot be provided, multiple measurement antennas must be switched at the focal position, necessitating a more complex measurement system.

Issue 3) Generally, as the frequency decreases (wavelength increases), the amount of electromagnetic wave absorption by radio wave absorbers diminishes. Consequently, within existing chambers, the use of radio wave absorbers is expected to worsen QoQZ due to increased electromagnetic wave disturbance inside the chamber. Furthermore, if radio wave absorbers are made thicker to match the operating frequency, the effective area within the chamber becomes smaller, potentially requiring a larger chamber.

Observation 4: The relevance of those issues significantly increases in the case FR1 needs also to be covered in one OTA chamber.

Observation 5: FR1-NTN is defined as up to 14.5 GHz.

Proposal 2: Further clarification is necessary if the previous agreement of Issue 6-1-1 in WF [3(R4-2514820)] denotes sub-6GHz or it includes the frequency up to 14.5 GHz.

Observation 6: A larger OTA chamber and an installing area are required to cover both TN and NTN devices in FR1 and FR1-NTN.

Observation 7: There might be a need to equip the chamber with the linear polarization antenna and the circular polarization antennae (LHCP and RHCP) if we try to cover both TN and NTN devices in one OTA chamber.

# 4.	References

[1] R4-2514647, “WF for [116bis][110] 6G testability and OTA”, Vivo (Feature Lead), RAN4 #116-bis, Prague, Czech Republic

[2] TR 38.762, “Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs”, v19.0.0, 2025-09

[3] R4-2514820, “WF on NR_UE_OTA_Enh”, Eutelsat Group, RAN4 #116-bis, Prague, Czech Republic