3GPP TSG-RAN4 #117	R4-2520919

Dallas, USA, 17th Nov. – 21st Nov. 2025

Title:	Views on test method of Initial Access Beam Correspondence in FR2

Source:			Anritsu Corporation

Agenda Item:			11.3

Document for:			Discussion

# 1.	Introduction

RAN5 continued a discussion on the test methodologies for FR2 initial access beam correspondence (IABC), and the LS was sent to RAN4 [1] for clarification of assumptions whether the use of UE beam lock function (UBF) was a mandatory condition when the requirements were derived. Although RAN4 discussed the issue at the RAN4 #116-bis meeting, the group didn’t converge, and the way forward [2] was noted.

In this contribution we report our review results with the previously submitted discussion papers and CRs from a test implementation and testability viewpoint.

# 2.	Discussion

In this contribution, we reviewed two test methodologies proposed in [3] [4] [5] and [6], which propose test methods without UBF or with UBF activation. Note that our review was made under a condition that details of UBF for initial access are not defined at this moment. For our reference, test procedures proposed in [4] and [6] are extracted in the appendix at the end of this contribution.

# 2.1 Views on test methodology without UE beam lock function (UBF) activation

First, we discuss the test methodology without the UBF activation. Although we understand an intention of the proponent company that an associated measurement should be carried out based on an actual field behaviour of the DUT, we observed three concerns by this method. First one is an expectation of a need of huge buffer for a test equipment to store measured samples, second is an implementation of a feature to find the highest average EIRP power after accumulating consecutive PRACH EIRP samples with a sliding window, and the last is a possibility of additional test tolerances due to unstable power measurement results.

2.1.1 Expectation of a need of huge buffer for a test equipment to store measured samples

Based on the previous CR [4], maximum number of preambles defined by the IE preambleTransMax is 200 times. And according to the previous LS from RAN4 [7], the working assumption of test condition when RAN4 defined the PRACH EIRP spherical coverage requirement is that UE shall transmit PRACH with gap <=20ms. Thus, although it depends on a design of a data processing in a test equipment, we expect that the test equipment needs to be equipped with a huge buffer which can store measured EIRP power samples at least for 4000 ms, which is a burden to the test equipment. A further optimization of test conditions would be necessary.

Observation 1: Based on the currently proposed test conditions, although it depends on a design of a test equipment, we expect that the test equipment needs to be equipped with a huge buffer which can store measured EIRP power samples at least for 4000 ms, which is a burden to the test equipment. A further optimization of test conditions would be necessary.

2.1.2 Feature to find the highest average EIRP power over accumulated samples with a sliding window

As mentioned at the test procedure in the CR [4], the TE needs to have a feature to find the highest average EIRP power after accumulating consecutive PRACH EIRP samples with a sliding window, which may add overhead to the TE implementation.

8. Among the collected PRACH transmission from the UE, SS picks the set of X (X = 24 for PRACH format C2) consecutive PRACH providing the highest average PRACH power.

Observation 2: The TE needs to have a feature to find the highest average EIRP power after accumulating consecutive PRACH EIRP samples with a sliding window, which may add overhead to the TE implementation.

2.1.3 Possibility of additional test tolerances due to unstable power measurement results

As already mentioned in [5], we share the similar view that we have a concern that we may obtain unstable power measurement results due to the lack of UBF activation during the test. From that perspective, we expect we may need to study additional test tolerance against the unstable test results.

Observation 3: An additional test tolerance may need to be studied against the unstable test results.

2.2 Views on test methodology with UBF activation

Next, we studied the other test methodology with UBF activation. According to the previously submitted contributions [5][6], we observed that it would be possible for our test system to carry out the tests with a smaller software implementation work. Although this test methodology is our preference from TE implementation perspective, it may become a burden for a UE vendor side since the implementation of UBF for this IABC test is different compared to the existing UBF for other TRx RF tests.

According to the test procedures in [6], a TE sends the UBF request just after connecting a UE, which is before the UE starts sending PRACH and before TE starts the measurement of the PRACH power from UE instead of sending the UBF request after counting the number of PRACH till the defined number. We understand this procedure is taking care of a test with none-standalone (NSA) UE, where there might be a case that test equipment to send LTE control signals (in this case UBF) and receive NR signals (in this case counting PRACH) are separately operating and these two test equipment may not be synchronized. And it allows for test system side to reduce the implementation work for synchronization of two test equipment. However, since UE need to activate the beam lock by itself with a proper timing, it might be a burden for the UE side when implementing the UBF.

Observation 4: Based on the proposed test procedures in [6], the UBF request is sent much earlier than the actual PRACH measurement, which allows for test equipment of LTE and NR to operate without synchronizing when testing NSA UEs and allows reducing the implementation work for TE side.

Observation 5: Although the proposed test procedure in [6] is preferrable from TE implementation perspectives, since the UE need to activate the beam lock by itself with a proper timing, it might be a burden for the UE side when implementing the UBF.

- Conclusion

In this contribution we reported our review results with the previously submitted discussion papers and CRs on the test methodologies for FR2 initial access beam correspondence from a test implementation and testability viewpoint.

Observation 1: Based on the currently proposed test conditions, although it depends on a design of a test equipment, we expect that the test equipment needs to be equipped with a huge buffer which can store measured EIRP power samples at least for 4000 ms, which is a burden to the test equipment. A further optimization of test conditions would be necessary.

Observation 2: The TE needs to have a feature to find the highest average EIRP power after accumulating consecutive PRACH EIRP samples with a sliding window, which may add overhead to the TE implementation.

Observation 3: An additional test tolerance may need to be studied against the unstable test results.

Observation 4: Based on the proposed test procedures in [6], the UBF request is sent much earlier than the actual PRACH measurement, which allows for test equipment of LTE and NR to operate without synchronizing when testing NSA UEs and allows reducing the implementation work for TE side.

Observation 5: Although the proposed test procedure in [6] is preferrable from TE implementation perspectives, since the UE need to activate the beam lock by itself with a proper timing, it might be a burden for the UE side when implementing the UBF.

# 4.	References

[1]	R4-2509022 (R5-253653), “LS on Beam Correspondence Initial Access”, RAN5, RAN4 #116, Bengaluru, India

[2] R4-2515162, “Way Forward for [116bis][324] LS_BDaT -- BC initial access”, Ericsson, RAN4 #116-bis, Prague, Czech Republic

[3] R5-253627, “Discussion paper on FR2 PRACH UE beam correspondence”, Qualcomm Inc., RAN5 #107, St. Julian’s, Malta

[4] R5-252987, “Updates to TC 6.6.3 Beam correspondence for initial access”, Qualcomm Inc., RAN5 #107, St. Julian’s, Malta

[5] R5-254819, “Impact of beam correspondence test method without UE beam lock”, Apple, RAN5 #108, Bengaluru, India

[6] R5-255393, “Updates to TC 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access”, Apple, RAN5 #108, Bengaluru, India

[7] R4-2513625, “FR2 TX requirements verification in beam locked mode”, Apple, RAN4 #116-bis, Prague, Czech Republic

[8] R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, Xiamen, China

# Appendix A: Extract of test procedures in [4] and [6]

Extract of test procedures in [4]: Without UBF activated

        - Test procedure

Without [UE_beamlock_idle]

1.	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-7 [3] to mount the DUT inside the QZ

2.	Position the DUT in DUT Orientation 1 from Tables  N.2-1 through N.2-7 [3].

3.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [10] clause 4.5.

4.	SS switches to SSB transmitting from PolLink = theta.

5.	SS transmits a Paging message (including PagingRecord with ue-Identity) for the UE to trigger PRACH

6.	The UE starts sending PRACH to the SS. In response, SS shall transmit RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

7.	SS shall measure simultaneously the PRACH transmission power of both measurement polarization.

8.	Among the collected PRACH transmission from the UE, SS picks the set of X (X = 24 for PRACH format C2) consecutive PRACH providing the highest average PRACH power.

9. SS determines the EIRP for PolLink = theta by adding up the EIRP of the 2 measurement polarization.

10.	Once SS has completed the PRACH transmission power measurement, SS shall send a valid RAR containing random access preamble identifier that matches the transmitted random access preamble to the UE to ensure it goes to RRC_CONN. If UE fails to go to RRC_CONN, power cycle the UE.

11.	The SS shall transmit RRC release to release the RRC connection to ensure UE is in state RRC_IDLE.

10.Repeat steps 5-11 with TE switching to SSB transmitting from PolLink = phi. This is the EIRP PRACH power for PolLink = phi .

11.SS calculates UE PRACH EIRP for that grid point as a result of  EIRP (PolLink = Phi) + EIRP (PolLink = Theta).

12.	Advance to the next grid point and repeat steps 4 through 11 until measurements within zenith range 0o≤≤90o  have been completed.

13.	After the measurements within zenith range 0o≤≤90o have been completed and

a) if the re-positioning concept is applied to the TX test cases, position the device in DUT Orientation 2 (either Options 1 or 2) from Tables N.2-1 through N.2-7 [3] for the Alignment Option selected in Step 1. For the TX beam peak search in the second hemisphere, perform steps 3 through 12 for the range of zenith angles 90o>q≥0o.

b) if the re-positioning concept is not applied to the TX test cases, continue steps 3 through 12 for the range of zenith angles 90o<≤180o

14.	Calculate the CDF for all grid points and pass the UE if the derived %-tile EIRP in measurement distribution exceeds the test requirement. Otherwise fail the UE.

Extract of test procedures in [6]: With UBF activated

## K.1.5a	EIRP spherical coverage for IDLE mode

In this EIRP spherical coverage for IDLE mode procedure, the minimum number of grid points defined in Annex M.3.1.1 for spherical coverage is used.

The measurement procedure includes the following steps:

- Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-7 [3] to mount the DUT inside the QZ.

- Position the DUT in DUT Orientation 1 from Tables N.2-1 through N.2-7 [3].

- Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

- The SS activates the UE Beamlock_IDLE Function (UBF_IDLE) by performing the procedure as specified in TS 38.509 [10] clause 5.4a.2.

- The SS shall initiate the RRCRelease procedure to transit a UE from RRC_CONNECTED to RRC_IDLE state.

- The SS does not transmit Random Access Responses. The network initiates the paging procedure by transmitting the Paging message.

- The UE shall send the PRACH preamble to the SS. The UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

- The UE shall lock the UE Tx antenna pattern starting at the latest from [n100] of the PreambleTransMax counter.

- The SS uses 10 UL slots of PRACH for a 1ms measurement period from [n101] to [n110] of the PreambleTransMax counter.

- EIRP is calculated considering both polarizations, theta and phi. Calculate total EIRP(PolLink=) = EIRP(PolMeas= PolLink=)  + EIRP(PolMeas= PolLink=).

- The SS shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble at the n200 PreambleTransMax counter.

- The SS shall ensure the UE is in RRC_CONNECTED state.

- The SS deactivates the UE Beamlock_IDLE Function (UBF_IDLE) by performing the procedure as specified in TS 38.509 [10] clause 5.4a.3.

- Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

- Repeat steps 4 through 13 and get the result of total EIRP(PolLink=) = EIRP (PolMeas= PolLink=) + EIRP (PolMeas= PolLink=)

- Advance to the next grid point and repeat steps 3 through 15 until measurements within zenith range 0o≤θ≤90o have been completed.

- After the measurements within zenith range 0o≤θ≤90o have been completed and

- if the re-positioning concept is applied, position the device in DUT Orientation 2 (either Options 1 or 2) from Tables N.2-1 through N.2-7 [3] for the Alignment Option selected in Step 1and perform steps 3 through 14 for the range of zenith angles 90o>≥0o

- if the re-positioning concept is not applied, continue steps 3 through 16 for the range of zenith angles 90o<≤180o

- The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maximum(EIRP(PolLink=), EIRP(PolLink=)) for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1, to account for the denser grid point distribution near the poles. In case of Clenshaw-Curtis weights, when just a single measurement at the poles is performed, the PDF probability contributions need to be scaled by M*W()/W(=90°) to account for the M longitudes at those two grid points. When using constant density grids, these corrections are not needed.

The Fast Spherical Coverage Method specified in K.1.5.1 could be applied.