3GPP TSG-RAN WG4 Meeting # 117	R4-2520934

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	4.8.2

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides analysis on the maintenance issues in core part for low band carrier aggregation via switching from RRM perspective.

Discussion

SDL SCell activation/deactivation in LB CA

As we know, an interruption window is defined for the legacy SCell activation procedure (duplicated as below). The candidate interruption location on PCell due to RF retuning to receive SSB on SCell is between after HARQ and before the first SSB.

The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is: - 0, if Tactivation_time is 3 ms; - TFirstSSB, for any scenario where Tactivation_time includes TFirstSSB; - TFirstSSB_MAX, for any scenario where Tactivation_time includes TFirstSSB_MAX; - Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time includes only TFineTiming and no TFirstSSB_MAX; - Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time includes Tfirst_TRS; - Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time includes Tfirst_ATRS.

In LB-CA scenario, for the case as presented in Figure 1 (SCell activation MAC CE is received during FDD PCell duration) the instance RF retuning to switch to SDL fixedly happens on “switch from” carrier. As per RAN4 agreement, RAN4 not to define interruption/scheduling restriction due to the SDL/FDD switching.

For the case as presented in Figure 2 (SCell activation MAC CE is received during SDL duration), UE can retune RF chain to receive SSB on SDL SCell after slot Tharq+3ms and before the first SSB on SCell. It shall be noted that there is no interruption on PCell, as there is no data on PCell at this moment.

Based on the above analysis, the current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario. There is no interruption during SCell activation in LB-CA.

Figure 1. SCell activation MAC CE is received during FDD PCell duration

Figure 2. SCell activation MAC CE is received during SDL duration

Proposal 1: Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario.

Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation

The legacy, timer based SCell activation delay is 3msNR slot length. Although sCellDeactivationTimer is maintained in MAC entity, there is no MAC CE decoding like MAC CE based SCell deactivation. So there is a time margin for timer based SCell deactivation in legacy requirements.

- 8.3.3 SCell Deactivation Delay Requirement for Activated SCell <other parts are skipped> Upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + 3msNR slot length. The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1 and not occur after slot n+1+ 3msNR slot length, where NR slot length is with respect to the numerology used in the SCell being deactivated. The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

In R19 LB-CA, some companies think UE is supposed to stop and re-load the non-switching pattern upon the SDL SCell is deactivated, and then additional processing time (1ms) is needed. Although we think the legacy SCell deactivation delay has sufficient margin to accommodate the additional time, it is also fine to have it, as the timeline of SCell deactivation is not urgent, the SCell is not scheduled after being deactivated.

Proposal 2: It is fine that the switching pattern is NOT applied after n+ ((3ms+ TLBCA)/slot-length) for timer based SCell deactivation where TLBCA =1 ms.

Issue 1-1-4b: switching pattern based carrier switching in SCell activation Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

For timer based SCell deactivation procedure in R19 LB-CA, we don’t observe the reason to introduce a different UE behaviour with the MAC CE based SCell deactivation.

Proposal 3: If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((3ms+ TLBCA)/slot-length) for timer based SCell deactivation. The carrier switching, if applicable, shall be placed at the end of a slot.

In last meeting, there is an open issue about the impact of C-DRX on LBCA measurement. One proposal is that switching pattern is not applied when UE is in DRX and to remove “the impact of switching pattern for Kp factor for measurements during DRX”.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

First, we review the above legacy measurement requirement. Kp is introduced to consider the impact of measurement gap on available SMTC occasions. When DRX is used, UE is only required to measure one available SMTC occasion during one DRX cycle. If DRX is large, there may be multiple available SMTC occasions even if some SMTCs are colliding with gap during one DRX cycle. In this case, in theory Kp is not necessary to be multiplied with DRX in legacy requirements.

In R19 LB-CA, we understand the logic of the proposal by some company that as switching gap can be regarded as a kind of “gap”, likewise the measurement requirement may not be further relaxed by multiplied by Kp in DRX under certain configuration. Since the legacy requirements has not been optimized, we prefer the related requirements in R19 LB-CA don’t need to be optimized either.

In addition, whether UE applies switching pattern during DRX OFF can leave to UE implementation.

Proposal 4: Prefer not to introduce the optimization of removing Kp factor for measurement during DRX.

BWP switching requirement for LB CA

One issue is whether specify the starting point/first slot of executing BWP switching for the LB CA case. The scenario discussed is that active BWP switching is triggered on PCell/SDL, but the BWP switching processing time is longer than the residual time on the current cell ON time according to the switching pattern. There are multiple proposals for resolving the issue. However in our understanding, this is not a typical case, as network has information of BWP switching processing time and BWP switching is triggered by network. In other words, network can avoid such configuration.

Proposal 5: RAN4 not to consider starting point/first slot of executing BWP switching for the LB CA case where the BWP switching processing time is longer than the residual time on the current cell ON time.

Others

One issue is whether CSSF inside gap shall additionally consider LB-CA switching pattern on top of SMTC and MG.

As per the existing requirement, CSSFwithingap is applied in the below categories of intra-frequency MO.  Otherwise CSSFoutsidegap is applied, if SMTC occasions are partially overlapping with gap.

- SSB-based intra-frequency measurement object without measurement gap as defined in clauses 9.2.1 and 9.2A.1, when - all of the SMTC occasions of this intra-frequency measurement object are overlapped with the associated measurement gap in concurrent GAPs, or - part of the SMTC occasions of this intra-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this intra-frequency measurement object are overlapped with the union of concurrent GAPs. - part of the SMTC occasions of this intra-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this intra-frequency measurement object are overlapped with the union of concurrent GAPs or with the union of concurrent concurrent GAPs and MUSIM gaps if MUSIM gaps are configured.

In R19 LB-CA, the SMTC occasions which are partially overlapping with GAP may fully collide with switching pattern. In this case, intra-frequency measurement requirement shall scale with CSSFwithingap rather than CSSFoutsidegap. Therefore RAN4 needs to revisit the CSSFwithingap by considering SMTCs, MGs and LB-CA switching pattern.

Proposal 6: RAN4 to revisit the CSSFwithingap by considering SMTCs, MGs and LB-CA switching pattern.

Conclusions

This contribution provides further analysis on low band carrier aggregation via switching from RRM perspective. The following proposals are provided:

Proposal 1: Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario.

Proposal 2: It is fine that the switching pattern is NOT applied after n+ ((3ms+ TLBCA)/slot-length) for timer based SCell deactivation where TLBCA =1 ms.

Proposal 3: If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((3ms+ TLBCA)/slot-length) for timer based SCell deactivation. The carrier switching, if applicable, shall be placed at the end of a slot.

Proposal 4: Prefer not to introduce the optimization of removing Kp factor for measurement during DRX.

Proposal 5: RAN4 not to consider starting point/first slot of executing BWP switching for the LB CA case where the BWP switching processing time is longer than the residual time on the current cell ON time.

Proposal 6: RAN4 to revisit the CSSFwithingap by considering SMTCs, MGs and LB-CA switching pattern.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- R4-2514794, WF for [116bis][202] NR_LBCA_Sw_RRM, Apple