3GPP TSG-RAN WG4 Meeting # 117	R4-2520938

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on enhancement of low NR band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.8.4

Document for:	Discussion

Introduction

R20 WI on enhanced low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

The contribution provides further analysis on enhanced low band carrier aggregation via switching from RRM perspective.

Discussion

Background

R20 LB-CA WI is an enhancement of R19 LB-CA especially for the case of switching between FDD and SDL bands with enabling dual downlink.

The R20 LB-CA allow switching between {case 1, case 3} which enables low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band:

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

RANP has made conclusion that semi-static RRC configured switching pattern defined in Rel-19 LB-CA is reused in R20.

Figure 1. case 1 and case 3

Figure 2. Switching in R20 enhanced LB-CA

It is observed in R20 enhanced LB-CA via switching:

- DL reception on FDD carrier is not confined to switching pattern. UE can perform DL reception on PCC during both FDD ON duration and “dual SDL+FDD DL” ON duration in accordance with switching pattern

- UL transmission on FDD carrier is confined to the switching pattern

- DL reception on SDL carrier is confined to the switching pattern

Note: In switching pattern, FDD UL ON duration responds to SDL DL OFF duration, vice versa. UE is not able to simultaneously receive on SDL and transmit on FDD UL.

General aspects

For R19 LB-CA, RAN4 only defines requirements for CA with FDD PCC and one SDL SCC. The same principle can be reused for R20 LB-CA.

Proposal 1: RAN4 only defines requirements for CA with FDD PCC and one SDL SCC for R20 eLB-CA.

For R19 LB-CA, it is agreed that the maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us [2]. The same principle can be reused for R20 LB-CA.

Proposal 2: Similar to R19 LB-CA, the maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us for R20 eLB-CA.

PCC related RRM requirements

As illustrated in Figure 2, DL reception on FDD carrier is not confined to switching pattern. UE can perform DL reception on PCC during both FDD ON duration and “dual SDL+FDD DL” ON duration in accordance with switching pattern. This is quite different with the R19 LB-CA where PCell DL reception depends on PCell ON duration. Therefore the below requirements on PCell defined for R19 LB-CA are not applicable for PCell in R20 LB-CA:

- Intra-frequency L3 measurement,

- L1 RSRP/SINR measurement,

- RLM/BFD/CBD evaluation,

- TCI state switching,

- PL-RS switch delay

Proposal 3: The below requirements on PCell defined for R19 LB-CA are not applicable for PCell in R20 eLB-CA:

- Intra-frequency L3 measurement,

- L1 RSRP/SINR measurement,

- RLM/BFD/CBD evaluation,

- TCI state switching,

- PL-RS switch delay

It shall be noted that RF will discuss the switch period between case 1 and case 3. During the switch period/gap, UE does not transmit and receive any signals. If SSB/CSI-RS on PCC collides with the switching period/gap, the L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC would be affected. From specifying requirements perspective, it can clarify that the available RS is the RS not colliding with switching period, or it can simply to say that the corresponding delay would be extended if the to-be measured RS is colliding with switching period.

Proposal 4: If to-be-measured SSB/CSI-RS on PCell collides with the switching gap, the above requirements on PCell would be affected. RAN4 to discuss how to clarify the above requirements on PCell, e.g., clarify that the available RS is the RS not colliding with switching gap, or simply to clarify the delay will be extended if the to-be-measured RS is colliding with switching gap.

Interruption due to L3 measurement on deactivated SDL

In last meeting, RAN4 agreed that switching pattern is not applicable when SDL SCell is deactivated in R20 eLB-CA.

Figure 3. DL and UL interruptions due to measurement on deactivated SCell in R20 eLB-CA

In R20 enhanced low band switching scenario, UE is not able to simultaneously receive on SDL and transmit on FDD UL. In order to measure on deactivated SDL, UE shall switch to deactivated SDL carrier and switch back to PCC after L3 measurement. As UE can support dual DL reception on both PCell and SDL, there is no interruption on PCell DL during UE performing measurement on deactivated SCell, while the interruption only occurs at “switch from” and “switch to” instances. Therefore the interruption length on PCell DL is 2x retuning time (switch to and back).

UL interruption length on PCell is different with the DL interruption length. As UE is not able to support simultaneous DL measurement on SDL and UL transmission on PCell, the interruption length on PCell UL shall comprise 2 x retuning time (switch to and back) and the time duration staying on SDL carrier (i.e., SMTC duration).

Proposal 5: When UE performs L3 measurement on deactivated SDL SCell:

- the DL interruption length on PCell is 2x retuning time

- the UL interruption length on PCell is (2 x retuning time + SMTC duration)

Regarding the interruption percentage, the loss of ACK/NACK may be caused by the interruption of PDCCH and the interruption of the UL transmission which carrying ACK/NACK. To be precise, the ACK/NACK loss on PCell would be twice the interruption length if both PCell UL and DL are interrupted. The ACK/NACK loss on PCell would be equal to the interruption length if only PCell UL is interrupted. Therefore in R20 eLB-CA, the missed ACK/NACK percentage shall be represented as X=2xA+BmeasCycleSCell  where A = 2x retuning time (switch to and back), B= the time duration staying on SDL carrier (i.e., SMTC duration).

Proposal 6: In R20 eLB-CA, the missed ACK/NACK percentage shall be represented as X=2xA+BmeasCycleSCell ,where A = 2x retuning time (switch to and back), B= SMTC duration (i.e., time duration staying on SDL carrier)

SDL SCell related requirements

SDL SCell activation

It is agreed in RAN4#116 bis meeting that switching pattern is applied to SCell activation procedure in R20 eLB-CA.

In R20 eLB-CA, as the available SDL SCell reference signals to be measured for SCell activation are also in accordance with the SDL SCell active period in switching pattern, so in general the SCell activation delay defined in R19 LB-CA can be reused. It shall be noted that, RF is discussing the RF chain structure (single RF chain or separate RF chain) to support dual DL. The component TSMTC_MAX in the SCell activation delay may need more discussion.

TSMTC_MAX: - In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

In addition, whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed, and we think it is just a wording clarification.

Proposal 7: In general, SCell activation delay defined in R19 LB-CA can be reused for R20 eLB-CA, as the SCell activation procedure is accordance with switching pattern. The below aspects can be further discussed:

-whether to update TSMTC_MAX which can wait for RF progress

-whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed (wording clarification).

BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

It is straight forward that the switching pattern is applicable to activated carriers. In R20 eLB-CA via switching, the BFD/CBD, TCI state switch delay, PL-RS switch, L3-RSRP/L1-RSRP/SINR measurement on SDL are in accordance with the SDL SCell active period based on switching pattern. This is quite similar as R19 LB-CA. Therefore the BFD/CBD evaluation period, TCI state switch delay, PL-RS switch delay, L3-RSRP/L1-RSRP/SINR measurement period defined in R19 LB-CA can be reused.

For R19 LB-CA, there are some descriptions like “overlapped with SCell ON duration”. As in R20 eLB-CA, dual DL reception is supported, whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed, and we think it is just a wording clarification.

Noutside_MG is the number of BFD-RS resource occasions within the window W, - that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, and - that are overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the BFD on PCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, - that are overlapped with SCell ON duration corresponding to the LB CA switching pattern, for the BFD on SCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, and

Proposal 8: The BFD/CBD evaluation period, TCI state switch delay, PL-RS switch delay, L3-RSRP/L1-RSRP/SINR measurement period defined in R19 LB-CA can be reused in R20 eLB-CA. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed.

Interruption requirement in RRM between FDD CC and SDL CC

In R19 LB-CA, as per the RAN1 agreement@ RAN1#121 meeting, RAN1 specify multiple kinds of collision handling. So RAN4 agree that for SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier [3].

RAN1 Agreement For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

In R20 eLB-CA, as UE supports dual DL reception, some of the above agreements specified for R19 LB-CA are not appliable for R20 anymore. We are open whether RAN4 shall trigger an LS to RAN1 to request RAN1 to define corresponding scheduling restriction or RAN4 define scheduling restriction.

Proposal 9: For SDL to FDD switching and FDD to SDL switching, RAN4 to discuss whether trigger an LS to request RAN1 to define corresponding scheduling restriction or RAN4 define scheduling restriction.

Conclusions

This contribution provides further analysis on enhanced low band carrier aggregation via switching from RRM perspective. The following proposals are provided:

General aspects

Proposal 1: RAN4 only defines requirements for CA with FDD PCC and one SDL SCC for R20 eLB-CA.

Proposal 2: Similar to R19 LB-CA, the maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us for R20 eLB-CA.

PCC related RRM requirements

Proposal 3: The below requirements on PCell defined for R19 LB-CA are not applicable for PCell in R20 eLB-CA:

- Intra-frequency L3 measurement,

- L1 RSRP/SINR measurement,

- RLM/BFD/CBD evaluation,

- TCI state switching,

- PL-RS switch delay

Proposal 4: If to-be-measured SSB/CSI-RS on PCell collides with the switching gap, the above requirements on PCell would be affected. RAN4 to discuss how to clarify the above requirements on PCell, e.g., clarify that the available RS is the RS not colliding with switching gap, or simply to clarify the delay will be extended if the to-be-measured RS is colliding with switching gap.

Interruption due to L3 measurement on deactivated SDL

Proposal 5: When UE performs L3 measurement on deactivated SDL SCell:

- the DL interruption length on PCell is 2x retuning time

- the UL interruption length on PCell is (2 x retuning time + SMTC duration)

Proposal 6: In R20 eLB-CA, the missed ACK/NACK percentage shall be represented as X=2xA+BmeasCycleSCell ,where A = 2x retuning time (switch to and back), B= SMTC duration (i.e., time duration staying on SDL carrier)

SDL SCell related requirements

Proposal 7: In general, SCell activation delay defined in R19 LB-CA can be reused for R20 eLB-CA, as the SCell activation procedure is accordance with switching pattern. The below aspects can be further discussed:

-whether to update TSMTC_MAX which can wait for RF progress

-whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed (wording clarification).

Proposal 8: The BFD/CBD evaluation period, TCI state switch delay, PL-RS switch delay, L3-RSRP/L1-RSRP/SINR measurement period defined in R19 LB-CA can be reused in R20 eLB-CA. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed.

Interruption requirement in RRM between FDD CC and SDL CC

Proposal 9: For SDL to FDD switching and FDD to SDL switching, RAN4 to discuss whether trigger an LS to request RAN1 to define corresponding scheduling restriction or RAN4 define scheduling restriction.

Reference

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching

- R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2512119, WF on RRM requirements for NR_LBCA_Sw, Apple