3GPP TSG-RAN WG4 Meeting # 117  		                                             R4-2520942

Dallas, US, 17th November – 21st November, 2025

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	4.15.1

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during the last meeting. Key contributions from companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

Optimising Rx BSF

In the last meeting, there is several remaining issue on the BSF core requirement. The following content is captured form the WF[1].

<Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements.

This issue has been discussed online, and still no further agreement on this issue. The agreement in #116 meeting is:

Agreement in the #116 meeting: No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

In our view, the T2 is defined for the UE behaviour when the condition is still matched, the UE needs to do the re-evaluation and trigger the FSB again. If this behaviour indicates the scenario that the UE will stay at the edge of the Cell, for instance, the re-evaluation to trigger the L3 fast Rx beam sweeping operation may not be necessary.

Observation 1: When the condition is maintained and matched, the UE may not need to be forced to re-trigger the L3 fast Rx beam sweeping operation. Because of the low mobility, and once valid fast measurement result.

Based on this understanding, option 2.2: Not to define T2 for re-evaluation after the UE exits FSB mode and no need to update the core requirement, is workable.

Proposal 1: Not to define T2 for re-evaluation after the UE exits FSB mode and no need to update the core requirement.

In the last meeting, there is one remaining issue about the BSF MO selection when multiple carriers are configured for BFS and reporting. One option captured in the WF [1] is captured in the following:

<Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

This issue discusses how the FBS mode operates on which carrier when the target carrier is different from the serving carrier. The scope of FBS is only one target cell and one serving Cell. The FSB mode can only apply to the serving cell or target cell. To simplify this solution, the pathway is to let the FSB apply to the target carrier only.

Proposal 2: When the target and serving carrier frequency are different, FBS applies to target carrier only.

Optimising CSSF outside gap

In the last meeting, several remaining issues have been maintained for Solution 3. There are two more points we would like to provide our views on.

Solution 3

Issue of inter-frequency measurement in SA mode FR1+FR2 scenario

In the last meeting, we provided our view that the inter-frequency MOs without a measurement gap apply for solution 3 on FR1 only cases, for instance, FR1 only CA and FR1 EN-DC. The agreement has been captured from the WF[1] as below.

Issue 1-2-1: Inter-frequency MOs without a measurement gap supported by CSSF solution 3 <Agreement>: - Apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap, as listed in CR: R4-2513478.

In this paper, we provide our views on further maintaining the 3-searcher solution core requirement. To apply the inter-frequency MOs with a relaxed CSSF value in FR1 only scenario is because there will be two searchers allowed to process all the MOs except the PCC’s. With the relaxed CSSF value for MOs without a measurement gap can guarantee that there will be only one searcher to process the MOs that are without a measurement gap, i.e. inter-frequency and inter-RAT.  This is aimed to be compatible with Gap enhancement from Release 16 to 18.

However, for the FR1+FR2 (PCell in FR1) SA mode scenario, the relaxation for the CSSF value is not needed. In the FR1 + FR2 cases, the searcher allocation of the 3-searcher solution is one for PCC, one for SCC with neighbour cell measurement and one for the remaining MOs. Here is the CSSF table for 3-searcher SA mode:

Table 9.1.5.1.2-2: Enhanced CSSFoutside_gap,i scaling factor for SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap; CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap. FR1 +FR2 CA (FR1 PCell) Note 1, Note 12; 1+NPCC_CSIRS; NSCC_SSB+2*(Y+Z) +2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM; N/A; 1+ NSCC_CSIRS_FR2_NCM Note 3,5; NSCC_SSB+Y+Z +2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM; NSCC_SSB+2*(Y+Z)+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM; 2 x ( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 12: For UE supporting FR1 only CA and FR1 only NR-DC 3-searcher capability or FR1+FR2 CA (PCell is FR1 only) 3-searcher capability in SA operation mode, the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG that are being measured outside of MG.

It is straightforward to notice that the searcher allocation on 3-searcher after we height the corresponding MOs sharing rules in the above table. Therefore, the MOs without the measurement gap will be processed on one of 3 searchers. The gap improvement from release 16 to release 18 is naturally supported.

Observation 2: The gap improvement from release 16 to release 18 is naturally supported in the solution 3 scenario FR1 +FR2 CA (FR1 PCell).

Due to this, the doubling of (Y+Z) in the CSSFoutside_gap,i for inter-frequency MO with no measurement gap is unnecessary, this is highlighted in the above table. So, our proposal is to keep it without doubling, and details can be found in our CR: R4-2520954.

Proposal 3: Do not double the value of (Y+Z) in FR1 +FR2 CA (FR1 PCell) scenario of solution 3 SA mode for inter-frequency MO with no measurement gap CSSF. This can be discussed in the CR R4-2520954.

- 3. Conclusions

Observation 1: When the condition is maintained and matched, the UE may not need to be forced to re-trigger the L3 fast Rx beam sweeping operation. Because of the low mobility, and once valid fast measurement result.

Proposal 1: Not to define T2 for re-evaluation after the UE exits FSB mode and no need to update the core requirement.

Proposal 2: When the target and serving carrier frequency are different, FBS applies to target carrier only.

Observation 2: The gap improvement from release 16 to release 18 is naturally supported in the solution 3 scenario FR1 +FR2 CA (FR1 PCell).

Proposal 3: Do not double the value of (Y+Z) in FR1 +FR2 CA (FR1 PCell) scenario of solution 3 SA mode for inter-frequency MO with no measurement gap CSSF. This can be discussed in the CR R4-2520954.

- 4. References

- R4-2514800 WF on RRM requirements for NR_RRM_Ph5_Part1