3GPP TSG-RAN WG4 Meeting #117	R4-2520943

Dallas, US, 17th November – 21st November, 2025

Agenda item:	6.10.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of optimising Rx beam sweeping factor

Document for:	Discussion

- Introduction

The enhancement on FR2-1 L3 measurement by optimising the Rx beam sweeping factor core part was closed in the last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on the performance part.

- Discussion

L3 measurement enhancement by optimising Rx BSF

The way forward from the last meeting is captured from WF[1] and shown below.

Issue 1-1-1: Whether to test FBS quitting - Recommended WF - Check if option 1 is agreeable? - Option 1: Not to test FBS quitting. Issue 1-1-2: Design of TCs for FBS - Recommended WF - Test cases for FBS are defined by following the framework of legacy TCs for SSB based L3 measurements, i.e., Event A3 triggered measurement report used. - In the TCs for FBS, use two different power levels of the serving cell to trigger FBS. - RAN4 to discuss: - The length of test cases after FBS condition is met: - Alt 1: 90 s; - Alt 2: Other values. - Whether to test re-evaluation of FBS condition after T1 with FBS condition is still met. For information: - Test cases for FBS are defined by referring to the framework of legacy TCs for SSB based L3 measurements, i.e., Event A3 triggered measurement report used.

During the last meeting, the key requirement regarding T1 has reached a consensus, with a fixed value of 90 seconds agreed upon.

Issue 1-1-1: Value for T1 Agreement: - A fixed value of 90 seconds.

After defining the core requirement, it is necessary to test whether the UE will exit fast BSF measurement after a duration of T1 when the BSF conditions are met.

Proposal 1: Define TC to test whether the UE will exit fast BSF measurement after a duration of T1 when the BSF conditions are met.

  - The length of test cases after the FBS condition is met can be set to 90 seconds.

In the last meeting, about when the UE will re-evaluate after the duration of T1 and the BSF condition is still met, RAN4 has no consensus reached. If the re-evaluation is up to the UE implementation, it is necessary to define a TC to test when or whether the UE can re-evaluate the condition. If no core requirement and no TC to verify the re-evacuation mechanism, the fast BSF measurement might be a one-time action when the condition is met to enter the FBS mode.

Observation 1: If no core requirement and no TC to verify the re-evacuation mechanism, the fast BSF measurement might be a one-time action when the condition is met to enter the FBS mode.

During the last meeting with some offline discussion, the scenario that after T1, the FSB condition is still met may not necessarily trigger the fast beam sweep again. But, if RAN4 will update the core requirement, the test case for testing re-evaluation when the FBS condition after T1 is still met is needed. If not, do not define the test case for it.

Proposal 2: This issue is supposed to be based on the core part discussion. If the core part has been updated, introducing one more test case for testing re-evaluation when the FBS condition after T1 is still met is necessary.

- Conclusions

Proposal 1: Define TC to test whether the UE will exit fast BSF measurement after a duration of T1 when the BSF conditions are met.

  - The length of test cases after the FBS condition is met can be set to 90 seconds.

Observation 1: If no core requirement and no TC to verify the re-evacuation mechanism, the fast BSF measurement might be a one-time action when the condition is met to enter the FBS mode.

Proposal 2: This issue is supposed to be based on the core part discussion. If the core part has been updated, introducing one more test case for testing re-evaluation when the FBS condition after T1 is still met is necessary.

4. References

[1] R4-2514801 WF on NR_RRM_Ph5_Part2