3GPP TSG-RAN WG4 Meeting #117	R4-2520950

Dallas, US, 17th November – 21st November, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM performance requirements for Rel-19 ATG

Agenda Item:	6.7.3

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The TU schedule of #114bis shows that the performance part for Rel-19 ATG is starting. This paper presents our observations and views on RRM impacts of performance parts on ATG CA.

RRM performance requirement analysis

In this part, we provide our analysis of the performance requirements to support CA in case-by-case manner in the following part.

Test scope

In the last meeting, the companies provided their views on which test cases the Rel-19 ATG is needed for. There are several remaining issue on the test case details. The following is the summary captured from WF [1].

Issue 2-1-1: Test scope - separate tests for neighbour cell skipping/not skipping Agreement: - Introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping) - FFS whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms or 160ms SCell measurement cycle (with signalling for neighbour cell skipping)

The open issue with neighbour cell skipping testing is how to test the functionality of the skipped or not of neighbour cell. To test the SCell neighbour Cell skipping, there are two approaches to achieve this. On the one hand, when configuring the MO on the neighbour CC, if the UE measured this neighbour Cell with this MO, there will be a valid measurement result in the report. On the other hand, to configure Event A4 (Neighbour cell becomes higher than the threshold) to test whether or not the neighbour Cell of the serving SCell has been measured. If the neighbour Cell has been measured, the A4 event will be triggered and reported.

According to the test load and functionality, using the measurement report to decide whether the test has passed is not straightforward to assess. Consequently, we would like to propose the test design for SCell neighbour Cell measurement skipping by configuring Event A4 and the neighSCellMeasSkipping signalling. The test requirement is simply to verify whether the UE reports an A4 event during the test.

Observation 1: The Event 4 triggering report method is more straightforward than the measurement report analysis.

Proposal 1: Introduce the Event A4 and the neighSCellMeasSkipping signalling for the test case of Event-triggered reporting on SCC with an deactivated SCell to test the functionality of neighbour SCell skipping.

Conclusion

Observation 1: The Event 4 triggering report method is more straightforward than the measurement report analysis.

Proposal 1: Introduce the Event A4 and the neighSCellMeasSkipping signalling for the test case of Event-triggered reporting on SCC with an deactivated SCell to test the functionality of neighbour SCell skipping.

References

[1] R4-2514795 WF on RRM requirements for NR_ATG_enh