3GPP TSG-RAN4 Meeting #117	R4-2520959

# Dallas, US, 17th November – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on the test cases of uplink transmission timing 2TA for scenario #2 SA and EN-DC mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case of asymmetric DL sTRP/UL mTRP for scenario#2 is missing in the RRM spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the TC for the asymmetric DL sTRP/UL mTRP scenario#2 SA and EN-DC mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements can not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.4.1.x, A.5.4.1.x, A.4.4.1.x, A.6.4.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7.4.1.x	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting single DCI

A.7.4.1.x.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits, for UE not configured PL offset and is configured with 2 TAGs for

single-DCI multi-TRP operation. UE is also configured with dl-OrJointTCI-StateList. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in table A.7.4.1.x.1-1.

Table A.7.4.1.x.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

For this test a single NR cell is used. Tables A.7.4.1.x.1-2 and A.7.4.1.x.1-2A define the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in table A.7.4.1.x.1-3.

Table A.7.4.1.x.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  | Test2 |  |  |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP #1 | TRP #2 |  | TRP #1 |  | TRP #2 |  |  |
| SSB ARFCN |  | 1,2 | Freq1 |  |  |  |  |  |  |  |
| Duplex Mode |  | 1,2 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | 1,2 | TDDConf.3.1 |  |  |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 100: NPRB,c = 66 |  |  |  |  |  |  |  |
| Data PRBs allocated |  | 1,2 | 66 |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD TRS.2.2 TDD |  |  |  |  |  |  |  |
| DRX Cycle | ms | 1,2 | N/A |  |  | DRX.8Note5 |  |  |  |  |
| PDSCH Reference measurement channel |  | 1,2 | SR.3. 3 TDD |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.3.2 TDD |  |  |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2 | 0 | 1 |  | 0 |  | 1 |  |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.3.4 TDD | CCR.3.6 TDD |  | CCR.3.4 TDD |  | CCR.3.6 TDD |  |  |
| TCI configuration |  | 1,2 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 |  | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 |  | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 |  |  |
| Timing difference compared to TRP#1 | us | 1,2 | 0 | 0.1 |  | 0 |  | 0.1 |  |  |
| OCNG Patterns |  | 1,2 | OP.1 |  |  |  |  |  |  |  |
| SSB Configuration |  | 1,2 | SSB.1 FR2 |  |  |  | SSB.1 FR2 |  |  |  |
| SSB index |  | 1,2 | 0 |  | 1 |  | 0 |  | 1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |  |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 |  |  | SRSConf.2Note6 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: DRX related parameters are given in table A.3.3.8-1 NOTE 6: SRS configs are given in table A.7.4.1.x.1-3 |  |  |  |  |  |  |  |  |  |  |

Table A.7.4.1.x.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 3 as specified in clause A.3.15 Note 7 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15 kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -100 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -96 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 7: AoA1 for TRP1 and AoA2 for TRP2 |  |  |  |

Table A.7.4.1.x.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 |  | SRSConf.2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 1 | 0 | 1 |  |
|  | srs-ResourceIdList | 0 | 1 | 0 | 1 |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 1 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | 17 | 17 | Matches NPRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl2, 0 | sl2, 1 | sl2560, 4 | sl2560, 9 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | 0 | 0 | Any 10 bit number |
|  | TCI state | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 |  |

Table A.7.4.1.x.1-4: Void

A.7.4.1.x.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test:

1)	Setup NR PCell according to parameters given in table A.7.4.1.x.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected corresponding path of DL SSB (TRP#1) for each TAG and detected another path of DL SSB (TRP#2).

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in table A.7.4.1.x.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.7.4.1.x.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of corresponding DL SSB (TRP#1) of each TAG used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For TRP#2, the test system shall verify there is adjusted as well. Skip this step for Test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

<End of Change 1>

<Start of Change 2>

A.5.4.1.x	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting single DCI

A.5.4.1.x.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits, for UE for UE not configured PL offset and is configured with 2 TAGs for single-DCI multi-TRP operation. UE is also configured with dl-OrJointTCI-StateList. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in table A.5.4.1.x.1-1.

Table A.5.4.1.x.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | LTE TDD, NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

The test consists of E-UTRA PCell and NR PSCell. The configuration for E-UTRA is given in A.3.7.2.1. Tables A.5.4.1.x.1-2 and A.5.4.1.x.1-2A define the parameters to be configured and strength of the transmitted signals. The NR PSCell is configured with two TRPs in the test. Each TRP is associated with a CORESET, with coresetPoolIndex-r16 is set to 0 for the first TRP and set to 1 for the second TRP. UE is also configured with tag2 in ServingCellConfig. Two SRS resource sets are configured and associated to different TAGs via TCI state configuration. The transmit timing is verified by the UE transmitting SRS using the configuration defined in table A.5.4.1.x.1-3.

Table A.5.4.1.x.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  | Test2 |  |  |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP #1 | TRP #2 |  | TRP #1 |  | TRP #2 |  |  |
| SSB ARFCN |  | 1,2 | Freq1 |  |  |  |  |  |  |  |
| Duplex Mode |  | 1,2 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | 1,2 | TDDConf.3.1 |  |  |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 100: NPRB,c = 66 |  |  |  |  |  |  |  |
| Data PRBs allocated |  | 1,2 | 66 |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD TRS.2.2 TDD |  |  |  |  |  |  |  |
| DRX Cycle | ms | 1,2 | N/A |  |  | DRX.8Note5 |  |  |  |  |
| PDSCH Reference measurement channel |  | 1,2 | SR.3. 3 TDD |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.3.1 TDD |  |  |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2 | 0 | 1 |  | 0 |  | 1 |  |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.3.4 TDD | CCR.3.6 TDD |  | CCR.3.4 TDD |  | CCR.3.6 TDD |  |  |
| TCI configuration |  | 1,2 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 |  | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 |  | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 |  |  |
| Timing difference compared to TRP#1 | us | 1,2 | 0 | 0.1 |  | 0 |  | 0.1 |  |  |
| OCNG Patterns |  | 1,2 | OP.1 |  |  |  |  |  |  |  |
| SSB Configuration |  | 1,2 | SSB.1 FR2 |  |  |  | SSB.1 FR2 |  |  |  |
| SSB index |  | 1,2 | 0 |  | 1 |  | 0 |  | 1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |  |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 |  |  | SRSConf.2Note6 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: DRX related parameters are given in table A.3.3.8-1 NOTE 6: SRS configs are given in table A.7.4.1.x.1-3 |  |  |  |  |  |  |  |  |  |  |

Table A.5.4.1.x.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 3 as specified in clause A.3.15 Note 7 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15 kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -100 |  |
|  | dB | 4 |  |
| SSB_RPNote2 | dBm/SCS Note4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation NOTE 7: AoA1 for TRP1 and AoA2 for TRP2 |  |  |  |

Table A.5.4.1.x.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 |  | SRSConf.2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 1 | 0 | 1 |  |
|  | srs-ResourceIdList | 0 | 1 | 0 | 1 |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 1 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | 17 | 17 | Matches NPRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl2, 0 | sl2, 1 | sl2560, 4 | sl2560, 9 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | 0 | 0 | Any 10 bit number |
|  | TCI state | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 |  |

Table A.5.4.1.x.1-4: Void

A.5.4.1.x.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up E-UTRA PCell according to parameters given in table A.3.7.2.2-1 and setup NR PSCell according to parameters given in table A.5.4.1.x.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected corresponding path of DL SSB (TRP#1) for each TAG and detected another path of DL SSB (TRP#2).

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in table A.5.4.1.x.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.5.4.1.x.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of corresponding DL SSB (TRP#1) of each TAG used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For TRP#2, the test system shall verify there is adjusted as well.  Skip this step for Test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB of each TAG used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

<End of Change 2>

<Start of Change 3>

A.4.4.1.x	NR UE Transmit Timing Test with 2-TA and two TRPs for FR1 UE supporting single DCI

A.4.4.1.x.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timings change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for UE not configured PL offset and is configured with 2 TAGs for

single-DCI multi-TRP operation. The test is configured with two TRPs in NR PSCell. This test will verify the requirements in clause 7.1.2. Supported test configurations are shown in table A.4.4.1.x.1-1.

Table A.4.4.1.x.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | LTE FDD, NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | LTE FDD, NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 4 | LTE TDD, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 5 | LTE TDD, NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 6 | LTE TDD, NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

The test consists of E-UTRA PCell and NR PSCell. For NR PSCell, two TRPs are configured. The configuration for E-UTRA is given in A.3.7.2.1. Table A.4.4.1.x.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in table A.4.4.1.x.1-3.

Table A.4.4.1.x.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP#1 | TRP#2 | TRP#1 |  | TRP#2 |  |
| SSB ARFCN |  | 1,2,3,4,5,6 | Freq1 |  |  |  |  |  |
| Duplex Mode |  | 1,4 | FDD |  |  |  |  |  |
|  |  | 2,3,5,6 | TDD |  |  |  |  |  |
| TDD configuration |  | 1,4 | Not Applicable |  |  |  |  |  |
|  |  | 2,5 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3,4,5,6 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3,4,5,6 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle | ms | 1,2,3,4,5,6 | N/A |  |  | DRX.8Note5 |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | 2,5 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3,6 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,4 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2,5 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2,3,4,5,6 | 0 | 1 | 0 |  | 1 |  |
| Timing difference compared to TRP#1 | us | 1,2,4,5 | 0 | 3 | 0 |  | 3 |  |
|  |  | 3,6 |  | 1.5 |  |  | 1.5 |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |  |
| SSB configuration |  | 1,4 | SSB.3 FR1 |  |  |  |  |  |
|  |  | 2,5 | SSB.3 FR1 |  |  |  |  |  |
|  |  | 3,6 | SSB.4 FR1 |  |  |  |  |  |
| SSB index |  | 1,4 | 0 | 1 | 0 |  | 1 |  |
|  |  | 2,5 | 0 | 1 | 0 |  | 1 |  |
|  |  | 3,6 | 0 | 1 | 0 |  | 1 |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD for DLorJoint TCI.State.1.3 & DLorJoint TCI.State.1.4 |  |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD for DLorJoint TCI.State.1.3 & DLorJoint TCI.State.1.4 |  |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD for DLorJoint TCI.State.1.3 & DLorJoint TCI.State.1.4 |  |  |  |  |  |
| PDSCH/PDCCH | kHz | 1,2,4,5 | 15 |  |  |  |  |  |
| subcarrier spacing |  | 3,6 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -98 |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -98 |  |  |  |  |  |
|  |  | 3,6 | -95 |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | 3 |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | 3 |  |  |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -95 |  |  |  |  |  |
|  |  | 3,6 | -92 |  |  |  |  |  |
| IoNote3 | dBm/9.36 MHz | 1,2,4,5 | -65.2 |  |  |  |  |  |
|  | dBm/38.1 MHz | 3,6 | -59.2 |  |  |  |  |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  |  |
| SRS Config |  | 1,2,4,5 | SRSConf.1Note6 |  | SRSConf.3Note6 |  |  |  |
|  |  | 3, 6 | SRSConf.1Note6 |  | SRSConf.2Note6 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: DRX related parameters are given in Table A.3.3.8-1 NOTE 6: SRS configs are given in Table A.4.4.1.x.1-3 |  |  |  |  |  |  |  |  |

Table A.4.4.1.x.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 | 0 |  |
| ResourceSet | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2,4,5 25 for test configuration 3,6 | 25 | 14 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 5 | sl320, 3 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

A.4.4.1.x.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up E-UTRA PCell according to parameters given in table A.3.7.2.1-1 and setup NR PSCell according to parameters given in table A.4.4.1.x.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset)×Tc ± Te of the first detected corresponding path of DL SSB (index 0) for each TAG and detected another path of DL SSB (index 1).

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in table A.4.4.1.x.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.4.4.1.x.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of corresponding DL SSB (TRP#1) of each TAG used by the UE to determine downlink timing is received from the reference cell at UE antenna. For TRP#2, the test system shall verify there is adjusted as well. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of corresponding DL SSB of TRP#1 for each TAG. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

<End of Change 3>

<Start of Change 4>

A.6.4.1.x	NR UE Transmit Timing Test with 2-TA and two TRPs for FR1 UE supporting single DCI

A.6.4.1.x.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for UE not configured PL offset and is configured with 2 TAGs for

single-DCI multi-TRP operation. The test is configured with two TRPs in NR PCell. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in table A.6.4.1.1.1-1.

Table A.6.4.1.x.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.x.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in table A.6.4.1.x.1-3.

Table A.6.4.1.x.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP#1 | TRP#2 | TRP#1 |  | TRP#2 |  |
| SSB ARFCN |  | 1,2,3,4,5,6 | Freq1 |  |  |  |  |  |
| Duplex Mode |  | 1,4 | FDD |  |  |  |  |  |
|  |  | 2,3,5,6 | TDD |  |  |  |  |  |
| TDD configuration |  | 1,4 | Not Applicable |  |  |  |  |  |
|  |  | 2,5 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3,4,5,6 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3,4,5,6 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle | ms | 1,2,3,4,5,6 | N/A |  |  | DRX.8Note5 |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | 2,5 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3,6 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,4 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2,5 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2,3,4,5,6 | 0 | 1 | 0 |  | 1 |  |
| Timing difference compared to TRP#1 | us | 1,2,4,5 | 0 | 3 | 0 |  | 3 |  |
|  |  | 3,6 |  | 1.5 |  |  | 1.5 |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |  |
| SSB configuration |  | 1,4 | SSB.3 FR1 |  |  |  |  |  |
|  |  | 2,5 | SSB.3 FR1 |  |  |  |  |  |
|  |  | 3,6 | SSB.4 FR1 |  |  |  |  |  |
| SSB index |  | 1,4 | 0 | 1 | 0 |  | 1 |  |
|  |  | 2,5 | 0 | 1 | 0 |  | 1 |  |
|  |  | 3,6 | 0 | 1 | 0 |  | 1 |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD for DLorJoint TCI.State.1.3 & DLorJoint TCI.State.1.4 |  |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD for DLorJoint TCI.State.1.3 & DLorJoint TCI.State.1.4 |  |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD for DLorJoint TCI.State.1.3 & DLorJoint TCI.State.1.4 |  |  |  |  |  |
| PDSCH/PDCCH | kHz | 1,2,4,5 | 15 |  |  |  |  |  |
| subcarrier spacing |  | 3,6 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -98 |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -98 |  |  |  |  |  |
|  |  | 3,6 | -95 |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | 3 |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | 3 |  |  |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -95 |  |  |  |  |  |
|  |  | 3,6 | -92 |  |  |  |  |  |
| IoNote3 | dBm/9.36 MHz | 1,2,4,5 | -65.2 |  |  |  |  |  |
|  | dBm/38.1 MHz | 3,6 | -59.2 |  |  |  |  |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  |  |
| SRS Config |  | 1,2,4,5 | SRSConf.1Note6 |  | SRSConf.3Note6 |  |  |  |
|  |  | 3, 6 | SRSConf.1Note6 |  | SRSConf.2Note6 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: DRX related parameters are given in Table A.3.3.8-1 NOTE 6: SRS configs are given in Table A.4.4.1.x.1-3 |  |  |  |  |  |  |  |  |

Table A.6.4.1.x.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | 14 | Matches NPRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | sl320, 0 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

A.6.4.1.x.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1) Setup NR PCell according to parameters given in table A.6.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected corresponding path of DL SSB (index 0) for each TAG and detected another path of DL SSB (index 1).

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in table A.6.4.1.x.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.6.4.1.x.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of corresponding DL SSB (TRP#1) of each TAG used by the UE to determine downlink timing is received from the reference cell at UE antenna. For TRP#2, the test system shall verify there is adjusted as well. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of corresponding DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna of TRP#1 for each TAG. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

<End of Change 4>