3GPP TSG-RAN WG4 Meeting #117							   R4-2520963

Dallas, USA, 17th – 21th Nov, 2025

Agenda Item: 	8.2.1

Source: 	MediaTek Inc.

Title: 	Views on 6G PA model

Document for: 	Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion started from RAN4#116bis meeting. During RAN4#116bis, a WF for 6GR system parameter [2] was agreed, classifying topics and directions for further study. In this paper, we provide our views on the PA model.

# 2	PA model

In RAN1#122bis meeting, a RAN1 LS [3] was agreed to request a PA model from RAN4 to facilitate the waveform evaluation in RAN1. Before providing RAN1 the PA model, we believe that there are some high-level principles that needs to be discussed first.

Referring to current EVM requirements, the 5G NR impairment assumptions can handle all uplink transmission modulation waveforms. For 6G study, same impairment assumptions can be re-used as starting point to characterize new configurations, new low PAPR waveforms from RAN1 study outcome and agreed DFT-s-OFDM UL transmission with wider CBW. The general part in 38.101-1 6.2.2 about applicability and current 5G NR requirements on MPR aspect can also be a baseline to characterize wider CBW.

Note that the PA model is only for the typical PA performance via a proper output power range under a specific bias condition e.g., APT for assumption for evaluations in 6G low PAPR waveform study in RAN1. It does not imply the final Tx performance of UE. The performance variation of a commercial PA may not be fully reflected in the PA model that RAN4 provided.

Proposal 1: On PA modeling, re-use current Tx impairment assumptions for legacy FR1 refarming bands evaluation as listed below:

- Carrier leakage: 28dBc

- IQ imbalance: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Calibration CBW to meeting [30]dB ACLR at PC3 [1]dB MPR, 20MHz CBW as the baseline to evaluate wider CBW

Proposal 2: FFS on impairment assumptions for around 7GHz

Proposal 3: For refarming bands to evaluate wider CBW,

- If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

- If the relative channel bandwidth > 4% for TDD bands or > 3% for FDD bands, the ∆MPR framework and those been specified in 5G NR specs can be reused for CBW <=100MHz as baseline

Proposal 4: The 9th-order polynomial equation currently used for 5G NR can serve as a starting point. However, for modeling the nonlinearity in AM/AM and AM/PM characteristics, especially when the output power approaches the saturation region—where the 9th-order polynomial may not provide an adequate fit—RAN4 may also consider adopting a measurement-based LUT (Look-Up Table) model

Proposal 5: The PA model is only for the typical PA performance under a specific bias condition e.g., APT (Average Power Tracking, where the PA's supply voltage to follow the average power of the input signal) for assumption for evaluations in 6G low PAPR waveform study in RAN1. The performance variation of a commercial PA is not fully reflected in the PA model provided because single PA curve cannot fully define Tx performance of UE.

# 3	Summary

In this paper, we provide our views on 6G system parameters with the following observations and proposals for the PA modelling:

Proposal 1: On PA modeling, re-use current Tx impairment assumptions for legacy FR1 refarming bands evaluation as listed below:

- Carrier leakage: 28dBc

- IQ imbalance: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Calibration CBW to meeting [30]dB ACLR at PC3 [1]dB MPR, 20MHz CBW as the baseline to evaluate wider CBW

Proposal 2: FFS on impairment assumptions for around 7GHz

Proposal 3: For refarming bands to evaluate wider CBW,

- If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

- If the relative channel bandwidth > 4% for TDD bands or > 3% for FDD bands, the ∆MPR framework and those been specified in 5G NR specs can be reused for CBW <=100MHz as baseline

Proposal 4: The 9th-order polynomial equation currently used for 5G NR can serve as a starting point. However, for modeling the nonlinearity in AM/AM and AM/PM characteristics, especially when the output power approaches the saturation region—where the 9th-order polynomial may not provide an adequate fit—RAN4 may also consider adopting a measurement-based LUT (Look-Up Table) model

Proposal 5: The PA model is only for the typical PA performance under a specific bias condition e.g., APT (Average Power Tracking, where the PA's supply voltage to follow the average power of the input signal) for assumption for evaluations in 6G low PAPR waveform study in RAN1. The performance variation of a commercial PA is not fully reflected in the PA model provided because single PA curve cannot fully define Tx performance of UE.

# 4	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514650, WF for 6GR system parameter, Feature lead (Huawei, HiSilicon)

- [3] R1-2508069, LS on PA models in 6GR waveform evaluations, RAN WG1