3GPP TSG-RAN WG4 Meeting #117								    		 R4-2520967

Dallas, USA, 17th – 21st Nov, 2025

Agenda Item: 8.4.1

Source: MediaTek Inc.

Title: Views on 6G Tx EVM requirement framework

Document for: Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In RAN4#116b meeting, an WF [2] was agreed to further identify any 6G BS RF requirements that need to be re-assessed.

### Issue 1-1-2: Requirements to be re-assessed Way Forward: For next meeting, companies are encouraged to evaluate how 5G BS RF requirements should be considered for 6G and provide their view using the table in Annex A as example. Note: The goal is to identify: - 5G requirements that might also be applicable to 6G. - 5G requirements which will need to be re-evaluated based on 6G system parameters but would not need a further study. - 5G requirements which should be further studied in the scope of the 6G SI, a prioritization might be made in next meeting.

In this paper, we provide our views on the 6G BS Tx EVM requirement framework. Compared to our view submission [3] in RAN4#116b meeting, in this paper we further added our field log on DMRS SNR.

# 2	Discussions

From LTE to NR, the BS Tx EVM requirements were defined based on the modulation order. For an example, Table 6.5.2.2-1 from TS 38.104 is captured below.

Such modulation-specific requirement is fine, only when the number of MIMO layers is not very high (e.g., 1 or 2 layers). However, when the number of MIMO layers is higher, such an approach becomes problematic. With higher MIMO layers, even with the same modulation order, the required SNR will increase correspondingly. However, if we keep the same EVM value, the maximum achievable SNR will be upper bounded, as well as the throughput.

Observation 1: With current BS Tx EVM requirements in TS 38.104, the required EVM changes with the modulation order without considering the target MIMO layers.

To provide more detail insight, we conducted link-level simulations according the assumptions in Annex. In the simulation, we tried 3 different modulation orders (64QAM, 256QAM and 1024QAM) and different MIMO layers (2, 4, 6, 8). Throughput curves for EVM value 0% and EVM values from Table 6.4.2.2-1 above are provided to understand the impact of Tx EVM. Both R-ML receivers and MMSE receivers are evaluated. The results can be found in Figures 1 to 6.

| (a) | (b) |  |
| --- | --- | --- |

Figure 1. Throughput performance of 4x4 MIMO with 64QAM for ranks 2, 3, 4: (a) R-ML receiver, (b) MMSE receiver

| (a) | (b) |  |
| --- | --- | --- |

Figure 2. Throughput performance of 4x4 MIMO with 256QAM for ranks 2, 3, 4: (a) R-ML receiver, (b) MMSE receiver

| (a) | (b) |  |
| --- | --- | --- |

Figure 3. Throughput performance of 4x4 MIMO with 1024QAM for ranks 2, 3, 4: (a) R-ML receiver, (b) MMSE receiver

| (a) | (b) |  |
| --- | --- | --- |

Figure 4. Throughput performance of 8x8 MIMO with 64QAM for ranks 4, 6, 8: (a) R-ML receiver, (b) MMSE receiver

| (a) | (b) |  |
| --- | --- | --- |

Figure 5. Throughput performance of 8x8 MIMO with 256QAM for ranks 4, 6, 8: (a) R-ML receiver, (b) MMSE receiver

| (a) | (b) |  |
| --- | --- | --- |

Figure 6. Throughput performance of 8x8 MIMO with 1024QAM for ranks 4, 6, 8: (a) R-ML receiver, (b) MMSE receiver

From above results, we can see that even for the easiest case in Figure 1, the current 64QAM EVM requirement (8%) already proved itself infeasible for 3-layer MIMO transmission. With 0.85 code rate, the maximum throughput is not achievable, and the SNR to achieve 70% max throughput is degraded by 3-5dB. More degradations can be found other figures with more MIMO layers or higher modulation orders.

Observation 2: With the EVM values based on current TS 38.104, the max throughput cannot be achieved with more MIMO layers, and the degradation compared to EVM 0% can be huge in some scenarios.

In addition to link-level simulation, we also provide our field log data for DMRS SNR that can be experienced by UE in Figure 7. Note that the DMRS is already pre-coded toward a particular UE. Therefore, it can typically achieve a higher SNR than SSB. From Figure 7, we can see that the probably to see DMRS SNR larger than 30dB is around 7~8%.

Figure 7. DMRS SNR from 5G field log

In our view, achieving a high spectrum efficiency requires efforts in both network and UE side. Defining only UE requirements for more MIMO layer or higher-order QAM is not sufficient, if the BS EVM requirement does not improve corresponding. The current BS Tx EVM requirement framework which only considers the modulation order will be infeasible when the whole industry moves toward higher and higher MIMO layers transmissions. Therefore, we suggest RAN4 to study how to define a more pragmatic BS TX EVM requirement framework that can avoid the throughput degradation with higher layer MIMO.

Proposal 1: RAN4 to study a more pragmatic BS TX EVM requirement framework that also takes the number of MIMO layers into account.

# 3	Summary

In this paper, we provide our view on BS Tx EVM requirements framework. We have the following observation and proposals:

Observation 1: With current BS Tx EVM requirements in TS 38.104, the required EVM changes with the modulation order without considering the target MIMO layers.

Observation 2: With the EVM values based on current TS 38.104, the max throughput cannot be achieved with more MIMO layers, and the degradation compared to EVM 0% can be huge in some scenarios.

Proposal 1: RAN4 to study a more pragmatic BS TX EVM requirement framework that also takes the number of MIMO layers into account.

# 4	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514638, WF on 6G BS RF and co-existence, feature lead (Ericsson)

- [3] R4-2513076, Views on 6G BS Tx EVM requirements, MediaTek inc.

# 5	Annex

Table A link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz for FDD |
| CBW | 10MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, Maximum Doppler frequency: 10Hz |
| MCS (Code rate) | MCS with lowest and highest code rate for 64QAM, 256QAM and 1024QAM from MCS table 4 (Table 5.1.3.1-4 of TS 38.211) - 64QAM: MCS-6 (0.46) and MCS-14 (0.85) - 256QAM: MCS-15 (0.67) and MCS-22 (0.93) - 1024QAM: MCS-23 (0.79) and MCS-26 (0.93) |
| Precoding | Random type-1 precoder |
| Symbol type | CP-OFDM |
| HARQ | disabled |
| RANK | - 2/3/4 for 4T4R - 4/6/8 for 8T8R |
| BS and UE antenna configurations | - 4x4 - 8x8 |
| MIMO correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE and R-ML |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |
| DMRS configuration | - Rank <= 4, Type 1, Single symbol, 1 additional DMRS - Rank > 4, Type 1, Double symbol, 1 additional DMRS |
| txEVM | - 0% - NR requirements in TS38.104 |