3GPP TSG-RAN4 Meeting #117	 R4-2520999

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0139 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.108 for correction of NTN Ku band SAN requirements for section 9.4, 9.6, 10.3, 10.4 and 10.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL, Ericsson, ZTE Corporation, SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The big CR for 38.108 for Re.19 NTN Ku band was agreed during last RAN4 meeting. The group agree to introduce the FR1-NTN Ku band SAN requirement by separating the subclause of SAN type 1-O to operating below 10GHz and above 10GHz. However, as some section are expected to be no impact in the beginning, this cause misalignment between different sections. This draft CR is proposed to correct this issue. In addition, few corrections are made after further review. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR provides the following corrections: 1. Aligning the clause title as “SAN type 1-O operating below 10GHz”, and “SAN type 1-O above operating 10GHz” for section 9.4.2.2, 9.4.4, 9.6.1.2, 9.6.1.2a (new), 9.6.2.2, 9.6.2.2a (new), 10.4.2. 2. Add note 2 to Table 10.3.2a-1 to align with Table 10.3.3-1. 3. Revise the EISREFSENS_10M to EISREFSENS_50M in Table 10.9.2a-1 to align the specification. Note that the proposed change 1 and 2 are already endorsed in draft CR R4-2515140 in RAN4#116bis meeting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some errors and misalignment remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2.2, 9.4.4, 9.6.1.2, 9.6.1.2a (new), 9.6.2.2, 9.6.2.2a (new), 10.3.2a, 10.4.2, 10.9.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

9.4	OTA output power dynamics

9.4.1	General

Transmit signal quality (as specified in clause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

9.4.2	OTA RE power control dynamic range

9.4.2.1	General

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a SAN at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

9.4.2.2	Minimum requirement for SAN type 1-O operating below 10GHz

The OTA RE power control dynamic range is specified the same as the conducted RE power control dynamic range requirement for SAN type 1-H in table 6.3.2.2-1.

9.4.3	OTA total power dynamic range

9.4.3.1	General

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range is the SAN maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

9.4.3.2	Minimum requirement for SAN type 1-O operating below 10 GHz

OTA total power dynamic range minimum requirement for SAN type 1-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified for the conducted requirement for SAN type 1-H in table 6.3.3.2-1.

9.4.3.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

OTA total power dynamic range minimum requirement for SAN type 1-O operating above 10 GHz is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.2a-1 in FR1-NTN.

Table 9.4.3.2a-1: Minimum requirement for SAN type 1-O total power dynamic range in FR1-NTN

| SAN Operating Band | SCS | OTA total power dynamic range (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| n248, n247 | 15 | 17.1 | 18.9 | N/A | 21.2 | 22.7 | 24.3 | N/A | N/A |
|  | 30 | 13.8 | N/A | 17 | 18.1 | 19.6 | 21.2 | 22.7 | 24.3 |

9.4.3.3	Minimum requirement for SAN type 2-O

OTA total power dynamic range minimum requirement for SAN type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.3-1 in FR2-NTN.

Table 9.4.3.3-1: Minimum requirement for SAN type 2-O total power dynamic range in FR2-NTN

| SAN Operating Band | SCS | OTA total power dynamic range (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | 60 | 18.1 | 21.2 | 24.2 | N/A |
|  | 120 | 15.0 | 18.1 | 21.2 | 24.2 |
| n509, n508 | 120 | 15.0 | 18.1 | 21.2 | 24.2 |

9.4.4	NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band operating below 10GHz

9.4.4.1	General

The NB-IoT RB power dynamic range (or NB-IoT power boosting) is the difference between the average power of NB-IoT REs (which occupy certain REs within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel) and the average power over all REs (from both NB-IoT and the NR carrier containing the NB-IoT REs).

9.4.4.2	Minimum Requirement

NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band shall be larger than or equal to the level specified in Table 6.3.4.2-1. This power dynamic range level is only required for one NB-IoT RB.

Table 9.4.4.2-1: NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 20 | Within center 102*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Within center 90% of SAN channel bandwidth | +6 |
|  | Other | +3 |

9.5	OTA transmit ON/OFF power

The requirement is not applicable in this version of the specification.

9.6	OTA transmitted signal quality

9.6.1	OTA frequency error

9.6.1.1	General

OTA frequency error is the measure of the difference between the actual SAN transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

OTA frequency error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

9.6.1.2	Minimum requirement for SAN type 1-O operating below 10GHz

For SAN type 1-O operating below 10GHz, the modulated carrier frequency of each carrier configured by the SAN shall be accurate to within 0.05 ppm observed over 1 ms.

The frequency error requirement for NB-IoT are specified in TS 36.108 [17] clause 9.6.1.2.

9.6.1.2a	Minimum requirement for SAN type 1-O operating above 10GHz

For SAN type 1-O operating above 10GHz, the modulated carrier frequency of each carrier configured by the SAN shall be accurate to within 0.05 ppm observed over 1 ms.

9.6.1.3	Minimum requirement for SAN type 2-O

For SAN type 2-O, the modulated carrier frequency of each carrier configured by the SAN shall be accurate to within 0.05 ppm observed over 1 ms.

9.6.2	OTA modulation quality

9.6.2.1	General

Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). Details about how the EVM is determined are specified in annex B for FR1-NTN.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

9.6.2.2	Minimum requirement for SAN type 1-O operating below 10GHz

For SAN type 1-O operating below 10GHz, the EVM levels of each carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met. Requirements shall be the same as clause 6.5.2.2 and follow EVM frame structure from clause 6.5.2.3.

The modulation quality requirements for NB-IoT are specified in TS 36.108 [17] clause 9.6.2.2.

9.6.2.2a	Minimum requirement for SAN type 1-O operating above 10GHz

For SAN type 1-O operating above 10GHz, the EVM levels of each carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met. Requirements shall be the same as clause 6.5.2.2 and follow EVM frame structure from clause 6.5.2.3.

9.6.2.3	Minimum requirement for SAN type 2-O

9.6.2.3.1	EVM frame structure for measurement

For SAN type 2-O, the EVM levels of each carrier for different modulation schemes on PDSCH outlined in table 9.6.2.3.1-1 shall be met.

Table 9.6.2.3-1: EVM requirements for SAN type 2-O carrier

| Applicability | Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- | --- |
| FR2-NTN | QPSK | 17.5 |
| FR2-NTN | 16QAM | 12.5 |
| FR2-NTN | 64QAM | 8 |

9.6.2.3.2	EVM frame structure for measurement

EVM requirements shall apply for each carrier over all allocated resource blocks. Different modulation schemes listed in table 9.6.2.3.1-1 shall be considered for rank 1.

9.6.3	OTA time alignment error

The requirement is not applicable in this version of the specification.

<< Next changes >>

10.3.2a	Minimum requirement for SAN type 1-O operating above 10GHz

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz SAN channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz SAN channel bandwidth it does not imply that SAN has to support 50 MHz SAN channel bandwidth.

For GEO class SAN, EISREFSENS_50M is an integer value in the range of minimum value to maximum value specified in table 10.3.2a-0. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range of minimum value to maximum value specified in table 10.3.2a-0. The specific value is declared by the vendor.

Table 10.3.2a-0: FR1-NTN EISREFSENS_50M ranges

| FR1-NTN bands above 10GHz | SAN class | EISREFSENS_50M |  |
| --- | --- | --- | --- |
|  |  | Maximum value (dBm) | Minimum value (dBm) |
| n248, n247 | GEO | -129 | N/A |
|  | LEO | -120 | -129 |

Table 10.3.2a-1: SAN GEO and LEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | EISREFSENS_50M -10.3+ ΔFR1_REFSENS |
| 10 | 30 | G-FR1-NTN-A1-2 (Note 1) | EISREFSENS_50M -10.8+ ΔFR1_REFSENS |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 (Note 1) | EISREFSENS_50M -4+ ΔFR1_REFSENS |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 (Note 1) | EISREFSENS_50M -4.2+ ΔFR1_REFSENS |
| Note 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. Note 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

10.3.3	Minimum requirement for SAN type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz SAN channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz SAN channel bandwidth it does not imply that SAN has to support 50 MHz SAN channel bandwidth.

For GEO class SAN, EISREFSENS_50M is an integer value in the range of minimum value to maximum value specified in table 10.3.3-0. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range of minimum value to maximum value specified in table 10.3.3-0. The specific value is declared by the vendor.

Table 10.3.3-0: FR2-NTN EISREFSENS_50M ranges

| FR2-NTN bands | SAN class | EISREFSENS_50M |  |
| --- | --- | --- | --- |
|  |  | Maximum value (dBm) | Minimum value (dBm) |
| n512, n511, n510 | GEO | -140 | -149 |
|  | LEO | -120 | -129 |
| n509, n508 | GEO | -129 | N/A |
|  | LEO | -120 | -129 |

Table 10.3.3-1: FR2-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-NTN-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-NTN-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-NTN-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

<< Next changes >>

10.4	OTA dynamic range

10.4.1	General

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received SAN channel bandwidth.

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

10.4.2	Minimum requirement for SAN type 1-O operating below 10GHz

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 10.4.2-1 for LEO SAN.

For NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A of TS 36.108 [17] with parameters specified in table 10.4.2-1a for LEO SAN.

The requirement is not applicable for SAN type 1-O operating above 10GHz.

Table 10.4.2-1: SAN LEO class dynamic range

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-NTN-A2-7 | -79.3- ΔOTAREFSENS | -90.4- ΔOTAREFSENS | AWGN |
| 5 | 15 | G-FR1-NTN-A2-1 | -76.4 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -77.1 - ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-NTN-A2-1 | -76.4 - ΔOTAREFSENS | -85.0 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -77.1 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -74.1- ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-NTN-A2-1 | -76.4- ΔOTAREFSENS | -83.2 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -77.1 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -74.1 - ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-NTN-A2-4 | -70.2 - ΔOTAREFSENS | -81.9 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-5 | -70.2 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-6 | -70.5 - ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |  |

Table 10.4.2-1a: LEO SAN dynamic range for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | FRC A15-1 in Annex A.15 in TS 36.108 [17] | -89.4 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
| 10 |  |  | -85.0 - ΔOTAREFSENS |  |
| 15 |  |  | -83.2 - ΔOTAREFSENS |  |
| 20 |  |  | -81.9 - ΔOTAREFSENS |  |
| 5 | FRC A15-2 in Annex A.15 in TS 36.108 [17] | -95.3 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
| 10 |  |  | -85.0 - ΔOTAREFSENS |  |
| 15 |  |  | -83.2 - ΔOTAREFSENS |  |
| 20 |  |  | -81.9 - ΔOTAREFSENS |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |

<< Next changes >>

10.9.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA.

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For SAN type 1-O operating above 10GHz, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 10.9.2a-1. The characteristics of the interfering signal is further specified in annex D.

Table 10.9.2a-1: OTA in-channel selectivity requirement for SAN type 1-O operating above 10GHz

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 25, 35 | 15 | G-FR1-NTN-A1-1 | EISREFSENS_50M -7 ΔFR1_REFSENS | EISREFSENS_50M + 3 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 50 | 15 | G-FR1-NTN-A1-4 | EISREFSENS_50M + ΔFR1_REFSENS | EISREFSENS_50M + 10 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 10, 20, 25, 35 | 30 | G-FR1-NTN-A1-2 | EISREFSENS_50M -7 ΔFR1_REFSENS | EISREFSENS_50M + 3 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RB |
| 50, 70,100 | 30 | G-FR1-NTN-A1-5 | EISREFSENS_50M + ΔFR1_REFSENS | EISREFSENS_50M + 10 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is declared by the vendor as specified in clause 10.3.2a. |  |  |  |  |  |

<< End of changes >>