# 3GPP TSG RAN WG4 Meeting#117	R4-2521001

# Dallas, USA, Nov 17 – 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Simulation results for 6Rx devices

Agenda item:	6.1.2

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the simulation assumptions for 6Rx devices were finalized [1]. In this paper, we present our simulation results for demodulation and CSI reporting requirements.

2. PDSCH Demodulation Results

Based on the agreed simulation assumptions in [1], below we provide our simulation results for PDSCH demodulation requirements for rank 4 and rank 6.

Table 1: Required SNR for achieving 70% peak throughput for rank 4

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 4 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6 ULA with α = 0 and β = 0.07179 | 70 | 15.7 |

Table 2: Required SNR for achieving 70% peak throughput for rank 6

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 6 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 8x6 ULA with α = 0 and β = 0 | 70 | 18.7 |

3. SDR Requirements

Based on the assumptions provided in [1], below we provide our simulation results for SDR tests with 6-layer (256QAM and 64QAM) and 2-layer (1024QAM).

Table 3: MCS values for SDR tests with 6-layer

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 25 |
| 6 | 6 | 1 | 26 |

Table 4: MCS value for SDR tests with 2-layer

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 2 | 10 | 1 | 23 |

4. CQI Reporting Requirements

In this section, we provide our simulation results for CQI reporting requirement based on the simulation assumptions provided in [1].

Table 5: SNR values for CQI reporting requirements

| Test 1 (SNR) |  | Test 2 (SNR) |  |
| --- | --- | --- | --- |
| 6 | 7 | 11 | 12 |

5. Conclusions

In this paper, we have provided our simulation results for the PDSCH, SDR and CQI reporting requirements for 6Rx devices.

6. References

[1] R4-2514788, “Way Forward for Topic for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx,” 3GPP TSG RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025.