3GPP TSG RAN WG4 Meeting #117	R4-2521007

Dallas, USA, 17 – 21 November, 2025

Agenda item:			4.8.2

Source:	vivo

Title:	Remaining issues on RRM requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the RAN4#116bis meeting, RAN4 discussed remaining issues on RRM core requirements for low band CA via switching WI. Following agreements were made and captured in the WF [1].

Issue 1-1-2: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA The issue can be closed (without any additional clarification or spec change in RAN4). Issue 1-1-3a: Application time of Switching Pattern in SCell activation Issue 1-1-3b: switching pattern based carrier switching in SCell activation - TLBCA =1ms for MAC CE based, 0 ms for RRC based. Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation Issue 1-1-4b: switching pattern based carrier switching in SCell activation - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

In addition, there are remaining open issues to be solved in maintenance phase. In the previous RAN4 meetings, we provided our views in [2, 3, 4, 5, 6]. In this contribution, we further provide our views on the remaining open issues for RRM core requirements for low band carrier aggregation with switching.

# Discussion

SDL SCell activation

In the last RAN4 meeting, it was agreed that processing margin applying switching pattern for SCell activation is TLBCA =1ms for MAC CE based, 0 ms for RRC based. However, how the UE performs switching based on applied switching pattern has not been decided yet.

There were extensive online and offline discussions on carrier switching for SCell activation. Following was discussed but not agreed.

Tentative Agreement: - For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot. - For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(TRRC_Process +TLBCA)/slot-length. Switching is placed at the end of the slot.

According to the procedures specified in TS 38.213, the switching gap ends at the end of the last slot from a number of consecutive slots of the carrier.

When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration

It cannot cover the switching case during SCell activation. For the switching from PCell to SCell, the switching should be located on the end of last slots of PCell. During SCell activation, when switching pattern applies after triggering by MAC-CE or RRC, the following slot may not be for the PCell depending on the switching pattern configuration. Therefore, additional clarification on switching during SCell activation is needed.

Proposal 1: UE switching during SCell activation/deactivation should be specified in TS 38.133.

When SCell is deactivated or not configured, the UE is continuing staying on PCell without switching though switching pattern is configured. If SCell activation is triggered via MAC-CE or RRC, after THARQ+3ms+1ms or TRRC_Process the UE applies the switching pattern. Then, UE should perform switching accordingly.

As depicted in Fig 1, the UE should be on PCell according to switching pattern. In this case, UE stay on the PCell and performs switching according to the configured switching pattern.

Fig 1. SCell activation: Switching pattern and switching case 1

As depicted in Fig 2-1 and Fig 2-2, the UE should be on SCell according to switching pattern. In this case, there could be different switching mechanism.

In Fig 2-1, UE switches to SCell immediately after switching pattern is applied by creating new switching gap for FDD carrier to SDL carrier. The pros are that UE can access the SCell spectrum as early as possible. The cons are that UE may stay on the SCell for a very short time depending on the timing of switching pattern application.

In Fig 2-2, UE switches to SCell at the next switching period according switching pattern after switching pattern is applied. It has less overhead due to switching compared to Fig 2-1.

Fig 2-1. SCell activation: Switching pattern and switching case 2

Fig 2-2. SCell activation: Switching pattern and switching case 2

According to the above RAN1 specification, there are two principles should be followed. Firstly, switching is placed at the end of the slot. Secondly, the switching from PCell to SCell is located at the last slot of PCell. Otherwise, there could be inconsistent between RAN1 and RAN4 specification.

With this principle, the mechanism in Fig 2-2 should be adopted. Though mechanism in Fig 2-1 was preferred in the last meeting, it is worthy of reconsideration.

Following the wording in the last meeting, which can be directly captured in the spec, following is proposed.

Proposal 2:

For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the last slot from a number of consecutive slots that belongs to the PCell as per the switching pattern, occurring after the slot n+(THARQ+3ms+TLBCA)/slot-length, Switching is completed at the end of the slot.

For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the last slot from a number of consecutive slots that belongs to the PCell as per the switching pattern, occurring after the slot: n+TRRC_Process/slot-length. Switching is completed at the end of the slot.

Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation Issue 1-1-4b: switching pattern based carrier switching in SCell activation - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

The UE switching procedure and LB-CA processing time for applying switching pattern for MAC-CE triggered SCell deactivation were agreed. However, it is still open for timer triggered SCell deactivation.

In the SCell deactivation requirements, following delay requirements are specified.

Upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + 3msNR slot length.

The triggering is based on timer sCellDeactivationTimer. It is maintained in MAC layer. It is obvious that no MAC-CE decoding is needed. However, there is still 3ms processing time allowed for timer triggering. Therefore, there is absolutely no need for additional time margin for applying switching pattern. It can be done within this 3ms.

For the switching procedure, there should be no difference for MAC-CE triggered and timer triggered SCell deactivation.

Proposal 3: The switching pattern is NOT applied after n+ (3ms+TLBCA)/slot-length) for timer based SCell deactivation and TLBCA = 0 ms.

Proposal 4: For timer based SCell deactivation, if UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ 3ms/slot-length.

Issue 1-2-1: Switching pattern application after BF(beam failure) or LF(link failure) - FFS: - Discussion following options: - Option 1: No optimization for switching pattern applicability is needed related to beam or link recovery (BFD, CBD, RLM) in Rel-19. - Option 2: - No optimization for switching pattern applicability is needed related to CBD in Rel-19. - The switching pattern is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). - The switching pattern is activated upon stopping RLF timer (i.e., upon stopping T310). - Option 3: - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD). - The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure.

For RLM requirement for PCell, further enhancement can be further considered. When there is bad radio link quality on PCell, the SCell operation is also highly impacted. There will be no uplink for ACK/NACK feedback. If SCell is scheduled by PCell, then there will also be no DL scheduling on SCell. Therefore, it is beneficial to prioritize the procedure of radio link recovery.

Then it comes to the question when the switching pattern is NOT applied and when it is applied again. Using timer T310 is one potential options. When timer T310 starts after the UE receives N310 consecutive OOS indications, the switching pattern is not applied anymore. After the UE receives N311 IS indications, timer T310 stops and the switching pattern can be applied again. With this approach, the radio link recovery procedure can be fastened. Furthermore, no interruption requirements need to be specified.

For CBD procedure, it was agreed to define requirement for BFD/CBD for SDL SCell in LB CA via switching. In our view, it is not necessary features for FR1 operation below 1GHz. It may not be used in practical deployment. Therefore, it would be not necessarily to enhance CBD measurement for SDL SCell further. Similar to SDL SCell we don’t think it is necessary feature and further enhancement is not needed.

Proposal 5: Radio link recovery procedure is enhanced for LB-CA via switching by not applying switching pattern when timer T310 starts and applying switching pattern when timer T310 stops.

Proposal 6: There is no need to enhance CBD measurement requirements for both FDD PCell and SDL SCell.

Issue 1-3-1: BWP switching requirement for LB CA - FFS: - Proposal 1 (Apple): - RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case. - Proposal 2 (LGE): - Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows: - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs - Proposal 3 (Ofinno): - Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the same serving cell on which the active BWP switching occurs. - Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the FDD PCell.

To avoid complicating the usage of this feature unnecessarily, it would be reasonable to assume the BWP switching happens when the UE stays on the carrier without switching. The BWP switching delay is either 1 slot or 3 slots, so it could be handled by network easily.

Proposal 7: No requirements are defined for overlapping of BWP switching and carrier switching.

# Summary

In this contribution, we further provided views on remaining issues for RRM core requirements for low band carrier aggregation with switching. Following proposals are provided.

Proposal 1: UE switching during SCell activation/deactivation should be specified in TS 38.133.

Proposal 2:

For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the last slot from a number of consecutive slots that belongs to the PCell as per the switching pattern, occurring after the slot n+(THARQ+3ms+TLBCA)/slot-length, Switching is completed at the end of the slot.

For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the last slot from a number of consecutive slots that belongs to the PCell as per the switching pattern, occurring after the slot: n+TRRC_Process/slot-length. Switching is completed at the end of the slot.

Proposal 3: The switching pattern is NOT applied after n+ (3ms+TLBCA)/slot-length) for timer based SCell deactivation and TLBCA = 0 ms.

Proposal 4: For timer based SCell deactivation, if UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ 3ms/slot-length.

Proposal 5: Radio link recovery procedure is enhanced for LB-CA via switching by not applying switching pattern when timer T310 starts and applying switching pattern when timer T310 stops.

Proposal 6: There is no need to enhance CBD measurement requirements for both FDD PCell and SDL SCell.

Proposal 7: No requirements are defined for overlapping of BWP switching and carrier switching.

- References

- R4-2514870 WF on NR_LBCA_Sw_RRM, Apple

- R4-2500828 Discussion on RRM requirements for low band carrier aggregation via switching, vivo

- R4-2504028 Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2506454	Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2510323	Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2514075 Remaining issues on RRM core requirements for LB CA via switching, vivo