3GPP TSG RAN WG4 Meeting #117	R4-2521010

Dallas, USA, 17 – 21 November, 2025

Agenda item:			7.8.4

Source:	vivo

Title:	Further discussion on LB CA via switching enhancement

Document for:	Discussion

# Introduction

In the RAN4#116bis meeting, there were initial discussions on LB CA via switching enhancement. Following agreements were achieved and captured in the WF [1].

- Issue 1-2-1: Switching pattern applicability Agreement: - Switching pattern applicability for RRM requirement in Rel-20 LB-CA via switching: - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is applied. - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - Issue 1-3-2: SDL SCell related requirement Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements

In this contribution, we further provide our views on RRM impacts on LA CA via switching enhancement.

# Discussion

## General

The main difference between Rel-19 and Rel-20 LB-CA via switching is the scenarios to be supported. There are 3 cases being considered in 3GPP.

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

Switching between Case 1 and Case 2 were supported in Rel-19. It is to standardization switching between Case 1 and Case 3 in Rel-20. For case 3, dual downlink of FDD + SDL are supported but there is no uplink for both downlinks. The uplink transmission is only possible when UE switches to Case 1.

The LB CA via switching for Rel-19 and Rel-20 are illustrated in Fig 1 and Fig 2, respectively.

Fig. 1 LB-CA via switching in Rel-19

Fig. 2 LB-CA via switching in Rel-20

Deployment constraints are also important in terms of RRM impact. It is summarized below for both Rel-19 and Rel-20.

| Rel-19 | Rel-20 |
| --- | --- |
| - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers | - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected |

It can be seen the only difference is that there is no in-gap interference between DL carriers. With this assumption, there will be no extra RRM impact due to deployments constraints compared to Rel-19.

Observation 1: The main difference between Case 2 and Case 3 is that dual DL is supported in Case 3.

Regarding the switching pattern, it is to reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching according to the WID. The switching pattern in Rel-19 is a 40bits bitmap and its field description is as below.

switchingPattern Indicates which carrier is configured for the corresponding slot for low NR band carrier aggregation via switching between FDD and supplementary downlink (SDL) carrier. ‘0’ indicates FDD carrier (PCell), and ‘1’ indicates SDL carrier (SCell). The pattern starts from the beginning of SFN 0 of PCell.

It can be seen this bit map configuration cannot be directly used for switching between Case 1 and Case 3. A simple approach is to introduce a new UE capability in Rel-20 and reuse the field description of Rel-19 switching pattern with changing the meaning of ‘1’ indication to indicate both FDD downlink carrier (PCell DL) and SDL carrier (SCell). With this change, the semi-static switching pattern design in Rel-19 can be reused.

Proposal 1: LS to RAN1 and RAN2 on necessary change of switching pattern to support switching between case 1 and case 3.

Issue 1-2-2: Interruption or scheduling restriction requirement for CC switching

In the Rel-19 LB-CA via switching, there is no interruption/scheduling restriction requirements for switching between DL CCs. The switching happens within configured gap duration. This should also apply to Rel-20 switching.

In the last RAN4 meeting, following agreements were made in RF session and captured in WF [2].

Issue 4: Switching period for switching between Case 1 and Case 3 - Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations - Switching period capability values {35us, 70us, 140us} - The capability shall be per band pair per band combination Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3 - Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching

The switching period capability for switching between case 1 and case 3 is the same as that between case 1 and case 2. With this agreement, the gap duration specified in Rel-19 can be reused without any change. From RRM perspective, no interruption/scheduling restriction requirements are needed.

Proposal 2: No interruption/scheduling restriction requirements for switching between case 1 and case 3.

## SCell related requirements impact

In the last meeting, following agreements on SCell related requirements for Rel-19 LB-CA via switching were reached.

Agreement:

  - It is RAN4 common understanding that the following RRM requirements are defined in Rel-19 LB-CA switching

| SDL SCell related RRM requirement | Index | Requirements |
| --- | --- | --- |
|  | 1 | Deactivated SDL SCell measurement |
|  | 2 | Interruption due to deactivated SCell measurement |
|  | 3 | Activated SDL SCell L3 measurement |
|  | 4 | BFD/CBD for SDL SCell |
|  | 5 | L1-RSRP/SINR for SDL SCell |
|  | 6 | SDL SCell activation/deactivation |
|  | 7 | TCI state switch delay |

  - For Rel-20, following the WID, RAN4 to Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

From RRM requirements perspective, all the impact requirements in Rel-19 should also be defined in Rel-20. The difference is only on switching cases and there is no other difference from deployment and operation point of view.

Proposal 3: In Rel-20 LB-CA enh WI, requirements should be specified for Deactivated SDL SCell measurement, Interruption due to deactivated SCell measurement, Activated SDL SCell L3 measurement, BFD/CBD for SDL SCell, L1-RSRP/SINR for SDL SCell, SDL SCell activation/deactivation and TCI state switch delay.

For SDL SCell operation, downlink is not always available when switching between case 1 and case 3 as illustrated in Fig 2. Comparing Fig 1 and Fig 2, there is no difference between Rel-19 switching and Rel-20 switching. In addition, it was agreed that switching pattern is not applicable when SCell is deactivated and it is applicable when SCell is activated and during SCell activation. Therefore, from SCell operation perspective, there is no difference compared to switching between case 1 and case 2. Rel-19 requirements for LB-CA via switching can be directly reused.

Regarding impact to other serving cells due to measurements on SCell, e.g., interruption to PCell due to deactivated SCell measurement, the Rel-19 requirements for LB-CA can also be reused as the switching pattern is not applied. Optimization may be considered due to UE capability of dual DL reception. It is similar to legacy deactivated SCell measurements. However, switching time could be different from legacy which would lead to a bit different interruption requirement in terms of missed ACK/NACK.

Proposal 4: In Rel-20 LB-CA enh WI, requirements for SDL SCell operation for Deactivated SDL SCell measurement, Activated SDL SCell L3 measurement, BFD/CBD for SDL SCell, L1-RSRP/SINR for SDL SCell, SDL SCell activation/deactivation and TCI state switch delay are to reuse corresponding Rel-19 LB-CA requirements.

Proposal 5: In Rel-20 LB-CA enh WI, RAN4 to study requirements for Interruption due to deactivated SCell measurements. Two options can be considered that option 1 is to reuse corresponding Rel-19 LB-CA requirements and option 2 is to update legacy interruption requirements for deactivated SCell measurement.

## PCell related requirements impact

In the last meeting, following agreements on SCell related requirements for Rel-19 LB-CA via switching were reached.

Agreement:

  - It is RAN4 common understanding that the following RRM requirements are defined in Rel-19 LB-CA switching

| FDD PCell related RRM requirement | 8 | PCell L3 measurement |
| --- | --- | --- |
|  | 9 | BFD/CBD for PCell |
|  | 10 | L1-RSRP/SINR for PCell |
|  | 11 | RLM for PCell |
|  | 12 | TCI state switch delay |
|  | 13 | PL-RS switch delay for PCell |

  - For Rel-20, following the WID, RAN4 to Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

For PCell operation, downlink is always available when switching between case 1 and case 3 as illustrated in Fig 2. It is different from Rel-19 switching. The requirements for PCell operation under Rel-19 LB-CA via switching is not appliable anymore.

The impacted requirements in Rel-19 including RLM, BFD/CBD, L1-RSRP/ L1-SINR, PL-RS switch delay, TCI state switching and intra-frequency measurement without gap. Compared to legacy PCell operation, there are extra switching between PCell and SCell. However, we don’t think we need to considered the above measurements or procedures overlapping with switch gap. It can be handled by network with proper configuration on measurements and switching pattern. With this, legacy requirements other than Rel-19 LB-CA requirements should be applicable.

Since legacy requirements are already exists, application of legacy requirements for Rel-20 LB-CA should be clarified in the specification.

Proposal 6: For UE supporting switching between Case 1 and Case 3, legacy requirements other than Rel-19 LB-CA requirements for RLM, BFD/CBD, L1-RSRP, L1-SINR, PL-RS, TCI state switching and intra-frequency measurement without gap are applicable for PCell operation.

Proposal 7: No requirements are defined for overlapping between switch gap and PCell measurements/procedure.

# Summary

In this contribution, we further provided views on provide our views on specification improvement and RAN4 operation related aspects. Following proposals are provided.

Observation 1: The main difference between Case 2 and Case 3 is that dual DL is supported in Case 3.

Proposal 1: LS to RAN1 and RAN2 on necessary change of switching pattern to support switching between case 1 and case 3.

Proposal 2: No interruption/scheduling restriction requirements for switching between case 1 and case 3.

Proposal 3: In Rel-20 LB-CA enh WI, requirements for SDL SCell operation should be specified for Deactivated SDL SCell measurement, Interruption due to deactivated SCell measurement, Activated SDL SCell L3 measurement, BFD/CBD for SDL SCell, L1-RSRP/SINR for SDL SCell, SDL SCell activation/deactivation and TCI state switch delay.

Proposal 4: In Rel-20 LB-CA enh WI, requirements for Deactivated SDL SCell measurement, Activated SDL SCell L3 measurement, BFD/CBD for SDL SCell, L1-RSRP/SINR for SDL SCell, SDL SCell activation/deactivation and TCI state switch delay are to reuse corresponding Rel-19 LB-CA requirements.

Proposal 5: In Rel-20 LB-CA enh WI, RAN4 to study requirements for Interruption due to deactivated SCell measurements. Two options can be considered that option 1 is to reuse corresponding Rel-19 LB-CA requirements and option 2 is to update legacy interruption requirements for deactivated SCell measurement.

Proposal 6: For UE supporting switching between Case 1 and Case 3, legacy requirements other than Rel-19 LB-CA requirements for RLM, BFD/CBD, L1-RSRP, L1-SINR, PL-RS, TCI state switching and intra-frequency measurement without gap are applicable for PCell operation.

Proposal 7: No requirements are defined for overlapping between switch gap and PCell measurements/procedure.

# References

- R4-2514824 WF on NR_LBCA_Sw_enh_RRM, Samsung

- R4-2515156 Way Forward for [116bis][320] NR_low_band_CA_switching, Apple

- Appendix: Draft LS on switching pattern for LB-CA enhancement

3GPP TSG RAN WG4 Meeting #117	R4-25x

Dallas, USA, 17th – 21st November, 2025

Title:	LS on switching pattern for LB-CA enhancement

Response to:

Release:	Rel-20

Work Item:	NR_LBCA_Sw_enh

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 discussed the reuse of R19 switching pattern for LB-CA via switching in R20 WI.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

The switching pattern in Rel-19 is a 40bits bitmap and its field description is as below.

switchingPattern

Indicates which carrier is configured for the corresponding slot for low NR band carrier aggregation via switching between FDD and supplementary downlink (SDL) carrier. ‘0’ indicates FDD carrier (PCell), and ‘1’ indicates SDL carrier (SCell). The pattern starts from the beginning of SFN 0 of PCell.

For LB-CA via switching, three cases are considered in R19 and R20.

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

The R19 switching pattern bit map configuration for switching between case 1 and case 2 cannot be directly reused for switching between Case 1 and Case 3 in R20 from field description of the switching pattern.

RAN4 thinks a simple approach is to introduce a new UE capability in Rel-20 and reuse the field description of Rel-19 switching pattern with changing the meaning of ‘1’ indication to indicate both FDD downlink carrier (PCell DL) and SDL carrier (SCell).

It is up to RAN1 to decide how to design switching pattern to be used for R20.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to address the applicability of switching pattern for R20 LB-CA via switching enhancement.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #118	 	   February 9 – February 13, 2026			Sweden