3GPP TSG RAN WG4 Meeting #117                                   R4- 2521015

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.11.7

Source:	vivo

Title:	Discussion on RRM performance requirements for positioning accuracy

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In last meeting, the following agreement is achieved [2].

Issue 3-1: Requirements for case 1 To be clarified in the requirements: - Request location information message should refer to “case 1” positiniong - Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation) - Beam sweeping factor for FR2 including sweeping reduction factor - Take the legacy values - Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled - Cover all RRC states CR structure: - Introduce new clause for AI/ML based positioning case 1 - Reference existing requirements wherever they are reused to avoid duplicating the same requirement Introduce requirements for each RRC state in the corresponding clauses: Draft for RRC idle: Nokia Draft for RRC connected: E/// Draft for RRC inactive: Nokia Issue 3-2: CR for case 3a/3b Agreement: Clause 13.2(Rx-Tx reporting) from R4-2513670 (CMCC) to be merged into R4-2514119 (Ericsson). CMCC to further check until end of this meeting on other clauses E/// to provide final CR in RAN4#117 Issue 3-3: Testing for case 1 Agreement: Introduce “case 1” tests only for reporting delay, one for FR1 and one for FR2 - Only connected mode - Only reporting delay to be tested - Only for the case with gaps

In this contribution, we further provide our views on RRM core requirements and test framework for positioning accuracy enhancement.

- Discussion

In last meeting, it was agreed to reuse the legacy RSTD measurement delay for all the cases. Tests will be defined only for connected mode UE location reporting. With the agreements, though the core requirements for AI/ML positioning case 1 have not been finalized yet, the existing RSTD measurement delay test cases in connected mode can be used as baseline to design the tests for case 1.

The RSTD measurement delay tests for SA are defined in clause A.6.6.12.1 for FR1 and in clause A.7.6.9.1 for FR2. For the test configuration for AI/ML positioning case 1, the general test parameters and cell specific test parameters in the two existing tests can be reused, respectively.

Since only reporting delay is to be tested, the test requirements should be only to verify the UE location delay. No accuracy requirements need to be met.

Proposal 1: The existing RSTD measurement delay test cases in clause A.6.6.12.1 for FR1 and in clause A.7.6.9.1 for FR2 are used as baseline in terms of test configuration for AI/ML based positioning case 1.

Proposal 2: The test requirement is to only verify reporting delay and no accuracy needs to be verified.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Proposal 1: The existing RSTD measurement delay test cases in clause A.6.6.12.1 for FR1 and in clause A.7.6.9.1 for FR2 are used as baseline in terms of test configuration for AI/ML based positioning case 1.

Proposal 2: The test requirement is to only verify reporting delay and no accuracy needs to be verified.

- References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

- R4-2514631	WF on [116bis][112] R19 AI for air interface, Qualcomm