3GPP TSG RAN WG4 Meeting #117                                   R4- 2521018

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.11.5

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In last meeting, the following agreement is achieved.

Issue 3-1: Requirements for case 1 To be clarified in the requirements: - Request location information message should refer to “case 1” positiniong - Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation) - Beam sweeping factor for FR2 including sweeping reduction factor - Take the legacy values - Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled - Cover all RRC states CR structure: - Introduce new clause for AI/ML based positioning case 1 - Reference existing requirements wherever they are reused to avoid duplicating the same requirement Introduce requirements for each RRC state in the corresponding clauses: Draft for RRC idle: Nokia Draft for RRC connected: E/// Draft for RRC inactive: Nokia Issue 3-2: CR for case 3a/3b Agreement: Clause 13.2(Rx-Tx reporting) from R4-2513670 (CMCC) to be merged into R4-2514119 (Ericsson). CMCC to further check until end of this meeting on other clauses E/// to provide final CR in RAN4#117 Issue 3-3: Testing for case 1 Agreement: Introduce “case 1” tests only for reporting delay, one for FR1 and one for FR2 - Only connected mode - Only reporting delay to be tested - Only for the case with gaps

In this contribution, we further provide our views on RRM core requirements and test framework for positioning accuracy enhancement.

- Discussion

For AI/ML positioning cases, 5 sub-use cases are identified and summarized in Table 1.

- A detailed list of various positioning cases

| Cases | Priority | Model deployment | Positioning methods | Measured by which entity | Model output | Position calculation |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1st priority | UE-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | UE-side |
| Case 2a | 2nd priority | UE-side | AI/ML assisted positioning | UE (Downlink positioning) | Intermediate feature | LMF |
| Case 2b | 2nd priority | LMF-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | LMF |
| Case 3a | 1st priority | gNB-side | AI/ML assisted positioning | gNB (uplink positioning) | Intermediate feature | LMF |
| Case 3b | 1st priority | LMF-side | Direct AI/ML positioning | gNB (uplink positioning) | Position | LMF |

In RAN2#126 meeting, agreements about UE side model are reached as

Agreements - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

For case 1, RAN2 agreed to use the existing LPP procedures related to Location Information Transfer to provide the UE sided model inference results, which is the UE position, to the NW side. It means that there is a potential interactive process between NW and UE. RAN4 could consider define a delay requirement from the start of UE measuring PRS, through the AI processing, to the end of UE report location to LMF. Since further signaling enhancements may needed in RAN2, more progresses are also needed for defining delay requirement for case 1.

In RAN4#116 meeting, agreements of reporting delay requirement are reached. No more new requirement would be needed for case 1

Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements

For the UL positioning method case 3a and 3b, the model is deployed in gNB side and LMF side, respectively. RAN4 does not need to define any new requirement for such 2 cases. Legacy requirement (if exists) can be reused.

In RAN4#122bis meeting, RAN4 has agreed not to define requirements for performance monitoring delay for both CSI prediction and positioning.

Issue 1-1: Requirements for performance monitoring Agreement: No performance monitoring delay requirements are specified in RAN4 for CSI prediction. Issue 2-3: Reporting delay requirements for monitoring Agreement: Do not define any explicit requirements for reporting delay of RS-PAI performance monitoring metric.

Thus, we have the following proposal.

Proposal 1: No performance monitoring delay requirements are specified in RAN4 for AI/ML based positioning.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Proposal 1: No performance monitoring delay requirements are specified in RAN4 for AI/ML based positioning.

- References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).