3GPP TSG-RAN WG4 Meeting #117	R4-2521041

Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on WUS demodulation requirements for FR1 FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The agreement of MDR performance requirements for WUS reached, hence related descriptions for WUS demodulation performance requirements should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - Add descriptions for FR1 FDD WUS demodulation performance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No FR1 FDD WUS demodulation performance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.X	WUS demodulation requirements

The receiver characteristics of the WUS are determined by the probability of miss-detection of WUS (Pm-wus), and the Pm-wus is derived with the assumption of false-alarm rate of WUS (FAR-wus).

The parameters specified in Table 5.X.2-1 are valid for all WUS tests unless otherwise stated.

5.X.3	Minimum requirements for FDD

The performance requirements are specified in Table 5.X.3-2 and Table 5.X.3-3 with the addition of test parameters in Table 5.X.3-1.

Table 5.X.3-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| Subcarrier spacing |  | kHz | 15 |
| Bandwidth |  | MHz | 10 |
| LP-WUS bandwidth |  | RB | 11 |
| Number of OOK symbols in an OFDM symbol (MWUS) |  |  | 4 |
| Number of overlaid sequences (Nseq) |  |  | 4 |
| Root sequence number (q) |  |  | 1 |
| Number of root sequence numbers (Nroot) |  |  | 1 |
| Number of information bits (K) |  | Bits | Test 1-1 and 2-1: 2 Test 1-2 and 2-2: 5 |
| Number of bits after rate matching (E0) |  | Bits | Test 1-1 and 2-1: 12 Test 1-2 and 2-2: 32 |
| Time offsets |  | us | 1 |
| Frequency offsets |  | kHz | OFDM based receiver: 4.5 OOK based receiver: 9 |
| EPRE ratio between WUS and SSB |  | dB | [0] |
| LP-SS | Number of OOK symbols in an OFDM symbol (MLPSS) |  | [4] |
|  | Number of overlaid sequences (Nseq) |  | [4] |
|  | Periodicity | ms | [160] |
|  | EPRE ratio between LP-SS and SSB | dB | [0] |

Table 5.X.3-2: Minimum performance requirements for OOK-based receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Information bits | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-wus (%) | FAR-wus (%) | SNR (dB) |
| 1-1 | R.WUS.1-1 | 10 / 15 | 2 | TDLA30-10 | 1x1, ULA Low | 1 | 1 | [TBD] |
| 1-2 | R.WUS.1-2 | 10 / 15 | 5 | TDLA30-10 | 1x1, ULA Low | 1 | 1 | [TBD] |

Table 5.X.3-3: Minimum performance requirements for OFDM-based receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Information bits | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-wus (%) | FAR-wus (%) | SNR (dB) |
| 2-1 | R.WUS.1-1 | 10 / 15 | 2 | TDLA30-10 | 1x1, ULA Low | 1 | 1 | [TBD] |
| 2-2 | R.WUS.1-2 | 10 / 15 | 5 | TDLA30-10 | 1x1, ULA Low | 1 | 1 | [TBD] |

==============End of change==============