3GPP TSG-RAN WG4 Meeting #117	R4-2521097

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 MIMO

Agenda Item:	6.12.2.2

Document for:	Discussion

- Introduction

The performance requirements of Rel-19 MIMO UEIEBM were initially discussed in last meeting [1]. In this contribution, we provide our views on performance part for CJTC and asymmetric TRPs.

- Discussion

For accuracy requirements for CJTC, the status are summarized as follows based on the discussion in last meeting.

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration Way forward: Alternatives for further discussion: Alt 1: Define one set of [tighten] accuracy requirements based on SNR >= 0 or 3dB Encourage further input on the exact performance requirement Note: “tighten” means it is tighten compared to -3dB side condition Alt 2: Define two sets of requirements with UE capability Encourage input on details of the UE capability Issue 1-2: Test scope for UE reporting enhancement for CJT calibration? Agreement: - Define 2 TCs - TC1: FR1 CJTC-Dd measurement period and accuracy - TC2: FR1 CJTC-F measurement period and accuracy

Based on discussion in last meeting, companies commented that performance of CJTC accuracy based on the simulations campaign show that the poor accuracy cannot help for CJT calibration. For instance, the accuracy of CJTC-Dd may exceed half of the total range. Thus, some companies proposed to define two set of accuracy requirements, and one of them is defined with high accuracy which may serve the purpose of CJTC. However, from our understanding, it is abnormal to define two set of accuracy requirements based on UE capabilities. And if it is claimed the accuracy proposed based on simulation campaign is useless, it is not clear how the tighter one can be achieved. Thus, based on above, we slightly prefer to define one set of requirements based on high SNR condition.

Proposal 1: Define one set of requirements at high SNR condition for CJTC.

For asymmetric TRPs, RAN4 has conclude on the test case list in last meeting. One remaining issue whether to define applicability rules between EN-DC and NR-SA.

Issue 2-1: Test scope for asymmetric DL sTRP/UL mTRP scenarios Agreement: - For scenario#1, - TC1: Timing Advance Adjustment Accuracy Test for FR1 - TC2: Timing Advance Adjustment Accuracy Test for FR2 - For scenario#2, - TC3: uplink transmission timing 2TA FR1 - TC4: uplink transmission timing 2TA FR2 Agreement: - Add all 4 TCs for EN-DC, and further discuss whether to add the applicability rule.

From performance perspective, there is no difference between NR-SA and EN-DC since the feature is operated on a single NR carrier. However, this is no precedent about applicability rules between EN-DC and NR-SA. Thus, as TC are defined for EN-DC, we prefer do not define appliability rule between EN-DC and NR-SA.

Proposal 2: For asymmtric TRPs, do not define applicability rules between EN-DC and NR-SA.

- Conclusion

Proposal 1: Define one set of requirements at high SNR condition for CJTC.

Proposal 2: For asymmtric TRPs, do not define applicability rules between EN-DC and NR-SA.

References

[1] R4-2514803	WF on NR_MIMO_Ph5_RRM_Part1