3GPP TSG-RAN WG4 Meeting #117	R4-2521101

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 NES OD-SIB1

Agenda Item:	6.15.2.3

Document for:	Discussion

- Introduction

The perf part of Rel-19 NES was initially discussed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on remaining issues on performance part for OD-SIB1.

- Discussion

The performance part has been initially discussed in last meeting with following agreements.

Issue 2-1: Whether to define TC for OD-SIB1 Background Agreements in RAN4#115 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.. Agreement: - RAN4 to define cell reselection TC with longer TSI-NR for OD-SIB1. Issue 2-2: Detail test case design Way Forward - Option 1: RAN4 to define a new test case to verify the cell reselection behaviour together with longer TSI-NR in case of OD-SIB1. - UE transmits UL-WUS transmission - UE receives and decodes RAR - UE receives OD-SIB1 within OD-SIB1 window - Further discuss the exact value on the longer TSI-NR and the ending point - Option 2: - Target Cell (Cell2) is a NES Cell. - The ending point of the TC is UL WUS transmission. - UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR. Issue 2-3: TC scope Agreement: - Cell reselection FR1 to FR1 target Cell - Cell reselection FR2 to FR2 target Cell

One remaining issue is how to design the TC for UE obtaining OD-SIB1 in the TC. As commented during last meeting, due to RAN1 agreement which allow UE to detect SIB1 before UL-WUS transmission, UE’s behaviour cannot be strictly restricted in the test case. For option 1, there is no need to verify RAR reception and SIB1 reception, which can already be verified in legacy test cases. The only new procedure compared with legacy Cell reselection is to transmit UL-WUS to obtain SIB after the UE decide to reselect to a new Cell. Thus, it is straightforward for TC design to define the ending point as the UL-WUS transmission. Besides, regarding the time duration for TSI-NR. Since 1280ms is assumed to in legacy TC as the maximum time that UE needs for SIB decoding, thus we think it is reasonable to reuse the same value for OD-SIB1 TC, which means after 1280ms (i.e the time that UE may decide to decode SIB before UL-WUS) without obtaining the SIB, UE shall transmit UL-WUS to trigger OD-SIB1 without further delay.

Proposal 1: For TC of OD-SIB1, the ending point of the TC is defined as the UL WUS on NES Cell. UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

For the detailed TC configuration, another issue we would like to discuss is the time periods defined in the TC. It could be observed in legacy Cell reselection that three time periods are configured in the TC and Cell reselection to a high priority cell and to a low priority cell are both verified in a single TC. It could be observed the total test time is verify long. Since the motivation is to verify the new procedure (i.e., OD-SIB1), there is no need to consider two times Cell reselection in the new added TC for OD-SIB1.

Proposal 2: In the new Cell reselection TC, only verify the case that UE reselect from Cell 1 to Cell2 (NES Cell), and no need to verify the case the UE reselect back from Cell2 (NES Cell) to Cell 1 as designed in legacy Cell reselection TC.

- Conclusion

Proposal 1: For TC of OD-SIB1, the ending point of the TC is defined as the UL WUS on NES Cell. UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

Proposal 2: In the new Cell reselection TC, only verify the case that UE reselect from Cell 1 to Cell2 (NES Cell), and no need to verify the case the UE reselect back from Cell2 (NES Cell) to Cell 1 as designed in legacy Cell reselection TC.

References

[1] R4-2514809	WF on Netw_Energy_NR_enh_RRM_Part1