guj3GPP TSG-RAN WG4 Meeting #117	R4-2521127

Dallas, USA, 17th – 21st November 2025

Agenda Item:	6.1.2

Source:	Ericsson

Title:	Simulation results for SDR and CSI reporting requirements for 6Rx

Document for:	Information

# 1	Introduction

In RAN4 #114 meeting, companies agreed to introduce 6Rx SDR requirements with 2-layers. In RAN4 #115, it’s been agreed to determine the MCS value for 1024QAM, 256QAM and 64QAM with scaling factor 1. In this contribution, we provided our simulation results for the purpose of facilitating those MCS values.

In the meantime, we provided our simulation results for 4-layer and 6-layer test cases. We also provided our results for CQI reporting requirements.

# 2	Simulation results for SDR

In this section, we first exhibit the parameter assumptions that we used for the SDR simulations. Then, we provided our simulation results showing the SNR point for different MCS values.

## 2.1	Simulation assumption

We followed table 5.5A-1 in TS38.101-4 and changed the antenna to 2T6R, 4T6R and 8T6R for simulations. Corresponding static channel precoding we have used the agreed matrixes in the WF [1].

## 2.2	Simulation results

As agreed in RAN4 #116bis meeting in Prague, RAN4 should determine the following TBD values for each modulation order.

Issue 3-1-4: SDR requirements - Agreement - Decide the MCS values as per the agreed static channel matrix in Issue 3-1-1 and parameters in Table 5.5A-1/2/3 of TS 38.101-4 in the next meeting - QPSK/16QAM/64QAM/256QAM with 6 MIMO layers as below table Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS. 6; 8; 1; TBD. 6; 8; 0.8; 23. 6; 8; 0.75; 22. 6; 8; 0.4; 12. 6; 6; 1; TBD. 6; 6; 0.8; 24. 6; 6; 0.75; 23. 6; 6; 0.4; 14. 6; 4; 1; 16. 6; 4; 0.8; 16. 6; 4; 0.75; 16. 6; 4; 0.4; 11. 6; 2; 1; 9. 6; 2; 0.8; 9. 6; 2; 0.75; 9. 6; 2; 0.4; 5. Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12]. - 1024QAM with 2 MIMO layers as below table Supported RX antenna ports; Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS. 6RX; 2; 10; 1; TBD. 2; 10; 0.8; 21. 2; 10; 0.75; 19. 2; 10; 0.4; 9. Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5.

For the purpose of determining the MCS value for 64QAM, 256QAM and 1024QAM, we have conveyed the following simulation results.

Table 2.2-1 Simulation results for 64QAM/256QAM/1024QAM

| TxRx antenna configuration | Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS index Table for PDSCH | MCS | Scaling factor | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 8Tx6Rx | 6 | 6 (64QAM) | Table 5.1.3.1-1: MCS index table 1 | 28 | 1 | 22.9 |
|  |  |  |  | 27 |  | 20.8 |
|  |  |  |  | 26 |  | 18.7 |
|  |  |  |  | 25 |  | 17.7 |
|  | 6 | 8 (256QAM) | Table 5.1.3.1-2: MCS index table 2 | 27 |  | 27.9 |
|  |  |  |  | 26 |  | 26 |
|  |  |  |  | 25 |  | 26 |
|  |  |  |  | 24 |  | 23.9 |
|  |  |  |  | 23 |  | 22 |
| 2Tx6Rx | 2 | 10 (1024QAM) | Table 5.1.3.1-4: MCS index table 4 | 24 |  | 25.9 |
|  |  |  |  | 23 |  | 23.8 |

# 3	Simulation results for 4-layer and 6-layer cases

| Duplex Mode and SCS | CHBW | Rank | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD 30kHz | 40MHz | Rank 4 | 4T6R | Random | TDLA30-10 | ULA, alpha=0, beta=0.07179 | MCS13 | 17.2 |
|  |  | Rank 6 | 8T6R |  |  | ULA, alpha=0, beta=0 | MCS 17 | 17.7 |

# 4	Simulation results for CQI reporting

## 4.1	Simulation assumption

We followed the agreed simulation assumption below to derive the results:

Issue 4-1-1: CQI test for 6Rx - Agreements: - Define CQI requirements with rank 4 under static channel - Propagation channel: AWGN - Antenna: 4x6 with static channel specified in Annex B.1 - RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

Detailed parameter assumptions can be found in the table below:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 4x6 with static channel H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 4,(13) |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration (Note 1) |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1, CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00010001 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | 8 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |
| Note 1: The PMI associated to i2 = 0 is always used as the precoder regardless of the reported i2 value. |  |  |  |  |  |  |  |

## 4.2	Simulation results

In this section, we provided our simulation results for CQI reporting requirements.

Figure 4.2-1 Simulation results depicting the median CQI as function of SNR for 6Rx UE

# 4	Summary

In this contribution, we shared the simulation results for PDSCH, SDR and CQI reporting for 6Rx UE demodulation.

# References

[1]	R4-2514788: Way Forward for Topic for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #116bis, Prague, CZ