TSG-RAN Working Group 4 (Radio) meeting #117	  R4-2521135

Dallas, TX, USA, 17 – 21 November 2025

Source:	Ericsson

Title:	General RF and UE RF: miscellaneous

Agenda item:	8.3

Document for:	Approval

# Background

In this contribution we discuss miscellaneous UE RF aspects based on the agreed WF in [1].

# UE RF characteristics: general considerations

NR still has a way to go, NSA operation is prevalent on many markets. A starting point for 6GR studies could be corresponding features for which NR performance could be further improved, e.g. more dynamic carrier/bandwidth aggregation schemes

Relating to UE RF requirements, listed below are areas in which NR performance could be improved:

- CA performance e.g. power derating for DL-only configurations, Pcell power degradation at configuration/activation, very large power back-off for concurrent transmission

- power capability reporting inconsistent in the field (based on capabilities and field descriptions proposed by RAN4)

- reciprocity-based DL-MIMO, non-reciprocal insertion loss and SRS output power performance

- UL MIMO coherent transmissions – performance?

- UE power back-off: requirements not based on state-of-the-art while recognising considerable efforts made to derive MPR based on measurements, P-MPR, unpredictable MPR for FR2

- MSD and support of simultaneous Rx-Tx

- uncertainty to what extent requirements apply for operations in the field (e.g. MPR)

Some other aspects are more related to the channel arrangement, bandwidths and configuration, but also UE RF performance requirements:

- guard bands for intra-band CA and uncertainty about the applicability for SCS deployed

- applicability of requirements for the UE-specific channel bandwidths within a wider carrier

- CA requirements applicable for configured CCs or activated CCs?

All is not bad, however: NR performance in the field is still being improved, although EN-DC operation is still prevalent. Nonetheless, the above are areas for possible improvements by corresponding features supported by 6GR and areas of improvements worth considering in the 6GR study.

In the current 3GPP conformance framework for FR1, conducted requirements are defined under an assumption that simplifies the role of the antenna. Specifically, TS 38.101-1 states that “for UEs with an integral antenna only, a reference antenna [external] with 0 dBi gain is assumed for each antenna port”. This approach provides a neutral baseline for conducted testing, but it effectively decouples the RF performance of the device from the realities of antenna efficiency and radiation characteristics. Since typical FR1 handset antennas rarely achieve 0 dBi effective gain and often exhibit efficiencies around –5.5 dB, it is worth considering whether conducted requirements could be more meaningfully related to OTA performance by factoring in realistic antenna efficiency values. This would help bridge the gap between laboratory conducted tests and real-world device behaviour.

Observation 2.1. In FR1 conducted TS 38.101-1, “for UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s)”. It is worth discussing if the conducted requirement could be related to the OTA requirement by at least assuming a reasonable antenna efficiency, e.g. a typical FR1 antenna efficiency is -5.5 dB.

Proposal 2.1: discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB.

# Transmitter characteristics

## TX power accuracy

For NR, the performance of reciprocity-based DL-MIMO is degraded by SRS power accuracy and unknown non-reciprocal losses in the UE. RAN4 output power requirements for 6GR should therefore also be specified and verified for reference symbols. Moreover, RAN4 requirements on coherent UL transmission should ensure performance for coherent UL-MIMO. We propose that

Proposal 3.1: RAN4 collaborates with RAN1 from the start of the 6G study on the design of any reference symbols to be used for reciprocity-based DL-MIMO, if specified, and for coherent UL-MIMO with due account of expected UE transmit performance.

## UE power class structure and the default power class

The default power class

Over time, the number of operating bands in 5G NR supporting Power Class 2 (PC2) or higher power class increased significantly. In the field, it has become common that a UE is equipped with at least one Power Amplifier (PA) supporting 26 dBm. In RAN4#116-bis meeting, many companies proposed to consider PC2 as the default power class for 6GR – either for all operating bands, either for TDD bands only, either for TDD bands above certain frequency only etc.

However, the default power class should not only consider what is possible in implementations today, e.g. PC2 for midbands, but also to account for:

- The power class assumed by operators for nominal cell plans is usually PC3;  To facilitate the most rapid possible migration from 5G NR, it would be beneficial for operators to reuse the same nominal cell plan for 6GR, due to site acquisition management and related constraints.

- The power class that does not need provisions for SAR compliance under a single CC and no other simultaneous transmissions assumption. In 5G NR, that is assumed to be PC3.

- Not all operating bands in 5G NR support PC2.

- The number of device types will be presumably larger in 6GR than in 5G NR. Should there be different default power classes for different types of UEs?

Remark: Once the UE capability framework is introduced by RAN2, it should be discussed whether it would be needed to introduce a default power class reporting per UE capability (for the relevant ones).

Proposal 3.2: Reuse PC3 (23 dBm) as the default power class for all operating bands in 6GR.

UE power ability for single CC and CA

Ambiguous and unclear power capability reporting are still issues for 5G NR. The problems are that:

- there wasno consensus that the per-band UE power class for non-CA also limits the per-band maximum power for UL/DL CA;

- the per-band-per-BC power class is therefore used beyond its intended purpose;

- the per-BC power class powerClass that should be limiting the total configured transmitted power PCMAX is not limiting all transmissions of a configuration e.g. for single-cell transmissions in an inter-band configuration in some implementations this power threshold is exceeded. The application of the default power class when the capability is absent varies between different UE implementations.

One example problem in NR is an ambiguity on the power class supported for the single UL configured for DL-only CA band combinations (BC), included in the supported band-combination list or fallbacks of another top-level BC, or for non-CA NR band combinations included in the band-combination to convey feature sets, as it can be indicated by three different capabilities: the per-band ue-PowerClass, the per-BC powerClass and the optional per-band-per-BC ue-PowerClassPerBandPerBC-r17. There is an ambiguity at the network side on which one does it apply for the UE. Absence of a per-BC capability could mean either that the HPUE only supports DL-only CA with PC3 or that this power class is unintentionally omitted for the supported BC in the BC list.

Proposal 3.3: Study how to design the UE capability framework on the supported UE power class for single carrier/CA, such that it is unambiguous for the NW and robust, so that it works for either single or multiple configured UL carriers.

Another important discussion point in the previous meeting was on removal of the definition of inter-band CA power class. The CA power class limits the total configured power PCMAX for the band combination, the threshold at which the UE prioritizes UL transmissions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order […]

presumably due to power limitations governed by SAR limit as determined by proprietary algorithms. RAN4 was supposed to specify the bounds of PCMAX in 38.101-1, but the specification in the latter is more related to compliance of the UE maximum output power for conformance tests.

In case RAN1 specifies a similar total power threshold for UL transmission prioritization for 6GR spectrum aggregation then the CA power class is relevant; if not, then a ‘higher-power limit’ would be relevant (i.e. absence of a total power limit) for inter-band CA with the power of UL carriers/BWP limited by the per-band power class. At any rate, the NW should be made aware of any changes of the power capability of uplink transmissions due to SAR/MPE or reasons of UE heat-management.

Observation 3.1: specification of a power class for CA depends on whether RAN1 specifies a total power limit at which the UE prioritizes UL transmissions.

Improvements on configured maximum output power requirements and associated parameters

In RAN4#116-bis meeting, there was some discussion on removing/relaxing the power class upper limit to improve the UL coverage and data throughput, by e.g. removing/relaxing the value of 2dB tolerance of the nominal maximum output power for the given power class (the discussion was not limited to that method only).

UL coverage and throughput have remained the bottleneck for 5G NR networks, and in our view, how they can be improved should be continuously studied. Relaxation or removal of the current maximum output power limits specified in 5G NR could be considered. However, allowing UEs to (significantly) increase their maximum output power without informing the NW would hardly bring any improvement in UL performance, since the scheduler wouldn’t be aware of the UE power capabilities. Hence, relaxation or removal of the current maximum output power limits could be studied, but only if accompanied by the appropriate UE reporting.

Proposal 3.4: modification or removal of current maximum output power limits could be studied, but only if accompanied by the appropriate UE reporting.

## UE time masks

Tx transients and relation to RRM requirementsIn Ericsson’s UERF contribution for 6G SI RAN4#116bis [2] we pointed out that the RRM requirementin RRM specification is related to Cell Phase Synchronization and TDD transient times in RF specifications through the formula:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period

It also needs to accommodate sufficient time for TX<=>RX switching. This means that , Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together.

In existing NR specification (TS 38.133, Table 7.1.2-2) there are several coexistence cases listed, since the RRM requirement NTA offset affects UL timing. There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2, like MRSS.

Observation 3.2: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2, like MRSS.

Furthermore, in our RRM contribution for 6G SI RAN4#116bis from Ericsson [3] we also observe that a large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network. A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics. We get a higher level of redundancy as more alternative synchronization sources can be used. This leads to a more robust network.

As highlighted above, we consider robustness, deployment flexibility, and the ability to leverage existing infrastructure to be more important than marginal TDD overhead reductions. Therefore, we propose to maintain the TDD Cell Phase Synchronization requirement at the same level as in NR NR, i.e., 3 μs. Similarly, we believe that synchronization enhancements are better utilized to improve overall system robustness rather than to achieve the aforementioned minor TDD overhead reductions.

Observation 3.3: In Ericsson’s RRM contribution we propose to keep TDD Cell Phase Synchronization requirement the same as in NR.

Keeping the same as in NR due to various configuration and inter RAT co-existence dependencies which in NR FR1 and FR2 and proposing that TDD Cell Phase Synchronization requirement the same as in NR leads to the conclusion that:

Proposal 3.5: Transmitter transient period can be the same in NR for FR1 and FR2.

# UE receiver characteristics

## Receiver reference architecture

In Rel-18, a type 2 handheld UE was defined to support inter-band non-contiguous EN-DC and intra-band non-contiguous NR-CA with up to 2 layers. Building on this, Rel-19 introduced a type 4 FWA UE for intra-band non-contiguous non-collocated EN-DC/NR-CA, featuring the capability of handling 25 dB power imbalance and a MRTD of less than 33 µs, with up to four MIMO layers (Rank 4) per Component Carrier.

According to the WF [1] RAN4 should

- Strive to agree UE reference architecture on case-by-case manner for features being studied. Discuss reference architecture together with the feature itself as necessary. (first 6G agreement in RAN4)

  - Base evaluations on agreed reference architecture, while resulting requirements should apply for any feasible implementation architecture as much as possible

- If no reference architecture is agreed, recommend companies to mention assumed architecture together with evaluation results for easier comparisons between inputs

While RAN4 will strive to agree reference architectures on a case-by-case basis based on “any feasible implementation architectures”, this should not be based on worst-case assumptions:

Proposal 4.1: 6GR feature requirements should be based on realistic UE architecture assumptions, rather than relying on basic UE profiles that may not support the intended feature; Rx reference architectures should be based on the capability required for support of a feature, e.g. support of large power differences between carriers for spectrum aggregation schemes should not be based on worst-case assumptions of any UE implementation.

## Selectivity and blocking

Adequate selectivity and blocking performance are also important for UE performance in the field. DL coverage and performance do not only depend on the link budget. In-band and out-of-band blocking, for example, ensure that the UE performance in the wanted channel can be maintained when the UE is operated in the neighbourhood of a site supporting another operating band or a site belonging to a uncoordinated network in the same operating band.

With 6G expected to involve denser deployments, more spectrum fragmentation, the selectivity requirements will be essential to prevent receiver desensitization.

Observation 4.1: selectivity/blocking are also important for user performance/DL-coverage, not only in-channel/link-budget performance.

## Simultaneous Rx-Tx and MSD requirements

Simultaneous Rx-Tx

The specification of MSD for NR band combination has been simplified in Rel-19. MSD is verified in conducted test essentially verifying the noise factor of the receiver when the UE is configured with CA or DC. The conducted MSD might also determine whether simultaneous Rx-Tx can be mandated for band combination (relevant for cross-band isolation performance). To this end, the OTA performance is more relevant for performance in the field. We therefore propose

Proposal 4.2: minimum requirements for MSD for band combinations are also specified OTA to complement the existing noise-factor (MSD) requirement since also relevant for support of simultaneous Rx-Tx for band combinations.

We also observe that

Observation 4.2: It is important that margins to actual REFSENS performance including any MSD are not too excessive as simultaneous Rx-Tx is crucial for performance regardless of any scheduler coordination.

MSD OTA

The measurement of MSD in conducted set-up isolates the effect of the internal RF chain and ignores the real antenna environment. A key drawback is that conducted tests do not capture antenna mutual coupling, radiation leakage, housing effects, or user interaction, all of which contribute to real-world desensitization. They also assume ideal impedance matching and duplexer isolation, which are rarely true in practice. By shifting MSD to OTA testing, these missing effects would be included, leading to results that better reflect end-to-end device performance. OTA testing would reveal degradations caused by antenna placement, device form factor, and near-field coupling, giving operators a more realistic view of UE behaviour in the field. It would also allow assessment of self-interference paths through radiation, not just circuit leakage. It would also indicate whether simultaneous Rx-Tx can be supported for a band combination.

The only limiting factor for measuring MSD in OTA set-up might be the potential longer measurement time compared to a conducted set-up. But there are methods to reduce this measurement time. These methods will be studied in the Testability and OTA thread.

# Low power WUR

For energy efficiency topic relating to WUR, RAN1 has lates agreement relating to the

Agreement

Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects:

- Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other)

- Measurements and/or synchronization.

- System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS.

- RRC states

- Other functionalities

- Observation 5.1: RAN1 start to evaluate the OFDM based DL WUS.

- Proposal 5.1: Study WUR feature enhancement from RAN4 aspects.

RAN1 allows the UE to autonomously enable/disable wake up receiver in 5G and this creates confusion on the testability of a wake up receiver on a LR (LP-WUS Receiver) level or on a UE level.  When companies talk about the “smart” wake up receiver with detection of the jammer presence and in such case wake the main receiver instead, it gives the impression such “wake up receiver” could never cope with a jammer but rely on main receiver to cope with this jammer. Though this complies with RAN1 design, the power saving gain is questionable as the jammer is not predictable in real network operation.

- Observation 5.2: UE autonomously enables/disables wake up receiver makes the power saving gain of WUR unpredictable.

Therefore, we propose not considering the autonomous WUR behavior in 6G study assuming the WUR RF parameter could be different with MR so as to motive the coexisting study using DL WUS.

Proposal 5.2: Not considering the 5G autonomous WUR/MR switching behaviour in 6G WUR study.

During the 5G work item, there is a discrepancy for the WUR NF definition between RAN1 and RAN4. E.g the implantation margin and band specific relaxation for NF is missing in RAN1 assumption. As RAN1 assumes only OFDM based DL WUS signal in 6G study, RAN4 could study the NF for WUR in study phase.

Proposal 5.3: Study the WUR NF in 6G study.

# Spectrum aggregation

For NR, the problems with CA from an RF and UE output power perspective are

- ambiguous reporting ion UE CA capabilities

- large MPR for concurrent transmissions

- degradation of Pcell coverage when CA is configured (UE power reduction increased at configuration)

- Scell output power when the UE is power limited

- PCMAX for CA as specified in the 38.101-1 is not related to UE behaviour as specified in 38.213 (only to the max power requirements in RAN4 conformance test)

- uncertain UL-MIMO output power capability when combined with CA, particularly when UL Tx switching is used

According to the 38.213, the total configured power PCMAX is the threshold at which the UE prioritizes UL transmissions, presumably due to power limitations governed by SAR limit as determined by proprietary algorithms as discussed above. RAN4 was supposed to specify the bounds of PCMAX in 38.101-1, but the specification in the latter is more related to compliance of the UE maximum output power for conformance tests.

We assume that 6GR output power for UE transmissions would also be limited by a configured maximum power per carrier similarly to NR. The power prioritization for new dynamic spectrum schemes and any equivalent PCMAX is RAN1 responsibility, but RAN4 engagement at an early stage is important. It may be more important that the NW is informed of any change of the total UE power capability relative to a total configured power more relevant for facilitating SAR compliance.

NR CA performance can still be improved, but the list above could be considered for the study of output power requirements for 6GR spectrum aggregation. To this end we propose

Proposal 6.1: for spectrum aggregation, study the feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme (cf. increase of the allowed MPR for the Pcell at NR CA configuration).

Proposal 6.2: for spectrum aggregation, the total configured power PCMAX, or equivalent power, should be related to the behaviour of the UE for any multi-carrier scheme specified. Changes of the total output power capability could be reported by (a)periodic dedicated signalling (cf. P-MPR reporting for NR).999

Proposal 6.3: transmission on a single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction (cf. no power degradation for NR DL-only CA).

# Proposal

We make the following observations and proposals.

UE RF characteristics: general considerations

Observation 2.1. In FR1 conducted TS 38.101-1, “for UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s)”. It is worth discussing if the conducted requirement could be related to the OTA requirement by at least assuming a reasonable antenna efficiency, e.g. a typical FR1 antenna efficiency is -5.5 dB.

Proposal 2.1: discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB.

Transmitter characteristics

Proposal 3.1: RAN4 collaborates with RAN1 from the start of the 6G study on the design of any reference symbols to be used for reciprocity-based DL-MIMO, if specified, and for coherent UL-MIMO with due account of expected UE transmit performance.

Proposal 3.2: Reuse PC3 (23 dBm) as the default power class for all operating bands in 6GR.

Proposal 3.3: Study how to design the UE capability framework on the supported UE power class for single carrier/CA, such that it is unambiguous for the NW and robust, so that it works for either single or multiple configured UL carriers.

Observation 3.1: specification of a power class for CA depends on whether RAN1 specifies a total power limit at which the UE prioritizes UL transmissions.

Proposal 3.4: modification or removal of current maximum output power limits could be studied, but only if accompanied by the appropriate UE reporting.

Observation 3.2: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2, like MRSS.

Observation 3.3: In Ericsson’s RRM contribution we propose to keep TDD Cell Phase Synchronization requirement the same as in NR.

Proposal 3.5: Transmitter transient period can be the same in NR for FR1 and FR2.

UE receiver characteristics

Proposal 4.1: 6GR feature requirements should be based on realistic UE architecture assumptions, rather than relying on basic UE profiles that may not support the intended feature; Rx reference architectures should be based on the capability required for support of a feature, e.g. support of large power differences between carriers for spectrum aggregation schemes should not be based on worst-case assumptions of any UE implementation.

Observation 4.1: selectivity/blocking are also important for user performance/DL-coverage, not only in-channel/link-budget performance.

Proposal 4.2: minimum requirements for MSD for band combinations are also specified OTA to complement the existing noise-factor (MSD) requirement since also relevant for support of simultaneous Rx-Tx for band combinations.

Observation 4.2: It is important that margins to actual REFSENS performance including any MSD are not too excessive as simultaneous Rx-Tx is crucial for performance regardless of any scheduler coordination.

Low-power WUR

- Observation 5.1: RAN1 start to evaluate the OFDM based DL WUS.

- Proposal 5.1: Study WUR feature enhancement from RAN4 aspects.

- Observation 5.2: UE autonomously enables/disables wake up receiver makes the power saving gain of WUR unpredictable.

Proposal 5.2: Not considering the 5G autonomous WUR/MR switching behaviour in 6G WUR study.

Proposal 5.3: Study the WUR NF in 6G study.

Spectrum aggregation

Proposal 6.1: for spectrum aggregation, study the feasibility of maintaining the ‘single carrier’ UE output power capability of a ‘primary’ or ‘anchor’ carrier at configuration/activation of the spectrum aggregation scheme (cf. increase of the allowed MPR for the Pcell at NR CA configuration).

Proposal 6.2: for spectrum aggregation, the total configured power PCMAX, or equivalent power, should be related to the behaviour of the UE for any multi-carrier scheme specified. Changes of the total output power capability could be reported by (a)periodic dedicated signalling (cf. P-MPR reporting for NR).999

Proposal 6.3: transmission on a single active/configured UL carrier within a DL multi-carrier bandwidth should not increase allowed UE power reduction (cf. no power degradation for NR DL-only CA).

# References

- RP-252912, “WF for [116bis][102] 6G general RF and UE”, Feature Lead (Qualcomm)

- R4-2513117, General and UE RF considerations for 6G, Ericsson

- R4-2513331, Discussion on 6G RRM requirements, Ericsson