3GPP TSG-RAN WG4 Meeting #117	                                   R4-2521145

Dallas, USA, Nov. 17-21, 2025

Agenda item:	4.23.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s beneficial to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design. From this objective [1], UE evaluated conditions will be specified by other groups to trigger LTM.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

Discussion

RAN4 has defined CLTM cell switch delay requirement, both intra-frequency and inter-frequency have been considered. The relevant requirements are captured in clause 6.3.2 and it refers to the conditional handover of L3 in clause 6.1.4. In the last meeting, some companies found that there is no clear scope for CLTM, and it would make confusion that whether we consider known cell or not [2].

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM Technical background: RAN4 previously agreed that specifications only cover the known cell case for CSI-RS L1 triggered CLTM. But for SSB L1 triggered CLTM, no agreement had been reached. Way forward in RAN4#115 For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. Agreement in RAN4#116 Issue 1-1-4: Whether to include unknown cell case < Agreement > - Only cover the known cell case.. Moderator comment: The following is included in the CR: 1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or. Proposals: - Proposal 1 (CTC): Discuss whether to include unknown cell case on SSB triggered CLTM, i.e., whether TPSS/SSS_sync_intra + TSSB_measurement_period_intra are needed. If so, the conditions that TSSB_time_index_inter = 0 maybe need to be updated. - Proposal 2 (Nokia): RAN4 to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event. Recommended WF: Discuss the following: - For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported: - Option 1: Support only known case (specification change needed) - Option 2: Support both unknown and known case (specification change needed) - Option 3: Support only unknown case (no specification change).

In pervious meeting, RAN4 has reached agreement that CHO requirements framework can be used as baseline for Conditional LTM. In clause 6.1.4, the definiation of Tmeasure is duplicated blew:

6.1.4.2.2 Measurement time The measurement time delay is defined from the end of TEvent_DU until UE starts handover execution preparation (TCHO_execution) to a target cell. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

For this part, the scope of CHO is also missing. There are two interpretation here. The first is only unknown cell considered for CHO, and the other understanding is that both unknown and known cells are consiered and this requirements are defined based on the worst case. Before we discuss whether the modification in specification is needed or not, we should align the inderstanding here. From our understanding, the requirement for CHO is not distinguished by know case and unknown case, and it can be applied for both cases.

Observation 1: For the definiation of Tmeasure in CHO, both known cell and unknown cell are considered, and the requirements defined based on the worst case.

For CLTM, we should confirm which scenario is supported based on the WF. For CSI-RS based measurement used with CLTM, only known case is considered due to complexity of the measurements. However, for SSB based measurement used with CLTM, which is more simpler, needs to consider unknown cell for covering more scenarios to ensure the completeness of requirements. Meanwhile, we already support unknown cells according to existing specification, and there are no any strong concerns about excluding this part.

Proposal 1: For SSB based L1-RSRP measurement used with CLTM, support both unknown and known case. And further discuss whether it is necessary to change existing specification.

Conclusions

In this contribution, we put forward the following proposal on conditional LTM.

Observation 1: For the definiation of Tmeasure in CHO, both known cell and unknown cell are considered, and the requirements defined based on the worst case.

Proposal 1: For SSB based L1-RSRP measurement used with CLTM, support both unknown and known case. And further discuss whether it is necessary to change existing specification.

References

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2514816, WF on NR_Mob_Ph4_RRM_Part2, China Telecom