3GPP TSG-RAN WG4 Meeting #117	R4-2521171

# Dallas, USA, 17th November – 21st November, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6140 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Conditional Intra-CU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Both ‘TLTM-IU’ and ‘TCLTM-IU’ are used in clause 6.3.2.2.3. and they should be same. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change ‘TCLTM-IU’ to ‘TLTM-IU’ in the interruption delay requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Using different terms to represent the same parameters could cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.3.2.2.3	Interruption time

The interruption time TCLTM-interrupt is between the time the UE starts to execute cell switch towards the target cell until the time the UE starts to transmit the new uplink transmission to the target cell, excluding TCLTM-RRC-processing.

TCLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms

Where:

-	TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

1> if the UE supports ltm-FastUE-Processing-r18 capability,

2> the value of TLTM-processing equals to

3> fr1-r18 for FR1 to FR1 LTM cell switch.

3> fr2-r18 for FR2 to FR2 LTM cell switch.

3> fr1-AndFR2-r18 for FR1 to FR2 and FR2 to FR1 LTM cell switch.

1> else:

2> the value of TLTM-processing equals to

3> 20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

3> 40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

-	Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

-	TRS-proc is the time for SSB processing.

1> if the target TCI state is in the serving cell active TCI state list or is associated to a TCI state in the serving cell active TCI state list

2> Tfirst-RS = 0 and TRS-proc= 0

1> else if the UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list or is associated to a TCI state in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

3> if the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the CLTM candidate cell TCI state activation MAC-CE is received

4> Tfirst-RS = 0 and TRS-proc= 0

1> else if the target cell is an FR1 cell, and the UE is not configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list or is associated to a TCI state in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if not more than TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

3> if not more than TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell

4> Tfirst-RS = 0 and TRS-proc= 0

2> else if the time between the latest PDCCH ordered RACH preamble transmission on the target cell and starting to execute cell switch towards the target cell is not more than 160 ms.

3> Tfirst-RS = 0 and TRS-proc= 0

1> else:

2> Tfirst-RS is the time to the first SSB transmission on the target cell after TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing +TLTM-processing.

2> TRS-proc = 2 ms.

-	TLTM-IU is the interruption uncertainty during CLTM cell switch.

-	For RACH-based CLTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

-	For RACH-less CLTM cell switch, TLTM-IU is the uncertainty on transmitting the new uplink transmission on the target cell.

==============End of change==============