3GPP TSG-RAN WG4 Meeting # 117	    R4-2521206

Dallas, US, 17 – 21 November 2025

Agenda Item:	7.12.2

Source: 	Ericsson

Title:	On general issues related to AI/ML for mobility

Document for:	Approval

# Introduction

In this contribution general aspects related to RRM measurement prediction for AI/ML aided mobility are discussed. Issues related to error modelling, clarification on L3 measurement for RAN4 evaluation(s), considerations for impact of noise, ground truth extraction, frequency pair to be considered for frequency domain prediction, and simulation assumptions for indirect event prediction are particularly discussed.

# Discussion

## Error Modelling

In the last meeting, RF error modelling was discussed but no conclusion could be reached. To further facilitate the discussion the following was captured in the WF document.

Way forward from RAN4#116bis [1]

Further discuss based on the open issues in [1] and the following way forward:

- Discuss the error model assumption for RF error taking the following as baseline:

  - Use the following assumption for RF error model in simulation:

    - Truncated Gaussian distribution under [±4] dB RF error (μ=0,  σ2=[4]) for FR2

    - Truncated Gaussian distribution under [±2.5] dB RF error (μ=0,  σ2=[2.5]) for FR1

  - RAN4 to not consider different RF error values for different RF chains for RRM measurement prediction simulations.

- Discuss how to add the measurement error including BB and RF in the simulation based on proposals 5-8.

To predict the RRM measurement, the UE needs to perform measurements for model input. The accuracy of predicted measurements depends on the accuracy of the measurements provided as an input to the AI/ML model. In RAN4#116 meeting, it was therefore agreed to consider the impact of baseband error and RF error in input measurements on the accuracy of the predicted measurements.

To evaluate the impact of RF error on RRM measurement prediction accuracy a truncated Gaussian distribution with zero mean and variance corresponding to the RF calibration error (±2.5dB for FR1 and ±4dB for FR2) for RSRP measurement shall be considered. Since RF error may appear differently when UE is performing measurements in different frequency layers, different RF error value may need to be considered for measurements in different frequency layers during simulations. Although a UE may have different RF chains that it can use to perform measurements to be used as input to its AI/ML model, but it is unlikely that the UE will use different Rx chains to perform measurements on different cells for cell level L3-RSRP measurements at a given time instant. Consideration of different RF error values for different RF chains therefore does not seem reasonable for L3-RRM measurement prediction and shall not be considered in RAN4 simulations.

Observation 1: Conclusion on RF error modelling is critical to ensure RAN4 simulation work starts in timely manner.

Observation 2: RF error can appear differently when UE is performing measurements in different frequency layers.

Observation 3: UE can use the same Rx chain to perform measurements on different cells for cell level L3-RSRP measurements at a given time instant.

Proposal 1: RF error in FR1 follows a Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

Proposal 2: RF error in FR2 follows a Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Proposal 3: RAN4 to assume same RF chain for measurements performed by UE on cells to produce model input(s) in RRM prediction simulations.

## Considerations for simulations

### L3 measurement and impact of RF error

In the last meeting, it was agreed to perform measurement prediction evaluations for both FR1 and FR2. As per the agreement (copied below for reference) L3 measurement is to be provided as input to the AI/ML model and output of the model shall be the predicted L3 measurement in time domain and frequency domain for the target cell. It should be noted that L3 filtering co-efficient was not considered in simulations to derive performance requirements for legacy L3 measurements and shall also be the approach RAN4 should govern while evaluating the prediction accuracy of RRM measurements for AI/ML aided mobility. In this regard the impact of RF error shall be considered on the measurements at point B of the measurement model in Figure 1.

Agreement from RAN4#116bis meeting [1]:

- As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2.

  - Interested companies can also provide results for other sub-cases.

  - It is not precluded to add additional sub-case for FR2 in future meetings.

- Further discuss how to define requirements based on the available simulation results.

Observation 4: L3 filtering co-efficient was not considered in simulations to derive performance requirements for legacy L3 measurements.

Figure 1. L3-RSRP measurement model from 38.300.

Proposal 4: RAN4 to not consider L3 filtering in the evaluations for the prediction accuracy of RRM measurements for AI/ML aided mobility. Impact of RF error shall therefore be considered on the measurements at point B of the measurement model in TS38.300.

### Ground truth for L3-RSRP

Two alternatives for ground truth extraction of L3-RSRP measurement in FR1 were identified based on the discussions in the Rel. 19 study item phase. Alternative 1 is to derive ground truth based on calculations. And alternative 2 is to consider the UE reported measurement value under certain conditions. When L3 filtering is not considered, alternative 1 becomes equivalent to the ideal RSRP calculations that was considered in the simulation assumptions for legacy L3-RSRP measurements. Based on this observation we propose to use alternative 1 for ground truth extraction to evaluate the accuracy of predicted L3 RRM measurement for AI/ML aided mobility.

Way forward from RAN4#116bis meeting [1]:

- Further discuss based on the open issues in [2].

Proposal 5: Alternative 1 for ground truth extraction is considered in evaluations for prediction accuracy of RRM measurements for AI/ML aided mobility.

### Frequency domain prediction

In the last meeting, frequency pairs for inter-frequency prediction were discussed, and the WF from RAN4#116bis captures the options for further consideration. These options currently focus on deployments where the measured and predicted carriers belong to the same band. To better leverage inter-frequency predictions for mobility, it would be beneficial to consider scenarios where a low-band carrier measurement is used to predict RRM measurements for a mid-band carrier. For example, using measurements on an 800MHz carrier to predict RRM measurement for a 3.5GHz carrier.

Way forward from RAN4#116bis meeting [1]:

- Consider the following frequency pairs as baseline for frequency domain prediction evaluation.

  - Candidate option 1: UE use the measurement on 2GHz to predict 4GHz.

  - Candidate option 2: UE use the measurement on 1.8GHz to predict 3.5GHz.

  - Candidate option 3: UE use the measurement on 1.8GHz to predict 2.1GHz.

  - Other options are not precluded.

- Further discuss how to define requirements based on the available simulation results.

Observation 5: Candidate options in the WF from RAN4#116bis consider measured and predicted carriers in mid-band only.

Observation 6: To better leverage inter-frequency predictions for mobility, it would be beneficial to consider scenarios where a low-band carrier measurement is used to predict RRM measurements for a mid-band carrier.

Proposal 6: For frequency domain prediction evaluations, RAN4 to consider UE to use measurement(s) on low-band carrier to predict RRM measurement(s) for mid-band carrier.

### Simulation assumption for indirect event prediction

UE can make use of the predicted RRM measurements in time domain and frequency domain to determine the likelihood of occurrence of an event before the actual event has occurred. This scenario is identified as indirect event prediction in the WID for Rel. 20 WI on AI/ML for mobility. For indirect event prediction, accuracy of the predicted RRM measurement becomes primary when it comes to determining how accurately UE can predict the likelihood of occurrence of an event before the event occurs. From evaluation point of view there should technically be no difference between the simulations that needs to be conducted by RAN4 to evaluate RRM measurement prediction accuracy and to evaluate the accuracy of indirect prediction of an event.

Observation 7: For indirect event prediction, accuracy of the predicted RRM measurement becomes primary when it comes to determining how accurately UE can predict the likelihood of occurrence of an event before the event occurs.

Proposal 7: Do not define explicit simulation assumptions for indirect event prediction evaluations. Accuracy of predicted event(s) based on predicted RRM measurement(s) shall be considered as one of the performance metrics to be reported by companies in their simulation results.

# Summary

In this contribution issues related to error modelling, clarification on L3 measurement for RAN4 evaluation(s), considerations for impact of noise, ground truth extraction, frequency pair to be considered for frequency domain prediction, and simulation assumptions for indirect event prediction are discussed.

Observations and proposals below summarize the discussion presented in this paper.

Observation 1: Conclusion on RF error modelling is critical to ensure RAN4 simulation work starts in timely manner.

Observation 2: RF error can appear differently when UE is performing measurements in different frequency layers.

Observation 3: UE can use the same Rx chain to perform measurements on different cells for cell level L3-RSRP measurements at a given time instant.

Proposal 1: RF error in FR1 follows a Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

Proposal 2: RF error in FR2 follows a Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Proposal 3: RAN4 to assume same RF chain for measurements performed by UE on cells to produce model input(s) in RRM prediction simulations.

Observation 4: L3 filtering co-efficient was not considered in simulations to derive performance requirements for legacy L3 measurements.

Proposal 4: RAN4 to not consider L3 filtering in the evaluations for the prediction accuracy of RRM measurements for AI/ML aided mobility. Impact of RF error shall therefore be considered on the measurements at point B of the measurement model in TS38.300.

Proposal 5: Alternative 1 for ground truth extraction is considered in evaluations for prediction accuracy of RRM measurements for AI/ML aided mobility.

Observation 5: Candidate options in the WF from RAN4#116bis consider measured and predicted carriers in mid-band only.

Observation 6: To better leverage inter-frequency predictions for mobility, it would be beneficial to consider scenarios where a low-band carrier measurement is used to predict RRM measurements for a mid-band carrier.

Proposal 6: For frequency domain prediction evaluations, RAN4 to consider UE to use measurement(s) on low-band carrier to predict RRM measurement(s) for mid-band carrier.

Observation 7: For indirect event prediction, accuracy of the predicted RRM measurement becomes primary when it comes to determining how accurately UE can predict the likelihood of occurrence of an event before the event occurs.

Proposal 7: Do not define explicit simulation assumptions for indirect event prediction evaluations. Accuracy of predicted event(s) based on predicted RRM measurement(s) shall be considered as one of the performance metrics to be reported by companies in their simulation results.

# References

R4-2514827, WF on NR_AIML_Mob_RRM, OPPO.

R4-2513532, Topic summary for [116bis][226] NR_AIML_Mob_RRM, Moderator (OPPO).