3GPP TSG-RAN WG4 Meeting #117	R4-2521235

Dallas, USA, 17 November – 21 November 2025

Agenda Item:	7.9.4

Source:	Ericsson

Title:	RRM requirements for two-sided AI/ML models

Document for:	Discussion

# Introduction

RAN4#116bis started the core part of WI AI/ML for NR air interface Phase 2, where RAN4 need to specify the necessary RRM requirements according to WID [1].

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In RAN4#116bis, several companies presented the LCM issues, and it was captured in the topic summary [1].

Issue 3-1-1: LCM requirement This sub-topic contains general considerations on LCM requirements Recommended WF - Discuss the following: - Should the Rel-20 CSI prediction LCM requirements be based on Rel-19 as a baseline ? - Rel-19 Beam Management, or Rel-19 CSI prediction ? - For performance monitoring, should requirements be placed on LCM decisions ? - Should RAN4 discuss performance metrics for monitoring ? - Set of potential requirements for discussion: - Activation - Deactivation -> Deprioritize ? - Switching - Fallback to non-AI -> Deprioritize ? - Performance monitoring - Reporting delay - Accuracy - Considering possibly averaging and connection with reporting time Issue 3-1-2: Functionality activation delay This issue considers specifically activation delay, due to specific proposals Recommended WF - Discuss whether the following can be agreed: - There is a need to wait for RAN2 design before making a final decision. - Assuming the same RAN2 design as for CSI prediction, then UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

This contribution discusses our view on the necessary RRM requirements for two-sided AI/ML models.

# RRM requirements

## Model activation/deactivation delay

In Rel-19 AI/ML-based PMI prediction, RAN4 does not define the AI/ML-based CSI prediction model activation/deactivation delay requirements because of the assumption that UE must be ready to start measurements for inference after sending RRC reconfiguration complete.

For one-sided AI/ML models such as PMI prediction, since RAN1 agreed to use the Rel-18 eTypeII-Doppler codebook, gNB can receive a PMI even if UE does not use AI/ML-based CSI prediction. Therefore it is not critical even if gNB does not know when the UE activates/deactivates the one-sided model, as far as UE reports the proper PMI according to the CSI report configuration.

On the other hand, for two-sided CSI compression, it is very important to activate/deactivate the encoder and decoder at the same time, otherwise, the gNB cannot reconstruct the target CSI.

Observation 1: For two-sided AI/ML models, when the UE encoder is activated, gNB also needs to activate the associated decoder at the same time.

We propose RAN4 should consider introducing activation/deactivation/switching delay requirements for two-sided CSI compression so that the gNB can activate/deactivate/switch the associated decoder at the same time.

Proposal 1: RAN4 should consider introducing activation/deactivation/switching delay requirements for two-sided CSI compression encoder so that the gNB can activate/deactivate/switch the associated decoder at the same time.

## Performance monitoring

In Rel-19 AI/ML-based PMI prediction, RAN1 defined UE-based and NW-based performance monitoring. The UE-based performance monitoring uses the CSI reporting framework, and it is called CSI-PAI, where UE is required to report two SGCS values. We think one issue of the CSI-PAI is testability; it is difficult to verify the quality of the reported CSI-PAI quantity (first/second SGCS values) because TE does not know the ground truth PMI at UE.

We expect RAN1 will discuss and define the performance monitoring function for two-sided AI/ML models also. Since performance monitoring is essential for gNB to avoid the negative impact to the network performance, it is important to ensure the testability of the reported quantity.

RAN4 should monitor the RAN1 discussion on the performance monitoring function for CSI compression. RAN4 should study the testability of the performance monitoring options, and if necessary, RAN4 informs RAN1 so that RAN1 chooses a method that is testable.

Proposal 2: RAN4 should monitor the RAN1 discussion on the performance monitoring functionality for CSI compression. RAN4 should study the testability of the performance monitoring options, and if necessary, RAN4 informs RAN1 so that RAN1 chooses a method that is testable.

# Summary

Observations:

Observation 1: For two-sided AI/ML models, when the UE encoder is activated, gNB also needs to activate the associated decoder at the same time.

Proposals:

Proposal 1: RAN4 should consider introducing activation/deactivation/switching delay requirements for two-sided CSI compression encoder so that the gNB can activate/deactivate/switch the associated decoder at the same time.

Proposal 2: RAN4 should monitor the RAN1 discussion on the performance monitoring functionality for CSI compression. RAN4 should study the testability of the performance monitoring options, and if necessary, RAN4 informs RAN1 so that RAN1 chooses a method that is testable.

# References

- RP-252445, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm.

- R4-2514530, “Topic summary for [116bis][123] NR_AIML_air_Ph2”, Moderator (Ericsson).