# 3GPP TSG-RAN WG4 Meeting #117		R4-2521243

# Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Agenda Item:	8.8.2

Title: 	Use cases for AI/ML in RAN4

Document for:	Approval

- Introduction

For the upcoming 6G, the SID [1] of 6GR was approved in RAN#108 meeting. The objectives related to AI air interface for 6GR are listed as follows.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we would like to provide our viewpoints on use cases for AI/ML in RAN4.

- Use cases for AI/ML in RAN4

  - AI based Non-linearity Compensation

Supporting the growing demand for high data rates with higher-order modulation requires a higher SNR, which, at a constant noise level, must be supported by increased transmission power. However, this introduces a new challenge, i.e., increasing transmission power is often limited by the non-linear characteristics of the PA.

Currently, it is desirable for the PA to operate in its linear region, or at least for its output to meet the EVM requirements, in order to avoid degradation of data demodulation performance at the receiver due to PA non-linearity. Additionally, non-linear compensation (NC) for the PA is another approach to address this issue, which can be implemented either at the transmitter side using Digital Pre-Distortion (DPD) or at the receiver side using Digital Post-Distortion (DPoD).

    - DPoD for PA Non-linearity Compensation

UL DPoD uses AI based post-processing to mitigate performance degradation caused by PA non-linearity at the NW side. This may relax the EVM requirements at the transmitter, allowing the PA to operate over a wider dynamic range. Specifically, at least two different implementations of DPoD at the receiver could be considered, as shown in Figure 1:

Figure 1: DPoD at the NW side

- DPoD-opt1: NW-side NC using a separate AI-based DPoD module + non-AI demodulation. An independent AI-based DPoD module is introduced before demodulation to handle PA non-linearity. The received signal can be converted to the time domain for DPoD processing and then converted back to the frequency domain. This approach may reduce model complexity and size compared to frequency-domain processing.

- DPoD-opt2: NW-side NC using a separate AI-based DPoD module + AI demodulation. The received signal can be converted to the time domain for DPoD processing and then converted back to the frequency domain for AI demodulation.

- Furthermore, above schemes could be extended to integrate channel estimation. The AI-based DPoD module itself could also be updated through online training/tuning to make it robust to the time-varying characteristic of PA non-linearity.

Proposal 1: 	For 6G, NW side DPoD (Digital Post-Distortion) should be studied in RAN4.

Proposal 2:  	For NW side DPoD, following sub cases could be considered in RAN4

      - DPoD-opt1: NW side NC by a separate AI DPoD module + Non-AI demod

      - DPoD-opt2: NW side NC by a separate AI DPoD module + AI demod

      - Note1: AI DPoD module could be updated through online training/tuning

      - Note2: Channel estimation may/may not be included

In RAN4 studies, different approaches could be considered based on the performance, cost analysis and standardization impact. Regarding the evaluation procedure for DPoD, in RAN4#116bis meeting, following agreements have been made:

Agreement: - Non-linearity model(s) of transmission signals: - Discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda), which is intended to discuss the common model as much as possible. - Using separate models for AI and non-AI DPoD is not precluded. - Evaluation Procedures and some detailed discussion points - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation. - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure - FFS a joint session or separate session of AI and non-AI non-linearity compensation. - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity. - FFS suitable AI/ML model, type of training. - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs). - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations - Focusing on higher modulation orders focusing on high-SNR conditions - Other details are not precluded - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation. - The potential MPR reduction enabled by AI-DPoD is evaluated. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged - The impact on UL coverage and UE transmit energy efficiency is evaluated. - Other design principle to be considered: - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

For Step-1, based on our understanding, the non-linearity model(s) of transmission signals should be aligned in RAN4(at least in RAN4 AI section, e.g., to have some options in simulation assumptions), which will facilitate better comparison of results from different solutions during the study. Besides, whether to consider an online update/tuning mechanism for PA non-linear processing is proposed in RAN4 discussion. This also raises the question of whether the time-varying characteristic of PA non-linearity should be considered in PA non-linearity modeling. One approach is not to involve explicit modeling of the PA time-varying characteristic. Instead, the simulation only assesses the training latency and training data requirement of DPoD, establishing these as assumptions for online model updates. Another approach is to involve explicit modeling of the PA time-varying characteristic. Simulations then evaluate the tuning latency and tuning data requirement of DPoD based on the modeled PA time-varying characteristic.

Proposal 3: 	RAN4 should consider and clarify whether the time-varying characteristic of PA non-linearity should be considered in PA non-linearity modeling.

For Step-2, achieving early alignment on simulation assumptions will contribute to research progress. For example, based on companies' inputs, during or after RAN4#117, RAN4 could agree on an aligned simulation assumption for further study. The simulation assumption from OPPO side is shown as follows:

Table 1 Configuration of simulation

| Parameters | Value |
| --- | --- |
| Carrier frequency | 3.5GHz |
| TX antenna number | 1 |
| RX antenna number | 4 |
| Waveform | CP-OFDM |
| Sub-carrier number | 96 |
| OFDM symbol number | 12 |
| DMRS | 1symbol, ρ=0.5 |
| MCS | MCS25（256QAM, CR=885/1024） |
| Channel model | TDL-A, 30ns |
| Subcarrier spacing | 15 kHz |
| UE speed | 3km/h |
| Channel estimation | Practical |
| Evaluation metric | BLER |
| EVM assumption | 3.5%, 8% |
| Benchmark | Conventional receiver (without non-linearity compensation) |
| PA setup (in simulation) : The non-linearity of PA is modeled using the RAPP model [2], as shown below, where - 𝐴_𝑖𝑛 denotes the input amplitude, - 𝐴_0=3 represents the limiting output amplitude - 𝑣=1 is small signal gain - 𝑝=1.83(for EVM=3.5%) or 1.18(for EVM=8.0%) is defined as the smoothness parameter - 𝐺(⋅) signifies the output amplitude |  |

Proposal 4: 	Based on companies' inputs, during or after RAN4#117, RAN4 should have on an aligned simulation assumption for further study.

As shown in Figure 2, link-level simulations have been conducted. DPoD-opt2(using both an AI-based DPoD module and an AI demodulator) is evaluated in this section. The performance is measured in terms of BLER under different EVM assumptions. Benchmark is the conventional receiver without non-linearity compensation.

Different EVM assumptions(e.g., 3.5% and 8%) are considered. It should be noted that 3.5% EVM represents the minimum EVM requirement for 256QAM modulation as specified by RAN4 in 5G NR. 8% EVM is also evaluated in simulations to verify the ability of AI-DPoD to handle more severe non-linear distortions.

According to the simulation results, the proposed AI-DPoD scheme achieves a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM. When EVM increases to 8%, the proposed AI- DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels.

Figure 2 Simulation results of the AI-DPoD scheme

Observation 1: 	According to the simulation results, the proposed AI-DPoD scheme achieves a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM.

Observation 2: 	When EVM increases to 8%, the proposed AI- DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels.

As suggested by FL in RAN4#116bis meeting, the following is the summarized description on DPoD.

Table 2 The summarized description on DPoD

| UL DPoD |  | OPPO view |
| --- | --- | --- |
| AI model input | Input in training | Received signal after channel estimation and equalization |
|  | Input in inference | Received signal after channel estimation and equalization |
| AI model output | Label in training | Received signal before demodulation [or before channel decoding] |
|  | Output in inference | Received signal before demodulation [or before channel decoding] |
| Assumption on training | Training type | offline training [FFS online training, tuning] |
|  | Label construction (if applicable) | May need data collection for offline training [FFS online training, tuning] |
| Model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | For inference, no collaboration/interaction For training, collaboration/interaction may be needed for date collection |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Value. Carrier frequency; 3.5GHz. TX antenna number; 1. RX antenna number; 4. Waveform; CP-OFDM. Sub-carrier number; 96. OFDM symbol number; 12. DMRS; 1symbol, ρ=0.5. MCS; MCS25(256QAM, CR=885/1024). Channel model; TDL-A, 30ns. Subcarrier spacing; 15 kHz. UE speed; 3km/h. Channel estimation; Practical. PA setup (in simulation) : The non-linearity of PA is modeled using the RAPP model [2], as shown below, where - 𝐴_𝑖𝑛 denotes the input amplitude, - 𝐴_0=3 represents the limiting output amplitude - 𝑣=1 is small signal gain - 𝑝=1.83(for EVM=3.5%) or 1.18(for EVM=8.0%) is defined as the smoothness parameter - 𝐺(⋅) signifies the output amplitude. |
| Evaluation KPI |  | BLER gain @ EVM(3.5%, 8%) |
| Evaluation benchmark |  | Conventional receiver (without non-linearity compensation) |
| Preliminary evaluation results |  | NW DPoD achieve a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM. When EVM increases to 8%, the proposed AI-DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels. |
| High level potential specification impact |  | EVM/[MPR] relaxing in RAN4 requirement |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Model; FLOPs; Trainable parameters. AI DPoD + AI Demod; 86.4M + 11.1M; 44K. |

    - DPD for PA Non-linearity Compensation

Figure 3 illustrates a method for implementing DPD at the UE side. Before the signal enters the PA, it is processed by an AI model that performs digital pre-distortion. This model can be trained either offline or online.

Figure 3: DPD at the UE side

The model complexity is summarized as following:

| Model | FLOPs | Trainable parameters |
| --- | --- | --- |
| DPD | 0.57M | 0.4K |

Figure 4 Simulation results of the AI-DPD scheme

For DPD, the simulation assumptions we have adopted are same as the assumptions that used for DPoD. Figure 4 shows the simulation results. The proposed AI-DPD scheme achieves a performance gain of approximately 1.2 dB at a BLER of 10% with 3.5% EVM. When EVM increases to 8%, the proposed AI-DPoD schemes still achieve 10% BLER at an SNR of 21.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels.

For DPD, the simulation results indicate that a simple AI-DPD model could be utilized to achieve PA non-linear compensation, delivering better performance. Here, we also need to note that, implementing DPD at the UE side using a pre-trained approach to obtain a fixed PA non-linear compensation model seems has low/less standardization impact, as it could be addressed through UE implementation. If online training/tuning approach is needed for AI-DPD to derive the PA non-linear compensation model and to handle the time-varying characteristic of PA non-linearity, the standardization impact requires further analysis, e.g., how to acquire the training/tuning data and how to evaluate performance gains while accounting for training overhead.

Observation 3: 	For AI-DPD, the simulation results indicate that a simple AI-DPD model could be utilized to achieve PA non-linear compensation, delivering better performance.

Observation 4: 	Implementing DPD at the UE side using a pre-trained PA non-linear compensation model seems has low/less standardization impact.

  - AI based RRM enhancement

From RRM perspective, measurement is the baseline for most of RRM procedures e.g., mobility, beam management, CA operation, active DL/UL TCI switch, and SRS antenna switch, etc. According to the existing requirements defined in TS 38.133, the measurement will require gap, interruption or scheduling restriction, which will cause data loss and highly impact the system throughput. And since UE can only measure one layer in a gap occasion for the measurement within gap, and UE measurement resources (e.g., searchers) is limited for the measurement without gap, the measurement delay needs to be extended by the number of layers. On the other hand, the measurement availability and delay may also impact the corresponding procedure delay or performance. Considering the above aspects, we think the measurement reduction and mobility performance improvement are still the baseline motivations for AI/ML based enhancement.

Proposal 5: 	The study goals for 6G AI/ML based RRM study are to reduce the measurement efforts and to improve the mobility performance.

In last meeting, we have provided the background information for AI/ML RRM related use cases studied in 5G NR in our contribution [5]. And the use cases and corresponding scenarios are summarized as below:

Table 3: Summary of AI/ML based use case and scenarios for measurement prediction in 5G NR

| Use case (purpose) | Report type | AI Model |  | Prediction domain | Included in 5G NR |
| --- | --- | --- | --- | --- | --- |
|  |  | Input | Output |  |  |
| Beam management | L1-RSRP (beam level) | L1 beam measurement | L1 beam measurement | Time | Yes |
|  |  |  |  | Spatial |  |
|  |  |  |  | Combination of time and spatial |  |
| LTM | L1-RSRP (beam level) | - | - | - | No |
| L3 Mobility | L3-RSRP (cell level) | L1 beam measurement | L1 beam measurement | Time Frequency Spatial | Spatial domain is only supported for NW side model |
|  |  | L3 cell measurement | L3 cell measurement |  |  |
|  |  | L1 beam measurement | L3 cell measurement |  |  |
|  | L3-RSRP (beam level) | L1 beam measurement | L1 beam measurement | Time Frequency Spatial | Only supported for NW side model |
|  |  | L3 beam measurement | L3 beam measurement |  |  |
|  |  | L1 beam measurement | L3 beam measurement |  |  |

Regarding 6G AI/ML RRM related use case, the proposals in RAN4#116bis meeting are captured in the WF [6] for information and the following agreements are reached for selection.

Issue 6-1c: Additional principle/method for AI-RRM use case selection Agreement: - Additional principle/method for AI-RRM use case selection - Whether the proposed AI-RRM use case should be driven by RAN4: - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study. - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

With the above agreement, the use cases of L1 beam measurement for beam management and L3 Cell measurement for mobility will not be considered in 6G SI, but whether they can be leveraged to 6G can be further discussed during WI stage.

According to the summary in last meeting, there are 12 AI/ML RRM use cases proposed by companies, but we think there are some overlapping and similarities among different use cases. To simplify and facilitate the discussion, we think the use cases can be categorized into three types as below. And for each category, we can discuss and select the scenarios after clarifying the motivation, performance gain and complexity etc.

- Cat1: Measurement related use cases, with different scenarios, e.g., different types of measurements for AI model input/output (including use case 1,2,3,4,7a)

- Cat2: AI-assistant enhancement for measurement procedure or scheduling e.g., measurement relaxation (including use case 5a,11,14)

- Cat3: AI-assistant enhancement for certain RRM procedures, e.g., AI/ML based RLM/LTM/CA operation (including use case 9,10,12,13)

Observation 5: 	The AI RRM related use cases can be summarized as the following three categories:

- Cat1: Measurement related use cases, with different scenarios.

- Cat2: AI-assistant enhancement for measurement procedure or scheduling.

- Cat3: AI-assistant enhancement for certain RRM procedures, e.g., RLM/LTM/CA operation.

For Cat1 (i.e., measurement related use cases), we think the use cases and scenarios not included in 5G NR can be considered in 6G study. For example, regarding L3 measurement for mobility, only cell level measurement prediction with time domain and collocated frequency domain prediction are included for UE sided model in 5G NR. Due to the limited time and RAN4 workload, L3 beam level measurement was not included in 5G NR, but considering the completion of L3 mobility and the similarity between L3 cell level and beam level in terms of AI/ML related mechanism, we think L3 beam level measurement prediction can be studied in 6G SI and the approaches of L3 cell level prediction can be extended.

Besides L3 measurement for mobility, L1 beam level measurement for mobility is also introduced in Rel-18 LTM. Referring to the requirements defined in TS 38.133 9.15.5 and 9.1.5.4, L1 beam level measurement can be used for neighbor cells which may also need measurement gaps and will participate the gap sharing with L3 measurement referring to the requirements. Reducing the measurement on L1 measurement for mobility would also be an approach to reduce the gap ratio and improve the system throughput. We think most of the approaches defined for L3 cell level and beam level measurement prediction can be reused.

For each above use case, time domain, frequency domain, spatial domain prediction (Tx beam prediction) and the combination of different domains are possible. But we need to consider the reality of each case, e.g., regarding frequency domain prediction for beam measurement, it is not realistic to predict the measurements on different frequencies with the same Tx beam. Because the measurements on different frequencies are from different cells, even the Tx beam (i.e., SSB index) is same, the related direction can be different. And the Tx beam index which UE can receive on the two frequencies are different, which may make the prediction on the same Tx beam meaningless. So the better way is to predicted the best beams on different frequencies which makes the prediction not a pure frequency domain prediction but a combination of frequency and spatial prediction. Based on such principle, we think we need to analyze the realistic prediction approach for each use case above rather than to consider all the combinations.

Proposal 6: 	The L1/L3 beam measurement prediction use case can be included in 6G SI considering:

      - the combination of spatial and time domain prediction and,

      - the combination of spatial and frequency domain prediction.

For frequency domain prediction, only co-located scenario is considered in 5G NR which highly restricts the usage of frequency prediction. It is noted that for different frequencies, non-collocated scenario is also typical deployment and it deserves some studies on the possible solutions to enable frequency domain prediction. We can understand that there may be some issues and difficulties for non-collocated scenarios, e.g.,

- The channel status changes for different frequencies are independent and with less relevance. It is hard to get the correlation between non-collocated frequencies.

- Some information of neighbor cells (e.g., relative location, transmission power) is not known to UE.

- The differences between site to site may be significant which makes the generalization hard to be done.

So we think we can firstly investigate the difficulties of frequency domain prediction for non-collocated scenario and potential standardized approaches to resolve identified issues. From our side, we think for a certain scenario e.g., the sites deployment is fixed, it is possible to find some solutions to enable the non-collocated frequency domain prediction e.g., through some assistant information. But for generalization part, further study is needed.  Different from the co-located scenario, once the environment is changed, the relevance between two non-collocated frequency layers would have big difference and unpredictable, which makes the prediction performance hard to be guaranteed. So before we support the frequency domain prediction, the generalization or model update issue should be considered.

Proposal 7: 	The solutions on non-collocated frequency domain prediction in stationary scenario and the corresponding generalization can be studied in 6G SI.

Among the above discussions, spatial domain prediction refers to Tx beam prediction which is included in 5G NR. Taking the following figure 5 as an example, the Tx beam prediction only focus on the Tx handling under same Rx beam (e.g., Case 1 or Case 2) which may reduce measurement efforts or network transmission overhead.

Figure 5: illustration of beam prediction

But from RAN4 perspective, for L1 and L3 measurement, Rx beam is also an important aspect since Rx beam sweeping is assumed for receiving different Tx beams which require additional measurement efforts and extend the measurement delay. So it is beneficial if some solutions can be found to predict the Rx beam or Tx-Rx beam pair (e.g., Case 3) to reduce beam sweeping and the measurement efforts on UE side. For this target, some studies can be done in 6G SI on the potential enhancements or difficulties for Rx beam or Tx-Rx beam pair prediction. For this prediction approach, it cannot be a single domain prediction and at least the combination of time domain and spatial domain prediction can be considered for intra-frequency measurement.

However, since the Rx beam sweeping is up to UE implementation and no report is needed, there are some difficulties to define the Rx beam prediction use case, e.g.,

- How to define the input and output of AI model, e.g., the reporting of Rx beam

- How to handle the different cells with single Rx

- How to differentiate from Tx beam prediction

Since Rx beam sweeping is UE implementation, and one Rx may receive multiple Tx beams and multiple cells, the prediction results may still need to reflect on measurements on Tx beams or cells. If that is the case, it needs to be considered what is the difference from the existing Tx beam prediction.

Proposal 8: 	Some issues need to be resolved before supporting Rx beam or Rx-Tx beam pair prediction in 6G SI, e.g.,

      - The definition of AI model input and output

      - The handling of different cells for different Rx beams

      - The difference from Tx beam prediction and cross-cell prediction

For the above use case in Cat1, we think they can be RAN4 driven use case since RAN4 is more familiar with the beam related measurement assumption and the impact on measurement behaviors or delay requirements, e.g., the Rx beam sweeping assumption, the difference between L1 and L3 beam, and the difference between intra-cell and inter-cell, etc.

For Cat2 (i.e., measurement procedure or scheduling enhancement), we see some example proposals in last meeting, e.g., the measurement on some frequencies or cells can be relaxed based on AI/ML prediction, or the criteria for neighbor cell measurement triggering can be enhanced based on future prediction. For this category of use case, we are open to discuss, but we think more justifications are needed. E.g., we have some existing criteria for measurement relaxation and neighbor cell triggering. The performance gain comparing to the existing mechanism should be clarified. And since the criteria for certain measurement behavior are usually discussed and specified in RAN2, the coordination with RAN2 should be considered.

For Cat3 (i.e., AI-assistant enhancement for certain RRM procedures), we think the scenarios not covered by the measurement prediction in Cat1 can be considered. One example is the enhancement for CA operation. We understand that CA operation is an important part for coverage enhancement or overhead offloading. And the CA related procedures play an important role in RRM requirements. But due to the preparation of software/ hardware and the synchronization/measurement on target cell, the activation delay is long. AI/ML is a good approach to reduce the activation delay by early preparation and improve the performance by SCell selection assistance.

Proposal 9: 	RAN4 to study the AI/ML assistant CA operation use cases in 6G SI, e.g., AI/ML assistant SCell enhancement.

With above discussions, we summarized the use cases of measurement prediction to be considered in 6G SI based on our proposals as below:

Table 4: Summary of AI/ML based use case and scenarios for measurement prediction in 6G

| Use case | Prediction domain | AI Model input and output | Model side and training | Note | Expected benefits |
| --- | --- | --- | --- | --- | --- |
| L1/L3 beam measurement for mobility (HO/LTM) | Combination of time and spatial domain | Input: L1/L3 measurement based on set B of beams at historic time instance(s). Output: predicted L1/L3 measurement on set A of beams. Set B is different from Set A | NW and UE side model; Offline training | Intra-cell | Measurement reduction; HO/LTM performance improvement |
|  | Combination of frequency and spatial domain | Input: L1/L3 measurement based on set B of beams at frequency B. Output: predicted L1/L3 measurement on set A of beams at frequency A. | NW and UE side model; Offline training | Inter-cell | Measurement reduction; Gap reduction |
| AI/ML assistant CA operation |  | Time and frequency domain measurement prediction | NW and UE side model; Offline training | Intra-cell and Inter-cell | Delay reduction |
|  |  | Input: Cell or beam measurement. Output: Predicted time for carrier/Cell switch. | NW and UE side model; Offline training | Intra-cell and Inter-cell | Performance improvement, e.g., load balance |

Above discussions are based on L1 measurement and L3 measurement respectively. However, at the meantime, we noticed that in 5G NR, there are some overlapping for L1 and L3 measurement e.g., for L1 MIMO, L1 mobility and L3 mobility. The UE measurement behavior is similar, but the measurement configurations, capabilities, mobility procedures and the requirements are defined separately. And the L1 and L3 measurement for the same cell may compete the measurement resources or gap resources which highly extend the measurement delay. To simplify the UE behavior, reduce the measurement efforts and provide flexibility of mobility management, we think a unified solution for L1 and L3 measurement can be considered.

When discussing the AI/ML based RRM use cases, the potential integration for L1 and L3 measurement should be also taken into consideration. Before the unified approach is concluded, the use cases can still be discussed, but the common design can be used as much as possible.

Proposal 10: 	In Rel-20 6G SI, the mechanism for AI/ML based L1 and L3 measurement can be designed as common as possible to accommodate the unified L1/L3 measurement framework.

- Conclusion

In this contribution, use cases regarding the AI based Non-linearity Compensation and AI based RRM enhancement are analyzed and got the following proposals/observations:

AI based Non-linearity Compensation:

Proposal 1: 	For 6G, NW side DPoD (Digital Post-Distortion) should be studied in RAN4.

Proposal 2:  	For NW side DPoD, following sub cases could be considered in RAN4

      - DPoD-opt1: NW side NC by a separate AI DPoD module + Non-AI demod

      - DPoD-opt2: NW side NC by a separate AI DPoD module + AI demod

      - Note1: AI DPoD module could be updated through online training/tuning

      - Note2: Channel estimation may/may not be included

Proposal 3: 	RAN4 should consider and clarify whether the time-varying characteristic of PA non-linearity should be considered in PA non-linearity modeling.

Proposal 4: 	Based on companies' inputs, during or after RAN4#117, RAN4 should have on an aligned simulation assumption for further study.

Observation 1: 	According to the simulation results, the proposed AI-DPoD scheme achieves a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM.

Observation 2: 	When EVM increases to 8%, the proposed AI- DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels.

Observation 3: 	For AI-DPD, the simulation results indicate that a simple AI-DPD model could be utilized to achieve PA non-linear compensation, delivering better performance.

Observation 4: 	Implementing DPD at the UE side using a pre-trained PA non-linear compensation model seems has low/less standardization impact.

AI based RRM use case:

Observation 5: 	The AI RRM related use cases can be summarized as the following three categories:

- Cat1: Measurement related use cases, with different scenarios.

- Cat2: AI-assistant enhancement for measurement procedure or scheduling.

- Cat3: AI-assistant enhancement for certain RRM procedures, e.g., RLM/LTM/CA operation.

Proposal 5: 	The study goals for 6G AI/ML based RRM study are to reduce the measurement efforts and to improve the mobility performance.

Proposal 6: 	The L1/L3 beam measurement prediction use case can be included in 6G SI considering:

      - the combination of spatial and time domain prediction and,

      - the combination of spatial and frequency domain prediction.

Proposal 7: 	The solutions on non-collocated frequency domain prediction in stationary scenario and the corresponding generalization can be studied in 6G SI.

Proposal 8: 	Some issues need to be resolved before supporting Rx beam or Rx-Tx beam pair prediction in 6G SI, e.g.,

      - The definition of AI model input and output

      - The handling of different cells for different Rx beams

      - The difference from Tx beam prediction and cross-cell prediction

Proposal 9: 	RAN4 to study the AI/ML assistant CA operation use cases in 6G SI, e.g., AI/ML assistant SCell enhancement.

Proposal 10: 	In Rel-20 6G SI, the mechanism for AI/ML based L1 and L3 measurement can be designed as common as possible to accommodate the unified L1/L3 measurement framework.

- References

- RP-251881, New SID: Study on 6G Radio, RAN#108, June 2025

- Kenington, Peter B. "Methods linearize RF transmitters and power amps." Microwaves & RF 37.12 (1998): 102-116.

- RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- R4-2513318 RAN4 driven AI based use cases, RAN4#116bis

- R4-2514637 WF on 6G AI, RAN4#116bis