# 3GPP TSG-RAN WG4 Meeting #117		R4-2521256

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.5.2

Source:		Qualcomm Incorporated

Title:			UE demodulation requirements for 5G broadcast over GSO

Document for:	Approval

# 1  Introduction

The core part of WI on new LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite was competed in RAN#109 [1] [2].

RAN4#116bis has started the performance part and the test parameters are agreed and captured in [4]. In this paper, we provide the simulation results for UE demodulation of 5G broadcast over GSO.

# 2  PMCH demodulation requirements

The simulation is conducted based on the paramteres and propagation condition agreed in [4]. Specifically, the k-factor of 8.05dB is used for NTN-TDLC5 channel model which is consistent with k-factor assumption used in NR NTN UE demodulation.

Proposal 1: With the agreed parameters, the impairment SNR values in Table 1 are proposed as the UE demodulation requirements.

Table 1: Minimum performance for MBMS dedicated cell over GSO

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | Alignment SNR(dB) | Impairment SNR [dB] |
| 1 | MBMS Dedicated Cell | 10 | R.PMCH.1 FDD | NA | NTN-TDLC5-5 (Table D.1.2-1) | 1x2 low | 1 | 1.7 | 3.7 |
| 2 | MBMS Dedicated Cell | 10 | R.PMCH.2 FDD | NA | NTN-TDLC5-5 (Table D.1.2-1) | 1x2 low | 1 | 8.6 | 10.6 |

# 3  Conclusion

In this paper, we provide the simulation results and propsoal for UE demodulation requirements for 5G broadcast over GSO.

Proposal 1: With the agreed parameters, the impairment SNR values in Table 1 are proposed as the UE demodulation requirements.

Table 1: Minimum performance for MBMS dedicated cell over GSO

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | Alignment SNR(dB) | Impairment SNR [dB] |
| 1 | MBMS Dedicated Cell | 10 | R.PMCH.1 FDD | NA | NTN-TDLC5-5 (Table D.1.2-1) | 1x2 low | 1 | 1.7 | 3.7 |
| 2 | MBMS Dedicated Cell | 10 | R.PMCH.2 FDD | NA | NTN-TDLC5-5 (Table D.1.2-1) | 1x2 low | 1 | 8.6 | 10.6 |

# 4  References

- RP-251943, Status Report for New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, RAN#109, September 2025

- RP-252885, Summary for WI New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, RAN#109, September 2025

- RP-251419, Revised WID: New LTE band for 5G broadcast for region 3 utilizing geosynchronous satellite, RAN#108, June, 2025.

- R4-2515068, WF on [116bis][313] 5G_Broadcast_GSO, RAN4#116bis, October, 2025.