# 3GPP TSG-RAN WG4 Meeting #117	                                                 	R4-2521258

Dallas, TX, USA, November 17-21, 2025

Agenda item:	8.11

Source:		Qualcomm Incorporated

Title:			Further views on 6G Testability and OTA

Document for:	Approval

# 1  Introduction

Following new SID for Study on 6G Radio [1] and Study on 6G Scenarios and requirements [2-3], RAN4 has started the 6G discussion from RAN4#116bis meeting. In RAN4#116bis meeting, the way-forward on 6G Testability and OTA was approved in [4].

In this paper, we provide further views on 6G testability and OTA.

# 2  Discussion

## 2.1 Improved testability of conducted requirements

In WF agreed in RAN4#116bis, the following agreements were reached regarding possibility of improving testability of conducted requirements:

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

In 5G and LTE era, both conductive spurious emission requirements and radiated spurious emission requirements were specified for FR1 devices.

For FR1, the general spurious emission requirements are specified in Table 6.5.3.1-2 of TS38.101-1 wihch is based on conductive testing.

Meanwhile, the radiated spurious emission requirements are specified in Table 8.2.4-1 of TS 38.124 for UE EMC.

Table 8.2.4-1: Radiated spurious emissions requirements for UE equipment supporting operations in FR1, traffic mode

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 | 1 MHz | 4 |
|  | -25 | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz. NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz. NOTE 3: As specified in TS 38.101-1 [3]: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: As specified in TS 38.101-1 [3]: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

From above two tables, the test configurations and limits for conductive spurious emission in UE RF requirements and radiated spurious emission are same for frequency range 30MHz to 26GHz. The difference is that frequency range for 9kHz to 30MHz is not tested for radiated spurious emissions. In addition, the effective radiated power (e.r.p.) is tested as the metric for radiated spurious emissions in which antenna gain is considered.

From the perspective of protecting other services or systems, it is important to verify the impact of antenna gain on compliance with unwanted emissions requirements.

Since the UE must meet both conducted and radiated spurious emission requirements, it is worth investigating whether test efforts can be reduced—for example, by verifying either the radiated or conducted spurious emission requirement for 6G UEs.

To assess whether compliance with radiated spurious emission limits consistently implies compliance with conducted spurious emission limits, we performed measurements of conducted power and maximum EIRP. The ERP power was calculated based on the measured EIRP results using the formula:

ERP (dBm) = EIRP (dBm) – 2.15.

Table 1: Measurement results for carrier frequency

| NR Band | NR Channel | NR_UL_Freq (MHz) | NR CBW (MHz) | MaxEIRP-conducted (dB) | maxERP-conducted (dB) |
| --- | --- | --- | --- | --- | --- |
| 3 | 349500 | 1747.5 | 10 | -0.15 | -2.3 |
| 5 | 376000 | 836.5 | 10 | -3.07 | -5.22 |
| 28 | 156100 | 725.5 | 10 | -4.79 | -6.94 |
| 41 | 518598 | 2952.99 | 100 | 2.53 | 0.38 |
| 71 | 136100 | 680.5 | 10 | -6.92 | -9.07 |
| 78 | 636666 | 3549.99 | 100 | 4.61 | 2.46 |

As shown in Table 1:

- When the difference between maximum e.r.p and conducted power is positive, the radiated requirement becomes the limiting factor.

- When the difference between maximum e.r.p and conducted power is negative, the conducted requirement becomes the limiting factor.

It should be noted that the results in Table 1 are measured only at the carrier frequency. For radiated spurious emission testing, the EUT shall be rotated at each spurious frequency where a component is detected to obtain the maximum response. Since antenna gain at the carrier frequency may differ from that at spurious frequencies, RAN4 should further study the possible equivalence between radiated and conducted spurious emission requirements, considering potentially impactful factors such as the difference between e.r.p and conducted power.

Proposal 1: RAN4 could further investigate the possible equivalence between radiated and conducted spurious emission requirements, taking into account factors such as the difference between e.r.p and conducted power at spurious frequencies, the dependence of antenna gain between carrier frequency and spurious frequencies, etc.

## 2.2 FR1 OTA testability enhancements

The OTA requiremetns inlcuidng both SISO OTA and MIMO OTA for 6G bands will not be defined in 6G Day 1 due to the absence of commercial devices for measurement commpaign.

Observation 1: The OTA inlcuidng both SISO OTA and MIMO OTA requirements for 6G bands will not be defined in 6G Day 1 due to the absence of commercial devices.

Study can be conducted to assess whether OTA requirement can be derived without measurement results or with partial measurement resluts. Condiering OTA performanc of handheld UE would be highly relying on the factors such as antenna placement and material covering the antenna, etc. Therefore, the study for the approach of not fully relying on measurement compaign could start from other UE types which might have common antenna placement option such as glass, etc.

A study may be conducted to evaluate whether OTA requirements can be derived without full measurement data or with partial results. For handheld UEs, OTA performance is highly influenced by factors such as antenna placement and surrounding materials. Therefore, initial investigation into reduced reliance on measurement campaigns could focus on UE types with more standardized antenna configurations, such as those incorporating glass structures.

Propsoal 2: Study can be conducted in RAN4 to assess whether OTA requirement can be derived without measurement results or with partial measurement resluts. The initial investigation into reduced reliance on measurement campaigns could focus on UE types with more standardized antenna configurations, such as those incorporating glass structures.

## 2.3 FR2 OTA testability enhancements

The mmW UE test can only be conducted performed by radiated/OTA due to high integration of RFIC and RF front end. And the complexity of radiated/OTA test is much higher than the conducted test. Due to the increase in complexity, the mobile industry has recognized the importance of reducing the testing time of OTA test for mmW bands. To enhance the test methods, 3GPP Rel-17 completed a SI and one of objectives is to reduce the testing time in IFF that is the main test system for current mmW testing.

The Reverberation Chamber (RC) offers a fundamentally different method for evaluating radiated performance in an OTA environment. Within an RC, device performance is generally characterized through statistical summaries derived from measurements in a rich multipath setting. This approach enables rapid assessment of complex effects, though it may sacrifice certain detailed information. For instance, in a conventional RC, the angle of arrival is randomized and essentially unknown, making it impossible to plot precise radiation patterns, even though their statistical impact can be measured quickly and accurately.

RC principles are well established in industry and have been widely discussed and used for NR FR1 and LTE; however, they have not yet been addressed for FR2 within 3GPP. The TRP measurement methodology for NR FR2 remains unchanged when using RC compared to NR FR1. As a result, TRP-based measurements could be adopted for FR2, and RC has the potential to significantly reduce test time. Therefore, RC should be considered as an alternative test method for FR2 TRP-based measurements, with further study needed to determine whether RC can also be applied to other test cases such as EIRP and EIS-based measurements.

Proposal 3: RAN4 to study the feasibility and testability of using RC as an alternative test method for FR2 TRP-based measurements. Further study is needed on whether RC can be applied to EIRP and EIS-based measurements.

## 2.4 Harmonized testing for TN and NTN

As agreed in [4], RAN4 targets a harmonized test system for TN and NTN devices for FR1.

Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN Agreements: - Targeting a harmonized test system/test method for TN and NTN devices for FR1. - The test complexity and cost should be considered.

In Rel-19, the support for CP (Circular Polarization) and LP (Linear Polarization) in NTN OTA testing was discussed, and it was concluded that LP should be supported in Rel-19 due to the complexity of upgrading the current test system.

LP is the dominant antenna design for handheld UEs. However, since satellites typically use circularly polarized antennas (e.g., LHCP or RHCP), future NTN handheld UEs may adopt more advanced antenna designs, such as a hybrid of LP and CP, or even full CP.

For TRP measurements, consider an NTN satellite using RHCP to receive signals from three different UEs:

- UE #1 transmits using an RHCP antenna.

- UE #2 transmits using an LP antenna, as commonly found in today’s smartphones.

- UE #3 transmits using an LHCP antenna.

Assuming all three UEs radiate the same Tx power level but with different polarizations as described above, an RHCP receiver on the satellite would receive:

- ~100% of UE #1’s signal

- ~50% of UE #2’s signal

- Very little of UE #3’s signal

Detailed measurements and analysis can be found in [5], which shows that the difference between LP and CP (RHCP or LHCP) for EIRP/TRP measurements is non-negligible.

Implementing CP using LP and a 90° hybrid is one of feasible solutions to support both CP and LP in one harmonized test system and this solution does not require significant upgrades or costs to the existing test system. Additionally, the calibration process takes approximately 1–2 hours, which is acceptable. To enable a unified TN and NTN test system, RAN could further study automation between CP and LP.

Propsoal 4: RAN4 to study a harmonized test setup for TN and NTN FR1 devices, supporting both CP and LP to reflect real deployment scenarios.

Proposal 5: Consider implementing CP measurement antennas using LP combined with a 90° quadrature hybrid as the starting point. RAN4 should further investigate the feasibility of an automated switching solution among RHCP, LHCP, and LP.

## 2.5 AI/ML Testability

In 5G, test method for AI/ML was studied with the assumptions that keeping the test costs in a reasonable level. Therrfore, the legacy test setup of enhanced IFF and 3D-MPAC were used as the baseline in AI/ML beam mamagement testing. While in 6G, new AI/ML cases may be driven in RAN1/2/4. The the correspoding test method to verify the new AI/ML features such as reduced RX beam sweeping factor will require further investigation assuming that new test setup could be considred.

Proposal 6: The the correspoding test method to verify the new AI/ML features such as reduced RX beam sweeping factor needs to be further studied in RAN4.

As the number of AI/ML use cases increases, RAN4 should consider how to define more generic test systems and test methods to support a wider range of AI/ML test cases. In addtion, the test sytem and test methods should target to support multiple tests with differnt channel conditions to check the genralization of AI/ML models.

Propsoal 7: RAN4 to consider how to define more generic test systems and test methods to support a wider range of AI/ML test cases. The test system and test methods should target to support multiple tests with differnt channel conditions to check the genralization of AI/ML models.

# 3. Conclusion

In summary, we have following observations and proposals.

Proposal 1: RAN4 could further investigate the possible equivalence between radiated and conducted spurious emission requirements, taking into account factors such as the difference between e.r.p and conducted power at spurious frequencies, the dependence of antenna gain between carrier frequency and spurious frequencies, etc.

Observation 1: The OTA inlcuidng both SISO OTA and MIMO OTA requirements for 6G bands will not be defined in 6G Day 1 due to the absence of commercial devices.

Propsoal 2: Study can be conducted in RAN4 to assess whether OTA requirement can be derived without measurement results or with partial measurement resluts. The initial investigation into reduced reliance on measurement campaigns could focus on UE types with more standardized antenna configurations, such as those incorporating glass structures.

Proposal 3: RAN4 to study the feasibility and testability of using RC as an alternative test method for FR2 TRP-based measurements. Further study is needed on whether RC can be applied to EIRP and EIS-based measurements.

Propsoal 4: RAN4 to study a harmonized test setup for TN and NTN FR1 devices, supporting both CP and LP to reflect real deployment scenarios.

Proposal 5: Consider implementing CP measurement antennas using LP combined with a 90° quadrature hybrid as the starting point. RAN4 should further investigate the feasibility of an automated switching solution among RHCP, LHCP, and LP.

Proposal 6: The the correspoding test method to verify the new AI/ML features such as reduced RX beam sweeping factor needs to be further studied in RAN4.

Propsoal 7: RAN4 to consider how to define more generic test systems and test methods to support a wider range of AI/ML test cases. The test system and test methods should target to support multiple tests with differnt channel conditions to check the genralization of AI/ML models.

# 4  References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025

- R4-2514647, WF for [116bis][110] 6G testability and OTA, Feature lead (vivo), RAN4#116bis, October, 2025.

- R4-2500400, Discussion on FR1 NTN devices OTA, Qualcomm, RAN4#114, February, 2025