# 3GPP TSG-RAN WG4 Meeting #117		  R4-2521259

Dallas, TX, USA 17th – 21st November, 2025

Agenda item:	6.11.4

Source:		Qualcomm Incorporated

Title:			Discussion on AI/ML beam management testability

Document for:	Approval

# 1  Introduction

In RAN4#116bis, the following agreements regarding test setup are captured in [1].

Issue 2-8: Test system setup Agreement: A multi-Ao IFF based test setup with 4 active cross-polarized probes placed in the single plane (xz plane). The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively. Issue 2-9: Simplified Channel models Agreement: RAN4 will further discuss the simplified Uma and UMi channel model by taking the following aspects into consideration - Commercial test system limitation, including the number of probes and the separation - Model simplification should not result in inference performance degradation beyond the margin when the training is done based on the original models. - The margin can be discussed and decided further. - Other aspects are not precluded.

In this paper, we provide further views on the testability issues for AI/ML beam management.

# 2  Discussion

## 2.1 General

In general, the high-level test procedure for AI/ML BM-case 1 can be described below:

- Progress A: UE BM performance with AI/ML for BM-case 1

  - Step 1: Model training step - Channel parameters set #1

    - Contuct by offline

  - Step 2: Measurement step - Channel parameter set #2

    - TE transmits Set B beams

  - Step 3: Prediction step

    - UE reports top K predicted beams of set A based on measurement of set B beams.

- Progress B: Ground truth of UE BM performance without AI/ML for BM-case 1

  - Step 1: Measurement step – Channel parameters set #2

    - TE transmits set A beams.

    - UE reports measured RSRP of all beams of set A.

  - Setp 2: TE compares the measured RSRP of the strongest beam and the maximum RSRP of top K predicted beam.

Figure 1: High-level test procedure for AI/ML BM-case 1

In addition, it is essential to randomize the transmission of setA beams during step1 of progress B of Figure 1 to properly test UE’s AI-ML model.

Additionally, if the transmission of setA beams is not randomized during measurement step of progressB, UE can simply inform TE that its top-1 strongest beam is same as its top-1 predicted beam of progressA. UE does not even need to measure the beams to predict it. However, UE won’t be able to cheat the test like this if the transmission of setA beams is randomized during measurement step of progressB.

Based on the illustration of the test steps, we have the following observation.

Observation 1: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - The test should reflect performance in the field. Hence, CDL or simplified CDL to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification.

## 2.2 Multiple AoA test setup with enhanced IFF (eIFF) system

FR2 RRM test setup eIFF was agreed as the baseline to verify the AI/ML beam management [1]. As shown in Figure 2, there are 4 physical probes in the current FR2 RRM test setup. These 4 probes can transmit power simultaneously with different amplitude and phase. Therefore, the combination of 4 probes can be considered to emulate a multiple-AoAs test environment. Hence the eIFF multiple AoAs/Probes setup can be used with simplified CDL model since there may be no need to consider implementing multiple clusters as CDL channel model for AI/ML BM testing.

Figure 2: An example test setup for multiple probes on top of legacy RRM test setup

Observation 2: To generate Set A and Set B, multiple AoAs/probes test setup is necessary.

Observation 3: Using Multiple AoA test setup with FR2 RRM test can reuse the existing FR2 test setup to save the test cost and a simplified CDL channel model needs to be specified. The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively.

## 2.3 Simplifed CDL channel model

Several potential approaches for simplifying the CDL model were proposed in [2] and [3], aiming to reduce the number of clusters.
For example, [2] suggested the following options for defining a simplified CDL channel model with the eIFF test setup:

- Remove weak clusters that do not influence UE beam management.

- Merge clusters with identical or similar AoAs.

- Reduce per-cluster parameter values, such as CASD, CASA, CZSD, and CZSA.

In [3], three candidate options (A, B, and C) for simplified channel model parameters for Umi CDL-C were proposed.

It should be noted that all options proposed in [2] and [3] for the simplified channel model are independent of gNB Tx beam number and Tx beam direction. Another option decried in [4] considered was to simplify the channel model based on Tx beam direction and the reflected PAS contributed by illuminated clusters. However, this approach would significantly increase complexity and make the channel model parameters harder to validate a chamber environment.

Observation 4: Simplifying the CDL channel model based on Tx beam direction and reflected PAS from illuminated clusters would make the model parameters much more complicated and harder to validate in the chamber.

Proposal 1: The simplified channel model parameters should remain independent of gNB Tx beam number and Tx beam direction.

With Observation 4 and proposal 1, we recommend RAN4 to start with channel models paramters decribed in Option A/B and C in  [3] to evlaute the inference perforamnce degradation.

- Option A: with flattened elevation (Table 1) and strongest cluster aligned to one probe;

- Option B:  with flattened elevation (Table 1) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes;

- Option C: with flattened elevation, AoAs aligned to probe layout and no intra cluster angle spread (Table 2)

Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF [3]

Propsoal 2: RAN4 to take the channel model parametres decriped in Table 1 and Table 2 as the starting point to evlaute the inference perforamnce degradation.

Table 1 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |

It should be noted that for both Option A and Option B, the channel model emulated in the chamber may not be exactly the same as the channel model used in simulation. This is because TE vendors might apply different weighting schemes to simulate AoAs between probes, depending on implementation and per probe optimization.

Specifically, for Option A, emulating cluster #13 would be challenging since it lies outside the probe spread, making it infeasible to reproduce signals reflected by cluster #13 in the chamber. However, all clusters listed in Table 1 can still be considered in simulation. In contrast, Option C does not have this issue, as each cluster is aligned with a probe location.

Based on the above, we propose the following:

Proposal 3: For Options A and B, RAN4 should further clarify how to address the following issues:

- The channel model emulated in the chamber may depend on TE-specific optimization weights.

- The channel model used for evaluation may not be consistent with the emulated channel model in the actual OTA chamber.

Regarding the selection of the most appropriate option, the criterion should be the performance gap between inference results based on the simplified channel model and those based on the original channel model specified in TS 38.151 for FR2 MIMO OTA. All simulation assumptions, except for the channel model, should follow those used in beam management performance evaluation.

Proposal 4: RAN4 should decide which option to adopt based on the performance gap between inference performance using the simplified channel model and the original channel model specified in TS 38.151 for FR2 MIMO OTA. All simulation assumptions, except for the channel model, should follow those used in beam management performance evaluation.

## 2.3 Impact of measurment uncertainty

RAN4 has agreed to perform the study to assess the impact of UE measurement error on the prediction accuracy. In the core requirements discussion, the impact of baseband error and RF error were studied. In addtion to UE measurmenet error, the measurement uncertainty introduced by test system in the OTA chamber may also have inpact on the prediction accuracy.

In general, the measurement uncertainty source inlcudes the following two stages:

- Stage 1: the calibration of the absolute level of the DUT measurement results is performed by means of using a calibration antenna whose absolute gain is known at the frequencies of measurement

- Stage 2: the actual measurement with the DUT as either the transmitter or receiver is performed.

An example of measurement uncertainty for multiple probes test setup is provided in Table B.2.5-1, TS 38.151[2]. The totally expanded uncertainty for DL MIMO sensitivity is 5.05dB. The measurement uncertainty for EIS with single probe test setup in IFF is also larger than 5dB [3]. Therefore, RAN4 needs to study whether to consider the impact of the measurement uncertainty when specifying AI/ML BM core requirements.

Proposal 5: RAN4 to investigate whether measurement uncertainty should be considered when defining AI/ML BM core requirements.

# 2  Conclusion

In summary, we have following observations and proposals.

Observation 1: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - The test should reflect performance in the field. Hence, CDL or simplified CDL to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification.

Observation 2: To generate Set A and Set B, multiple AoAs/probes test setup is necessary.

Observation 3: Using Multiple AoA test setup with FR2 RRM test can reuse the existing FR2 test setup to save the test cost and a simplified CDL channel model needs to be specified. The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively.

Observation 4: Simplifying the CDL channel model based on Tx beam direction and reflected PAS from illuminated clusters would make the model parameters much more complicated and harder to validate in the chamber.

Proposal 1: The simplified channel model parameters should remain independent of gNB Tx beam number and Tx beam direction.

Propsoal 2: RAN4 to take the channel model parametres decriped in Table 1 and Table 2 as the starting point to evlaute the inference perforamnce degradation.

Table 1 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |

Proposal 3: For Options A and B, RAN4 should further clarify how to address the following issues:

- The channel model emulated in the chamber may depend on TE-specific optimization weights.

- The channel model used for evaluation may not be consistent with the emulated channel model in the actual OTA chamber.

Proposal 4: RAN4 should decide which option to adopt based on the performance gap between inference performance using the simplified channel model and the original channel model specified in TS 38.151 for FR2 MIMO OTA. All simulation assumptions, except for the channel model, should follow those used in beam management performance evaluation.

Proposal 5: RAN4 to investigate whether measurement uncertainty should be considered when defining AI/ML BM core requirements.

# 4  References

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm, RAN4#116bis

- R4-2513691, Discussion on AI/ML beam management testability, Qualcomm, RAN4#116bis

- R4-2514248, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, RAN4#116bis

- R4-2514404, RRM performance requirements for Beam Management and Positioning accuracy, Nokia, RAN4#116bis

- TS 38.151

- TR 38.508-1