3GPP TSG-RAN WG4 Meeting #117                                           R4-2521261

Dallas, United States, Dec 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Co-existence study for HPUE with maximum output power up to 32dBm

Agenda Item:	7.1.3

Document for:	Approval

Introduction

In RAN4 #116bis meeting [1], the parameters and candidate options on simulation assumptions for adjacent channel co-existence study for 4Tx HPUE with up to 32dBm max output power were discussed.

This document provides our considerations on the assumptions to be further checked in the last meeting, along with simulation results for evaluating ACLR requirements.

Discussion

- 2.1 Network layout model

In RAN4 #116bis meeting [1], the ISD for 3.5 GHz and 4.9 GHz are in bracket for further check, and the ISD in ITU WP5D and TR 38.921 are the candidate options. The ISD of urban macro for 4.9 GHz and 7 GHz in ITU WP5D for WRC-27 is 600 m [3]. In TR 38.921, there is only the co-existence study for 7 GHz, and the ISD for 7 GHz is 450 m. But the antenna configuration in TR 38.921 is non-subarray-based and the size is smaller than that of subarray-based antenna, so the ISD can be further increased due to the large EIRP.

In TR 37.829, only the co-existence study for 2.6 GHz and 3.5 GHz was conducted and the ISD of 3.5 GHz is 500 m. Since 4.9 GHz and 3.5 GHz are close in frequency, the ISD for 3.5 GHz can be reused for 4.9 GHz. For 7 GHz, the ISD of C-band with 500 m can also be reused to achieve the same coverage as C-band.

Proposal 1: The proposed ISD for 4.9 GHz and 7 GHz is 500 m, which is aligned with the ISD of 3.5 GHz in TR 37.829.

In TR 37.829, the antenna height of BS for 3.5 GHz is 45 m, which is too high and severely inconsistent with actual deployment. In addition, considering that 7 GHz and C-band will be likely deployed at the same site, the antenna spacing 3.5 GHz can also be 25 meters, just like that for 4.9 GHz and 7 GHz.

Proposal 2: The proposed BS antenna height for 3.5 GHz is 25 m, which is aligned with the antenna height of 4.9 GHz and 7 GHz.

- 2.2 Uplink transmission power control model

For the uplink transmission power control model, the current formula for CLx-ile in TR 38.921 is:

CLx-ile = 88 + 10*log10(200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

It can be confirmed that this CLx-ile was calculated with the UE’s maximum output power set to 23 dBm, targeting an uplink SINR of 15 dB. This calculation is based on [2], which was agreed in RAN4#95-e.

If the UE’s maximum output power is not equal to 23 dBm, the formula for CLx-ile should be updated as follows.

CLx-ile = 88 + 10*log10(200/X) + 11 – Y-23+Pmax

= 10*log10(200/X)-Y+ Pmax +76

Considering the minimum output power of UE is -33dBm for 100MHz CBW [1], the Rmin = Pmax - (-33) = Pmax +33.

Proposal 3: The parameters of power control model should be updated to include the impact of the maximum output power:

-	CLx-ile = 10*log10(200/X)-Y+Pmax +76, where X is UL transmission BW (MHz) and Y is the BS noise figure

-  Rmin = Pmax +33

- 2.3 Performance metric

The Option 2 of WF [1] actually follows the simulation procedure specified in TR 37.829. This method has the following issues:

(1) This procedure is designed to verify that an ACLR of 37 dB is acceptable for UE with 31 dBm max output power. However, it does not specify the evaluation metrics based on which the ACLR value of 37 dB is derived. As can be seen from TR 37.829, when the ACLR is 37 dB, the throughput loss of 31 dBm UE is far less than 5% (approximately 2%) for many cases. We consider this requirement is too strict and it will increase the stress of implementation complexity.

(2) This verification method works by ensuring that the performance degradation of the victim system caused by 31 dBm interfering UE does not exceed that caused by 23 dBm interfering UE. Therefore, the first step is to determine the proper ACLR value for 23 dBm UE. According to TR 37.829, the ACLR value for 23 dBm UE is 30 dB. However, evaluation results show that when the ACLR is 30 dB, the throughput loss of 23 dBm UE is less than 5% (approximately 2%) for many cases, which is very stringent. Thus, if Option 2 is adopted, the ACLR value for the 23 dBm UE in Step 1 must first be determined, and discussions should be held on whether it is possible to relax the ACLR requirement for 23 dBm UE in the candidate bands in that case.

Observation 1: In step 1 of option 2 of performance metric in WF, we should get the proper ACLR value of 23 dBm UE at first and use the resulted ACLR of 23dBm UE for the candidate bands.

It can be observed that in many previous coexistence studies [3][4][5][6] in Rel 15~19, the default evaluation criterion for 23 dBm UE is the throughput loss of 5%. Therefore, the ACLR value for 23 dBm UE can still be determined based on this metric. For option 2 of WF in last meeting, ensuring that the performance degradation of the victim system caused by the 31 dBm interfering UE does not exceed that caused by the 23 dBm interfering UE implies comparable performance degradation levels between the two scenarios. If the performance degradation is considered as ~5% throughput loss, i.e., the throughput loss of the 31 dBm UE and 23dBm UE are both close to 5%. In such case, we can directly adopt the throughput loss of less than 5% as the performance metric, which is consistent with option 1 in WF.

Proposal 4: Adopt option 1 in the WF (R4-2515085) as the baseline of performance metric.

Simulation results

Considering the parameters of 2.6 GHz and 3.5 GHz were agreed in the last meeting and 7 GHz is critical, the simulations of these three bands are conducted to derive the required ACLR of HPUE. Based on the assumptions of annex, we set 42 (7 GHz) / 45dB (below 6 GHz) ACS for BS and30 dB ACLR for UE as the baseline (DeltaACIR=0). The corresponding UL ACIR is about 29.7(7 GHz)/29.9 dB (below 6 GHz). When we simulate the scenario, different delta-ACIR X values were considered in order to find the reasonable requirements.

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 2.6 GHz centre frequency in table 1. The required ACIR for 2.6 GHz is 30 dB under the assumption of max 32dBm output power for urban macro scenario.

Table 1: Urban macro simulation results of the UL throughput loss at 2.6 GHz

| ACIR offset =X [dB] | 0 | -1 |
| --- | --- | --- |
| Average throughput loss (2.6 GHz) | 0.47 % | 0.40% |
| 5%-tile throughput loss (2.6 GHz) | 4.94 % | 5.94% |

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 3.5 GHz centre frequency in table 2. The required ACIR for 3.5 GHz is 28 dB under the assumption of max 32 dBm output power for urban macro scenario.

Table 2: Urban macro simulation results of the UL throughput loss at 3.5 GHz

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 |
| --- | --- | --- | --- | --- |
| Average throughput loss (3.5 GHz) | 0.61% | 0.64% | 0.76% | 0.89% |
| 5%-tile throughput loss (3.5 GHz) | 3.83% | 3.87% | 5.09% | 7.29% |

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 7 GHz centre frequency in table 3. The required ACIR for 7 GHz is 24 dB under the assumption of max 32dBm output power for urban macro scenario.

Table 3: Urban macro simulation results of the UL throughput loss at 7 GHz

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 | -4 | -5 | -6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 0.09% | 0.10% | 0.11% | 0.13% | 0.13% | 0.16% | 0.17% |
| 5%-tile throughput loss (7 GHz) | 2.25% | 2.70% | 2.79% | 3.14% | 3.19% | 3.42% | 5.53% |

Observation 2: Under the assumption of a max 32 dBm output power for urban macro scenario, the required ACIR for 2.6 GHz, 3.5 GHz and 7 GHz are as follows.

Table 4: Simulation results of ACIR for 2.6GHz, 3.5GHz and 7 GHz

|  | 2.6 GHz | 3.5 GHz | 7 GHz |
| --- | --- | --- | --- |
| ACIR | 30dB | 28 dB | 24 dB |

Proposal 5: The proposed ACIR values for 2.6GHz, 3.5GHz and 7GHz based on simulations are 30dB, 28dB and 24dB respectively.

Proposal 6: Given the simulated ACIR values for different example bands and to keep the consistence in the specification with other power classes, the proposed PC1 ACLR for bands above 2.5GHz supporting 4Tx is 31dB.

Proposal 7: If further evaluation is needed, it is proposed to update the co-existence simulation assumptions for some parameters considering real deployment scenarios.

Conclusion

In this contribution, we provide assumptions and preliminary simulation results of coexistence study for evaluating the ACLR requirements, based on the assumption of a 4x26 dBm PA configuration.

The observations and proposals made in this contribution are summarized below:

Proposal 1: The proposed ISD for 4.9 GHz and 7 GHz is 500 m, which is aligned with the ISD of 3.5 GHz in TR 37.829.

Proposal 2: The proposed BS antenna height for 3.5 GHz is 25 m, which is aligned with the antenna height of 4.9 GHz and 7 GHz.

Proposal 3: The parameters of power control model should be updated to include the impact of the maximum output power:

-	CLx-ile = 10*log10(200/X)-Y+Pmax +76, where X is UL transmission BW (MHz) and Y is the BS noise figure

-  Rmin = Pmax +33

Proposal 4: Adopt option 1 in the WF (R4-2515085) as the baseline of performance metric.

Observation 1: In step 1 of option 2 of performance metric in WF, we should get the proper ACLR value of 23 dBm UE at first and use the resulted ACLR of 23dBm UE for the candidate bands.

Observation 2: Under the assumption of max 32dBm output power for urban macro scenario, the required ACIR for 2.6 GHz, 3.5 GHz and 7 GHz are as follows.

Table: ACIR for 2.6GHz, 3.5GHz and 7 GHz

|  | 2.6 GHz | 3.5 GHz | 7 GHz |
| --- | --- | --- | --- |
| ACIR | 30dB | 28 dB | 24 dB |

Proposal 5: The proposed ACIR values for 2.6GHz, 3.5GHz and 7GHz based on simulations are 30dB, 28dB and 24dB respectively.

Proposal 6: Given the simulated ACIR values for different example bands and to keep the consistence in the specification with other power classes, the proposed PC1 ACLR for bands above 2.5GHz supporting 4Tx is 31dB.

Proposal 7: If further evaluation is needed, it is proposed to update the co-existence simulation assumptions for some parameters considering real deployment scenarios.

Our suggested parameters are listed in the Annex. Different parameters compared to the WF in last meeting are highlighted in yellow color.

- References

- R4-2515085, Way Forward for [116bis] [329] NR_UE_RF_Ph5_Part2 - HPUE coexistence, Huawei, HiSilicon.

- R4-2008928, TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, Nokia, Nokia Shanghai Bell.

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz.

- TR 38.876, Air-to-ground network for NR.

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz.

- TR 38.803, Radio Frequency (RF) and co-existence aspects.

- ITU WP 5D/792. Working document on characteristics of terrestrial component of IMT sharing and compatibility studies in preparation for WRC-27(Annex 4.32 to Working Party 5D Chair’s Report)

# Annex: Simulation assumptions

Table A-1: Co-existence study scenario

| Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- |
| eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6 GHz,3.5 GHz,7 GHz | Urban macro |

Table A-2: Network layout model for urban macro

| Parameters |  | Value |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 750m @ 2.6 GHz ,500m @ 3.5 GHz / 4.9 GHz / 7 GHz |
| Multi operator’s layout |  | Uncoordinated |
| BS antenna height |  | 45m @2.6GHz, 25 m@ 3.5 GHz / 4.9 GHz / 7 GHz |
| UE location | Outdoor/indoor | Outdoor and indoor |
|  | Indoor UE ratio | 20% (TR 38.921) |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 (TR 38.921) |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |
| Channel model |  | UMa |

Table A-3: Propagation model

| Propagation loss | Value |
| --- | --- |
| Path loss | TR 38.901 |
| LOS probability | TR 38.901 |
| Penetration loss | TR 38.901 O2I building penetration loss |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |

Table A-4: BS antenna characteristics of urban macro

| Parameter | Symbol | 2.6 GHz, 3.5 GHz,4.9 GHz | 7 GHz |
| --- | --- | --- | --- |
| Antenna pattern |  | TR 38.922 | TR 38.922 |
| Front to back ratio | Am | 30 dB | 30 dB |
| Side lobe suppression | SLAv | 30 dB | 30 dB |
| Horizontal HPBW | θ3dB | 90 deg. | 90 deg. |
| Vertical HPBW | θ3dB | 65 deg. | 65 deg. |
| Element peak gain | GE,max | 6.4 dBi | 6.4 dBi |
| Number of element rows in sub-array | Msub | 3 | 3 |
| Vertical element separation in sub-array | dv,sub | 0.7λm | 0.7λm |
| Pre-set sub-array down-tilt | θsubtilt | 3 deg. | 3 deg. |
| Number of radiating elements rows | M | 4 | 8 |
| Number of radiating elements columns | N | 8 | 16 |
| Horizontal element separation | dh | 0.5λm | 0.5λm |
| Vertical element separation | dv | 2.1λm | 2.1λm |
| Electrical down-tilt angle | θetilt | 90-100 deg. | 90-100 deg. |
| Electrical scan angle | φescan | ±60 deg. | ±60 deg. |
| Mechanical downtilt | θmech | 6 deg. | 6 deg. |

Table A-5: UE configuration

| Parameter | Value |
| --- | --- |
| Antenna pattern | isotropic with 0dBi (TR 38.921) |

Power control model specified in Section 9.1 TR 36.942 as follows is adopted.

where,

-	Pmax = 32dBm

-	CLx-ile = 10*log10(200/X)-Y+Pmax +76, where X is UL transmission BW (MHz) and Y is the BS noise figure

-  Rmin = Pmax +33

-	γ = 1

Table A-6: Other simulation parameters

| Parameters | Value |
| --- | --- |
| Frequency reuse | 1x3x1 |
| The number of active UE (UL) | 1 or 3 |
| Scheduled channel bandwidth per UE (UL) | 98.28 MHz (1 user) 32.76 MHz (3 user) |
| Link Level Performance Model | TR 38.921 |
| Received power model | TR 38.921 |
| Traffic model | Full buffer |
| UE max TX power in dBm | 23 (as reference), 32 |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 6 (@7 GHz) 5 (@below 6 GHz) |
| UE Noise figure in dB | 9 or 12 (Note 1) |
| ACLR of UE | 30 dB +X |
| ACS of BS | 45 dB (@below 6 GHz) 42 dB (@7 GHz) |
| Handover margin | 3 dB |
| Note 1: NF of 12 dB is for band n104. For bands below 6 GHz, NF is 9dB. |  |

The performance metric is average throughput loss and 5%-ile throughput loss less than 5%.